### Antarctic Automatic Weather Station Data

for the calendar year 2001

by

- Linda M. Keller

George A. Weidner

Charles R. Stearns

Jonathan E. Thom

Matthew A. Lazzara

Space Science and Engineering Center

University of Wisconsin

1225 W. Dayton St.

Madison, Wisconsin

February, 2007

- Antarctic Automatic Weather Station Data

- For The Calendar Year

2001

by

- Linda M. Keller

- George A. Weidner

- Charles R. Stearns

- Jonathan E. Thom

Matthew A. Lazzara

- Space Science and Engineering Center

- University of Wisconsin

- 1225 W. Dayton St.

- Madison, Wisconsin

- February, 2007

- TABLE OF CONTENTS

- Page

1.  Introduction.........................................................................................................................	  1

2.  Data Transmission..............................................................................................................   	  1

3.  AWS Identification and Location......................................................................................... 	  1

4.  AWS Data Summaries........................................................................................................   	  9

4.1. Monthly Data Summaries..............................................................................................  	  9

D-10..............................................................................................................................	 10

D-47..............................................................................................................................	 10

Dome C II..................................................................................................................... 	 10

Port Martin…………………………………………………………………………………….	 11

Cape Denison…………………………………………………………………………………	 11

Penguin Point............................................................................................................... 	 11

Mount Siple...................................................................................................................  	 12                   Theresa........................................................................................................................ 	 12

Doug............................................................................................................................. 	 12

Elizabeth....................................................................................................................... 	 13

Brianna…………………………………………………………………………………………           13

Harry……………………………………………………………………………………………	 13

Erin............................................................................................................................... 	 14

Siple Dome................................................................................................................... 	 14

Swithinbank................................................................................................................... 	 14

Marble Point..................................................................................................................	 15

Ferrell............................................................................................................................ 	 15

Pegasus North...............................................................................................................	 15

Pegasus South.............................................................................................................. 	 16

Minna Bluff.................................................................................................................... 	 16

Linda............................................................................................................................. 	 16

Willie Field....................................................................................................................	 17

Windless Bight..............................................................................................................	 17

Herbie Alley……………………………………………………………………………………	 17

Cape Spencer…………………………………………………………………………………	 18

Cape Bird....................................................................................................................... 	 18

Laurie II……...................................................................................................................	 18

Whitlock………………………………………………………………………………………..           19

Possession Island..........................................................................................................	 19

Manuela.........................................................................................................................	 19

Marilyn...........................................................................................................................	 20

Schwerdtfeger................................................................................................................	 20

Gill.................................................................................................................................	 20

Lettau............................................................................................................................	 21

Elaine............................................................................................................................	 21

Larsen Ice Shelf.............................................................................................................	 21

Butler Island................................................................................................................... 	 22

Uranus Glacier…………………………………………………………………………………          22

Limbert.......................................................................................................................... 	 22

Racer Rock.................................................................................................................... 	 23

Bonaparte Point.............................................................................................................	 23

Sky-Blu........................................................................................................................... 	 23

Kirkwood Island………………………………………………………………………………..           24

Dismal Island................................................................................................................. 	 24

Clean Air (South Pole)................................................................................................... 	 24

Henry............................................................................................................................. 	 25

Nico............................................................................................................................... 	 25

Relay Station.................................................................................................................	 25

Dome Fuji………………………………………………………………………………………           26

Mizuho………………………………………………………………………………………….           26

4.2. Three Hourly Data Summaries....................................................................................... 	 27

5.   AWS Calibration................................................................................................................. 	 27

5.1. Temperature................................................................................................................. 	 27

5.2. Pressure....................................................................................................................... 	 27

5.3. Wind Direction and Speed............................................................................................ 	 27

5.4. Relative Humidity......................................................................................................... 	 28

5.5. Vertical Air Temperature Difference.............................................................................. 	 28

6.   AWS Operations Summary for 2001...................................................................................	 28

6.1. AWS Performance........................................................................................................ 	 28

6.2. AWS Antarctic Field Activities....................................................................................... 	 30

7.   Global Telecommunications System.................................................................................... 	 31

8.   Data Availability................................................................................................................... 	 31

9.   Acknowledgments................................................................................................................ 	 32

- 1.  INTRODUCTION

A network of automatic weather station (AWS) units is deployed to collect Antarctic surface weather observations in support of specific meteorological research projects as well as operational activities in  Antarctica.  The 2001 network consisted of 52 installed AWS units providing observations on the Ross Ice Shelf, east of the Transantarctic Mountains and north of McMurdo to the Adelie Coast, along the Antarctic Peninsula, West Antarctica, and climatological locations such as the South Pole.  Each unit measures air temperature, wind speed, and wind direction at the top of the unit's tower at a nominal height of three meters and air pressure at the electronics enclosure (Figure 1).  Some AWS units also measure the relative humidity at three meters and vertical air temperature difference between 0.5 and 3 meters.  Measurement heights relative to the actual surface at the site are nominal due to snow accumulation around the AWS unit.

- 2.  DATA TRANSMISSION

The transmitted AWS data are received and stored by the Data Collection System on the NOAA series of polar orbiting satellites.  The data are retransmitted by the satellite for use in the High Resolution Picture Transmission (HRPT) broadcast at McMurdo, Antarctica.  The data are also received with the Global Area Coverage (GAC) data collected at the Space Science and Engineering Center, University of Wisconsin-Madison. The data are processed into scientific units and are available for local use.  The complete data set is received at the University of Wisconsin-Madison from CLS America (Service ARGOS), Largo, Maryland, for processing and distribution to the users.

- 3.  AWS IDENTIFICATION AND LOCATION

Site location is defined by the latitude and longitude which is determined by various methods: sun shots, angles to geographical features, aircraft data, ice breaker data, the platform location system of CLS America (Service ARGOS), and the Global Positioning System.  AWS elevation is obtained by barometry and UNAVCO Global Positioning System (GPS) and should be correct to within +/- 5 meters.  Site names were introduced for convenience. Table 3.1 lists the site name, ARGOS identification number, latitude, longitude, elevation, start date for the site, and the World Meteorological Organization (WMO) number for the site.  Figures 2, 3, and 4 show the locations of the AWS units in the Antarctic for 2001.

The ARGOS identification number (ID) is used to identify the data sets distributed to the users.  AWS units are sometimes moved from one location to another, and as a result, the ID at a given site may change from year to year. The site name does not change. Table 3.2 lists the site name with the ARGOS ID, the site start date, and the ID start and stop dates.

Figure 1.  Layout of the AWS unit used in the Antarctic.  The installed AWS unit has a 3-meter tower with a horizontal boom supporting the antenna, aerovane for measuring wind speed and direction, air temperature resistance thermometer, upper thermopile for measuring vertical air temperature difference, and the relative humidity sensor.  The electronics enclosure is mounted at the midpoint of the tower.  The gel cell batteries are placed at the tower base.  The solar panel, located near the tower top, faces north. The Acoustic Depth Gauge (ADG) is installed on some of the AWS units to measure snow depth for precipitation studies.

# Table 3.1

AWS site name, geographic location and elevation, site start date, and WMO number for 2001.

| Site Name | ARGOS ID | Lat. (deg ) | Long. (deg) | Elev. (m) | Site Start Date | WMO No. |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | ## Adelie Land |  |  |  |
| D-10 | 8914 | 66.71oS | 139.83oE | 243 | 08 Jan 80 | 89832 |
| D-47 | 8986 | 67.397oS | 138.726oE | 1560 | 24 Jan 83 | 89834 |
| Dome C II | 8989 | 75.121oS | 123.374oE | 3250 | 12 Dec 95 | 89828 |
| Port Martin | 8909 | 66.82oS | 141.39oE | 39 | 19 Jan 90 |  |
| Cape Denison | 8988 | 67.009oS | 142.664oE | 31 | 20 Jan 90 |  |
| Penguin Point | 8910 | 67.617oS | 146.180oE | 30 | 24 Dec 92 | 89847 |
|  |  |  | ## West Antarctica |  |  |  |
| Byrd Station | 8903 | 80.007oS | 119.404oW | 1530 | 05 Feb 80 | 89324 |
| Mount Siple | 8981 | 73.198oS | 127.052oW | 230 | 20 Feb 92 | 89327 |
| Theresa | 21358 | 84.599oS | 115.811oW | 1463 | 29 Nov 94 | 89314 |
| Doug | 8922 | 82.315oS | 113.240oW | 1433 | 29 Nov 94 |  |
| Elizabeth | 21361 | 82.607oS | 137.078oW | 519 | 30 Nov 94 | 89332 |
| Brianna | 21362 | 83.889oS | 134.154oW | 526 | 30 Nov 94 |  |
| Harry | 8900 | 83.003S | 121.393W | 945 | 29 Nov 94 | 89329 |
| Erin | 21363 | 84.904oS | 128.828oW | 990 | 29 Nov 94 |  |
| Siple Dome | 8938 | 81.656oS | 148.773oW | 668 | 21 Jan 97 | 89345 |
| Swithinbank | 21356 | 81.20oS | 126.174oW | 945 | 18 Jan 97 |  |
|  |  |  | ## Ross Island Region |  |  |  |
| Marble Point | 8906 | 77.439oS | 163.754oE | 108 | 05 Feb 80 | 89866 |
| Ferrell | #8929 | 77.910oS | 170.817oE | 45 | 10 Dec 80 | 89872 |
| Pegasus North | 21357 | 77.952oS | 166.500oE | 8 | 23 Jan 90 | 89667 |
| Pegasus South | 8937 | 77.99oS | 166.576oE | 10 | 14 Jan 91 |  |
| Minna Bluff | 8935 | 78.554oS | 166.691oE | 895 | 22 Jan 91 | 89768 |
| Linda | 8919 | 78.464oS | 168.382oE | 47 | 21 Jan 91 | 89769 |
| Willie Field | #21364 | 77.865oS | 167.017oE | 40 | 25 Jan 92 |  |
| Windless Bight | 8927 | 77.728oS | 167.703oE | 61 | 09 Dec 98 |  |
| Herbie Alley | 8697 | 78.10oS | 166.67oE | 30 | 11 Jan 99 |  |
| Cape Spencer | 8722 | 77.97oS | 167.55oE | 30 | 11 Jan 99 |  |
| Cape Bird | 8901 | 77.224oS | 166.440oE | 42 | 28 Jan 99 |  |
| Laurie II* | #21360 | 77.549oS | 170.817oE | 30 | 01 Feb 00 |  |
|  |  |  | ## Ocean Islands |  |  |  |
| Whitlock | #8907 | 76.144oS | 168.392oE | 274 | 01 Jan 82 | 89865 |
| Possession Is. | 8984 | 71.891oS | 171.210oE | 30 | 29 Dec 92 | 89879 |
| Manuela | 8905 | 74.946oS | 163.687oE | 78 | 06 Feb 84 | 89864 |
|  |  |  | ## Ross Ice Shelf |  |  |  |
| Marilyn | #8934 | 79.954oS | 165.130oE | 75 | 16 Jan 84 | 89869 |
| Schwerdtfeger | 8913 | 79.904oS | 169.973oE | 60 | 24 Jan 85 | 89868 |
| Gill | 8911 | 79.985oS | 178.611oW | 55 | 24 Jan 85 | 89376 |
| Lettau | 8908 | 82.518oS | 174.452oW | 55 | 29 Jan 86 | 89377 |
| Elaine | 8915 | 83.134oS | 174.169oE | 60 | 28 Jan 86 | 89873 |

| Site Name | ARGOS ID | Lat. (deg) | Long. (deg) | Elev. (m) | Site Start Date | WMO No. |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | ## Antarctic Peninsula |  |  |  |
| Larsen Ice | 8926 | 66.949oS | 60.897oW | 17 | 07 Feb 83 | 89262 |
| Butler Island | 8902 | 72.207oS | 60.160oW | 91 | 01 Mar 86 | 89266 |
| Uranus Glacier | 8920 | 71.43oS | 68.93oW | 780 | 06 Mar 86 | 89264 |
| Limbert | 8925 | 75.422oS | 59.851oW | 40 | 30 Nov 95 | 89257 |
| Racer Rock | 8947 | 64.067oS | 61.613oW | 17 | 15 Oct 89 | 89261 |
| Bonaparte Pt. | 8923 | 64.778oS | 64.067oW | 8 | 05 Jan 92 | 89269 |
| Sky-Blu | 8917 | 74.972oS | 71.488oW | 1510 | 07 Feb 99 | 89272 |
| Kirkwood Island* | #8930 | 68.340oS | 69.007oW | 30 | 26 May 01 |  |
| Dismal Island* | #8932 | 68.087oS | 68.825oW | 10 | 27 May 01 |  |
|  |  |  | ## High Polar Plateau |  |  |  |
| Clean Air | 8987 | 90.00oS |  | 2835 | 29 Jan 86 | 89208 |
| Henry | 8985 | 89.011oS | 1.025oW | 2755 | 26 Jan 93 | 89108 |
| Nico | 8924 | 89.00oS | 89.669oE | 2935 | 26 Jan 93 | 89799 |
| Relay Station | 8918 | 74.017oS | 43.062oE | 3353 | 01 Feb 95 | 89744 |
| Dome Fuji | #8904 | 77.31oS | 39.70oE | 3810 | 08 Feb 95 | 89734 |
| Mizuho | 21359 | 70.70oS | 44.29oE | 2260 | 07 Oct 00 |  |

* New sites started during 2001

# New ARGOS ID for 2001 at the site

Figure 2.  Antarctic automatic weather station locations during 2001 identified by the site name. Area         around Ross Island is shown in Figure 3. Adelie Coast area is shown in Figure 4.

Figure 3.  Location of Antarctic automatic weather stations in the vicinity of Ross Island, Antarctica during 2001.

Figure 4.  Location of Antarctic automatic weather stations along the Adelie Coast during 2001.

- Table 3.2

2001 Antarctic AWS site name, ARGOS identification number (ID), site start date, ID start date, and ID stop date if occurring in 2001.

| Site | ARGOS ID | Site Start Date | ID Start Date | ID Stop Date |
| --- | --- | --- | --- | --- |
| D-10 | 8914 | 08 Jan 80 | 28 Jan 98 |  |
| D-47 | 8986 | 24 Nov 83 | 11 Feb 96 |  |
| Dome C II | 8989 | 12 Dec 95 | 12 Dec 95 |  |
| Port Martin | 8909 | 19 Jan 90 | 18 Dec 99 | 18 Dec 00 |
| Cape Denison | 8988 | 20 Jan 90 | 17 Dec 99 |  |
| Penguin Point | 8910 | 24 Dec 92 | 21 Dec 98 |  |
| Byrd Station | 8903 | 05 Feb 80 | 05 Feb 80 |  |
| Mount Siple | 8981 | 20 Feb 92 | 20 Feb 92 |  |
| Theresa | 21358 | 29 Nov 94 | 29 Nov 94 |  |
| Doug | 8922 | 29 Nov 94 | 20 Jan 97 |  |
| Brianna | 21362 | 30 Nov 94 | 30 Nov 94 |  |
| Harry | 8900 | 29 Nov 94 | 26 Jan 99 |  |
| Elizabeth | 21361 | 30 Nov 94 | 17 Jan 96 |  |
| Erin | 21363 | 29 Nov 94 | 18 Jan 96 |  |
| Siple Dome | 8938 | 21 Jan 97 | 24 Jan 99 |  |
| Swithinbank | 21356 | 18 Jan 97 | 18 Jan 97 |  |
| Marble Point | 8906 | 05 Feb 80 | 05 Feb 80 |  |
| Ferrell | 21355 | 10 Dec 80 | 01 Feb 00 | 09 Jan 01 |
|  | 8929 |  | 09 Jan 01 |  |
| Pegasus North | 21357 | 23 Jan 90 | 03 Feb 00 |  |
| Pegasus South | 8937 | 14 Jan 91 | 14 Jan 91 |  |
| Minna Bluff | 8935 | 22 Jan 91 | 25 Jan 99 |  |
| Linda | 8919 | 21 Jan 91 | 15 Jan 98 |  |
| Willie Field | 8929 | 25 Jan 92 | 16 Jan 99 | 29 Jan 01 |
|  | 21364 |  | 29 Jan 01 |  |
| Windless Bight | 8927 | 09 Dec 98 | 25 Jan 99 |  |
| Herbie Alley | 8697 | 11 Jan 99 | 11 Jan 99 |  |
| Cape Spencer | 8722 | 11 Jan 99 | 11 Jan 99 |  |
| Cape Bird | 8901 | 28 Jan 99 | 28 Jan 99 |  |
| Laurie II | 21364 | 01 Feb 00 | 01 Feb 00 | 16 Jan 01 |
|  | 21360 |  | 16 Jan 01 |  |
| Whitlock | 8907 | 01 Jan 82 | 07 Feb 01 |  |
| Possession Island | 8984 | 29 Dec 92 | 29 Dec 92 |  |
| Marilyn | 8931 | 16 Jan 84 | 18 Jan 91 | 30 Jan 01 |
|  | 8934 |  | 30 Jan 01 |  |
| Schwerdtfeger | 8913 | 24 Jan 85 | 22 Jan 93 |  |
| Gill | 8911 | 24 Jan 85 | 25 Jan 91 |  |
| Elaine | 8915 | 28 Jan 86 | 02 Jan 97 |  |
| Lettau | 8908 | 29 Jan 86 | 29 Jan 86 |  |
| Manuela | 8905 | 06 Feb 84 | 15 Feb 87 |  |
| Larsen Ice Shelf | 8926 | 07 Feb 83 | 01 Jan 86 |  |
| Butler Island | 8902 | 01 Mar 86 | 01 Mar 86 |  |
| Uranus Glacier | 8920 | 06 Mar 86 | 06 Mar 86 |  |
| Limbert | 8925 | 30 Nov 95 | 30 Nov 95 |  |
| Racer Rock | 8947 | 15 Oct 89 | 08 Dec 91 |  |
| Bonaparte Point | 8923 | 05 Jan 92 | 23 Dec 96 |  |
| Sky-Blu | 8917 | 07 Feb 99 | 07 Feb 99 |  |
| Kirkwood Is. | 8930 | 26 may 01 | 26 May 01 |  |
| Dismal Is. | 8932 | 27 May 01 | 27 May 01 |  |
| Clean Air | 8987 | 29 Jan 86 | 25 Jan 94 |  |
| Henry | 8985 | 26 Jan 93 | 26 Jan 93 |  |
| Nico | 8924 | 26 Jan 93 | 26 Jan 93 |  |
| Relay Station | 8918 | 01 Feb 95 | 01 Feb 95 |  |
| Dome Fuji | 8904 | 08 Feb 95 | 04 Feb 97 |  |
| Mizuho | 21359 | 07 Oct 00 | 07 Oct 00 |  |

4. AWS DATA SUMMARIES

The data received by the University of Wisconsin, Space Science and Engineering Center, contain all the information received by the ARGOS system including duplicate and erroneous transmissions.  Invalid data are eliminated during a quality check, and the valid data are converted to scientific units producing the complete data set.  Data selected at three hourly intervals, plus or minus forty minutes, produce a three hourly data set for each AWS unit month.  Section 6.1, AWS Performance, provides some explanations for missing and invalid data.

Use of the 2001 Antarctic AWS data for publication should acknowledge the support of NSF-OPP Grants `0088058 and 0338147 or reference this publication.

- 4.1.  Monthly Data Summaries

The monthly summaries consist of the monthly means, from the three hourly data set, and the extremes, from the complete data set.  For monthly values to be included, 25% of the three hourly observations must be available.  Months with 50-75% of data missing occur most often when a station is started or stopped in the middle of the month.  This can cause a bias in the monthly mean, especially during seasons when parameters such as temperature change rapidly.  Annual means are calculated only when twelve months of data are available.  The data are presented in the same order as the sites listed in Table 3.1.  Definitions of the monthly data summary headings are listed below.

Heading                                                Definition

Mean air temperature, oC.	Mean value for the month.

Percent of monthly data missing. 	Ratio of the number of missing observations to the number of

possible observations X 100.

Maximum air temperature, oC.	Maximum value for the month.

Minimum air temperature, oC.	Minimum value for the month.

Mean wind speed, m/s. 		            Mean value for the month.

Percent of monthly data missing.	Ratio of the number of missing observations to the number of

possible observations X 100.

Resultant wind speed, dir/vv.	Resultant speed and direction for the month.

Constancy.                  		Ratio of the monthly resultant to the monthly mean wind speed.

Maximum wind speed, dir/vv.	Maximum wind speed and direction for the month.

Mean air pressure, mb.		Mean value for the month.

Percent of monthly data missing.	Ratio of the number of missing observations to the number of

possible observations X 100.

Maximum air pressure, mb.	Maximum value for the month.

Minimum air pressure, mb.	Minimum value for the month.

Potential temperature, K                    Mean value for the month.

4.2. Three Hourly Data Summaries

After the data are received from Service ARGOS, ten minute interval data are created for each AWS unit. The data are calibrated for the individual station instruments, but no other corrections are made. This data set is created for those users who need fairly current information. These data are available by anonymous FTP (see Section 8).

The 10 minute data set for each AWS unit for the month is scanned to pick out the nearest observation within forty minutes of the UTC hours 00, 03, 06, 09, 12, 15, 18, and 21 to produce the three hourly data set. If valid data are not available within forty minutes of the three hourly time interval, then the entry is left blank to indicate missing data. The means, standard deviations, resultant wind speed and direction, the distribution of temperature, and wind speed with wind direction are determined from the three hourly observations and are presented as a monthly summary at the bottom of each page. A wind direction value of zero indicates a wind speed less than 0.50 m s-1.  North is indicated by a value of 360 degrees. The maximum and minimum values are taken from the complete 10 minute data set, not the three hourly data set. The appropriate monthly data from the three hourly data set are used for the monthly summaries presented in Section 4.1. In the presence of sunlight the air temperatures are questionable if the wind speed is less than 1 m s-1. These summaries are available by anonymous FTP (see Section 8).  If you are unable to access the Internet, we will send the information either on diskettes or paper. Please contact us for further information (the address is at end of Section 8).

5.  AWS CALIBRATION

- 5.1. Temperature

The external and internal temperatures are calibrated using a 1000 ohm 0.05% resistor in place of the platinum resistance thermometers with 1000 ohms resistance at 0oC.  Because the other resistances in the temperature circuit are known only to 1%, the temperature calibration will vary from one electronic unit to another.  The correction factor determined from the calibration resistor is programmed into the read-only-memories for each unit.  After the correction factors have been programmed into the AWS, a calibration box with 0.1% resistors is used to check the temperature calibration.

- 5.2. Pressure

In 2001 pressure calibration was performed by using a Paroscientific portable pressure standard model 760-16B. It has a 0.0001% resolution (<1 microbar resolution) with a ±0.08 hPa or better accuracy and a 0.1 hPa/year or better stability over the pressure range of 500 hPa to 1100 hPa.  There is ISO 9001:2000 compliance and is a NIST traceable standard.

Comparisons are made between AWS units and with the mercury barometers in the Crary Laboratory at McMurdo, Antarctica.  The calibrations should be within +/- 0.2 hPa.

The reference vacuum on the older pressure transducers can degrade with time with a maximum observed 4 hPa shift to lower pressure after fifteen years. Recalibration of each pressure transducer would be desirable when each unit is serviced.

- 5.3. Wind direction and Speed

The Belfort model 123 aerovane measures wind direction and speed.  The aerovane rotates a potentiometer wiper, and the fraction of full scale of the potentiometer is measured.  The wind direction is checked by positioning the aerovane to the cardinal directions relative to the boom supporting the aerovane.  North or the potentiometer zero is towards the antenna on the boom and has a maximum dead zone of 3o.  During the field installation, the boom is usually aligned along the north-south line as determined from the sun's azimuth, longitude, and Greenwich Mean Time.  In some cases the 180o end of the boom may point in a direction other than south.  At Manuela site, the 180o end of the boom points up the glacier and a correction is added to the data during processing.  At Byrd site the wind is usually out of the north so the boom was rotated 120o and the correction added during the data processing.  The wind speed is determined from the aerovane tachometer voltage output as 0.0472 volt per meter per second.  The aerovane tachometers are spun at 1800 rpm with a load of 1071.5 ohms, and the output should be 9.20 +/-0.05 vdc.

Additional wind sensors were used with AWS units for 2001.  These were the R.M. Young wind monitor model 05103 and the Hydro-Tech WS-3 anemometer with the WP-3 aerovane.  The Hydro-Tech system was used for measuring wind speed in the Adelie Coast area. The Hydro-Tech WS-3 is a disk rotor, 3 in. high and 12 in. overall diameter, with radial cups, and the threshold sensitivity is 3 mph.  The anemometer utilizes a commercial dc tachometer generator.  Output is 0 to +5 vdc (and 0 to 1 ma) over the desired full scale wind speed of 85 m/s.  Accuracy is +/- 2%.

The R.M. Young monitor 05103 also used a 10,000 ohm potentiometer with a 3o dead zone so that the wind direction was recorded identically with the Belfort/Bendix aerovanes.  The wind speed was from the range of 0 to 1.0 volt full scale corresponding to 50 m/s.  Thus the calibration for wind speed was a nominal .195 m/s/bit for the R.M. Young with +/- 1% up to 50 m/s.

- 5.4. Relative Humidity

The Vaisala HMP-35A/45A humidity sensor output voltage varies linearly with relative humidity (U).  The sensor is calibrated by placing it over saturated salt solutions with known relative humidities at room temperature:  sodium chloride (U=75%), and lithium chloride (U=12%) are used.  In addition, a dry inert gas, forced past the sensor, gives a 0% U, and the sensor output can be zeroed.  Then, the gain setting can be set directly using a salt solution with a high relative humidity, such as sodium chloride.  The resolution of the humidity sensor is about 1% and the drift is 2 to 3% per year in the field.  The relative humidity data are not included on the summary pages but are included in the 3 hourly data sets.

- 5.5. Vertical Air Temperature Difference

Two junction thermocouples are used to measure the air temperature difference between 3 m and 0.5 m on the tower.  The output is about 78 microvolts for 1oC temperature difference between the junctions at 0.0oC, dropping to 60 microvolts at -80oC.  Zero output is adjusted to 0.4 volts, so that 0 to 1 volt corresponds to a -6oC to +9oC range of air temperature differences between 3 m and 0.5 m.  The resolution is 0.05oC.  Calibration of the individual systems is done by applying known voltages to the amplifier input. The vertical temperature difference data are not included on the summary pages but are included in the 3 hourly data sets.

- 6.  AWS OPERATIONS SUMMARY FOR 2001

- 6.1.  AWS Performance

Forty-eight AWS units were installed at the start of 2001 and 51 were installed by the end of 2001.  Based on the installation months the AWS units delivered 91% of the temperature data, 88% of the pressure data and 82% of the wind data during 2001. Complete data sets were received from 5 AWS units and 40 AWS units operated for the installed period. Seven AWS units were not received for one month or more during the year or stopped during the year.  Some of the stations were not received during the winter months due to low battery voltage.

The wind system has the poorest performance. If the wind speed is zero or the wind direction is constant for extended periods (days to months) then the data is considered invalid. The reason for this behavior is not known but is believed to be due to the build up of frost on the wind system. This usually occurs in the winter season and at several AWS sites. The wind speed is most frequently zero when the wind direction is constant.

Site	Performance

D-10 			The station transmitted erratically from July through the beginning of October. The relative humidity sensor and delta-T sensor were not functioning.

D-47			Temperature sensor not functioning. Station stopped transmitting 4 January and began again 27 February. Station transmitted erratically from April through early August with another break the first half of September. No relative humidity or delta-T sensors.

Dome C II		OK.

Port Martin		The aerovane was not functioning. No delta-T or extra high wind speed sensor. Pressure corrected for high wind speed conditions.

Cape Denison		Station stopped transmitting 29 April and resumed 27 May. Several transmission gaps of a week or more in April, May, and September. Pressure corrected for high wind speed conditions.

Penguin Point		The station stopped transmitting from 15 May to 21 June. Pressure corrected for high wind speed conditions.

Byrd			Station transmitted from 3 to 8 October only.

Mount Siple		Site has a "dog house" AWS without wind speed and direction. Pressure sensor 				not functioning correctly January and February.

Theresa		Delta-T sensor buried.

Doug			Aerovane not functioning in July. Relative humidity sensor not functioning. Station transmitted erratically in September and stopped transmitting on 8 October.

Elizabeth		Aerovane "frozen" the last part of September.

Brianna			The relative humidity sensor functioned erratically after June. The station stopped transmitting 3 November.

Harry			The delta-T sensor is not functioning. The aerovane was "frozen" occasionally from June through November.

Erin			The aerovane was "frozen" occasionally from May through July.

Siple Dome		Aerovane “frozen” occasionally from April through November. Delta-T sensor not installed.

Swithinbank		Station stopped transmitting 8 October. Delta-T sensor not functioning.

Marble Point		OK.

Ferrell			New station installed 10 January.

Pegasus North		Station stopped transmitting on 25 January. Delta-T not functioning. Station resumed transmitting 4 November.

Pegasus South		Station stopped transmitting on 6 May and resumed on 24 June. Relative humidity sensor not functioning. Aerovane “frozen” from June through October. Delta-T erratic all year.

Minna Bluff		Station started transmitting 10 July. Delta-T sensor not functioning. Aerovane occasionally “frozen” July through November. Station stopped transmitting 13 December.

Linda			Aerovane occasionally "frozen" April through October.

Willie Field		Station replaced 29 January.

Windless Bight		Station stopped transmitting 12 December due to low battery voltage.

Herbie Alley		Pressure functioned erratically August,  October, and November.

Cape Spencer		Station stopped transmitting 25 December.

Cape Bird		OK.

Laurie II		Station replaced 17 January. Delta-T sensor not functioning. Aerovane occasionally "frozen" in September.

Whitlock		Station replaced 7 February. Delta-T sensor not functioning. Aerovane “frozen”  most of April and May and parts of June and July.

Possession Island	Site has a "dog house" AWS without wind speed and direction.

Manuela		Aerovane broken.

Marilyn			Station replaced 30 January. Aerovane occasionally "frozen" May through November.

Schwerdtfeger		Relative humidity sensor not functioning from mid February through November Aerovane “frozen” occasionally in July.

Gill			Relative humidity sensor not functioning. Aerovane “frozen” occasionally in October

Lettau			Transmitting with many gaps in January, November, and December. Aerovane “frozen” occasionally from March through December.

Elaine			OK.

Larsen Ice Shelf	Aerovane “frozen” occasionally in July.

Butler Island		Aerovane not functioning beginning of January and “frozen” occasionally from May through November. Pressure continues to need correction due to the failure of the precision time-based correction to the system clock.

Uranus Glacier		Large gaps in transmission in March, November, and some of December.

Limbert			Delta-T sensor not functioning. Aerovane not functioning. Station stopped transmitting in July and resumed in September due to low battery voltage.

Racer Rock		Station transmitted erratically from the end of May to October due to low battery voltage. Station stopped transmitting 30 November.

Bonaparte Point		Relative humidity and delta-T sensor not functioning. Aerovane not functioning January through May, July, and October through December. Pressure is too low and has been removed.

Sky Blu			Aerovane “frozen” occasionally in September.

Kirkwood Island		New station installed 21 May. Aerovane not functioning properly in May.

Dismal Island		New station installed 22 May. Aerovane not functioning properly in May.

Clean Air		Occasional problems with relative humidity sensor. Pressure functioning erratically February through November.

Henry			Aerovane occasionally "frozen" June through September. Station stopped transmitting 20 September and resumed transmitting 30 October as the battery recharged in the austral spring.

Nico			Aerovane occasionally "frozen" in August and September.

Relay Station		OK.

Dome Fuji		Repaired station installed on 22 December.

Mizuho			No relative humidity or delta-T sensors. Occasional gaps in transmission during winter months.

- 6.2.  AWS Antarctic Field Activities

# McMurdo area

Pegasus North (8927) was visited on 7 January by Jonathan Thom, Rob Flint, and Ernie Mastroianni. The station was operating nominally, but the RPSC airfield manager requested that the station be relocated. The old location was directly in the middle of the approach apron for the runway. The station was removed, and the following day another station (8927) was installed about 1 mile southeast of the old location. On 9 January, Ferrell (21355) site was visited by Jonathan Thom and Rob Flint. The station and boom were removed and replaced with 8929. On 16 January, Laurie II (21364) was visited by Jonathan Thom. The station was replaced with 21360. Willie Field (8929) site was visited by Jonathan Thom, Doug MacAyeal, and Andy Bliss on 29 January. The AWS had been removed earlier to provide a station for Ferrell site. AWS 21364 was installed and the station was raised. An R.M. Young wind monitor was also installed. On 30 January, Marilyn (8931) site was visited by Jonathan Thom, Andy Bliss, and Ben Kerman.The station was raised, a new AWS enclosure (8934) was installed, and 2 boxes of batteries were connected. The boom is 60 degrees off north in a counterclockwise direction.

On 7 February, Whitlock site was visited on Franklin Island by USCGC Polar Sea. MST3 David Otani oversaw the replacement of the AWS enclosure. The AWS unit is now 8907. The Polar Sea went on to Manuela (8905) site to fix the aerovane but were unable to spot the station. The helicopter had problems so another attempt was not made.

# West Antarctica

# No operations for 2001 due to limited resources of time and AWS.

# Polar Plateau

Dome Fuji (8904) was reinstalled 22 December by members of JARE.

# Adelie Coast

No field work due to lack of resources.

# Antarctic Peninsula

The Southern Ocean GLOBEC group installed Kirkwood Island (8930) AWS unit on 26 May and Dismal Island AWS unit on 27 May. The Palmer Station Science Technician, John Booth, attempted to repair the Bonaparte Point AWS unit (8923) on 12 May.

7.  GLOBAL TELECOMMUNICATIONS SYSTEM

The data from 35 Antarctic AWS units were entered into the Global Telecommunications System (GTS) during 2001. The data are collected by CLS America (Service ARGOS). As soon as the data are received, CLS America (Service ARGOS) processes them and sends them on to the National Weather Service which distributes the data to the GTS.The data headers are:

- SMAA14 KARS YYGGgg

SIAA14 KARS YYGGgg

SNAA14 KARS YYGGgg

where S indicates surface, M is main observations (at 00, 06, 12, and 18 UT), I is intermediate observations (at 03, 09, 15, and 21 UT), and N is any other time. AA14 is for Antarctica, and KARS stands for the Largo receiving center (backup is LFPW for the center in Toulouse, France). YY indicates the day in the month, GG is the hour, and gg is the minutes. Table 3.1 contains the WMO identification number used by the GTS grouped according to their purpose and proximity where possible.

The University of Wisconsin-Madison is responsible for obtaining WMO numbers for AWS sites and for providing CLS America (Service ARGOS) with calibration information for processing the data. The main reason for getting the AWS data into the GTS is to make sure that the data are available in near real time for all organizations operating in Antarctica and for numerical weather prediction models.

- 8.  DATA AVAILABILITY

The data from our Automatic Weather Stations are available by anonymous FTP.  The IP address and domain name are 128.104.109.33 (ice.ssec.wisc.edu).  The login is "anonymous" (do not use the quotation marks), and the password is your email address.  Once you have logged in, change to the pub/aws subdirectory.  A listing of our station locations, names, and ARGOS ID numbers is located in the file "biglist" in this subdirectory.  It is meant to serve as a guide to our stations as their ID numbers sometimes change. A complete guide for navigating the site may be found in the file “readme.faq”.

Our three-hourly interval data for Antarctica in ASCII text format are contained in the year subdirectories of pub/aws/antrdr.  The data have been corrected, i.e. an effort has been made to remove the bad data points.  These data take longer to process, so the data for recent months are not available in this format.  Within each of the year subdirectories of pub/aws/antrdr, there are text files named "3hrlist??" (where ?? indicates the last two digits of the year). These files list what station's data are contained in which files. The files “readme.updates?” in pub/aws/antrdr contain information on updates and/or corrections to the data, and the file “readme.3format” contains file name construction information and format of the three-hourly data. The file “readme.mailinglist” contains information on joining a mailing list which distributes information on data updates and changes. To subscribe, send email to aws@ssec.wisc.edu requesting to be added to our mailing list.

The directory pub/aws/summary/monthly contains ASCII printable text files of the paper data summary sheets.  The format of the files can be found in the file "readme.sum" while updates and corrections to the data are located in "readme.sumupdates".  The data are located in year subdirectories of pub/summary/monthly.

For those users who need more current information, we have created 10 minute interval data in ASCII text format for each station.  These data are located in year subdirectories of pub/aws/10min/rdr.  The data have been calibrated for the individual station instruments, but no other corrections have been made.  The data are generally available up to and including the last full month of this year.  The year subdirectories also contain a text file named "namelist??" (where ?? indicates the last two digits of the year in question).  These files list what station's data are contained in which files.

Several important readme files are located in pub/aws/10min/rdr. The file "readme.format" contains information on filename construction of the data, as well as well as file content and is a must for those unfamiliar with the data. The files "readme.updates?" contain important information on changes/additions to the data.

Our site is available 24 hours a day, 7 days a week.  If you have questions or problems, send email to aws@ssec.wisc.edu.  We can also be reached by phone at (608) 265-2209 or (608) 265-4816 or fax at (608) 262-5947.

By mail, please contact:

Linda M. Keller

University of Wisconsin

Department of Atmospheric and Oceanic Sciences

1225 W. Dayton St.

Madison, WI  53706

USA

or

Shelley Knuth

University of Wisconsin

Space Science and Engineering Center

1225 W. Dayton St.

Madison, WI  53706

USA

- 9.  ACKNOWLEDGMENTS

This work is supported by the National Science Foundation, Office of Polar Programs, Grants 0088058 and 0338147 under the management of Dr. Bernhard Lettau. Assistance from the following countries is gratefully acknowledged. Expeditions Polaires Francaises installs and maintains the AWS units from the Adelie Coast to Dome C II.  The British Antarctic Survey maintains the AWS units on the east side of the Antarctic Peninsula and south of Adelaide Island. The Japanese Antarctic Research Expedition (JARE) installs and maintains the AWS units at Relay Station, Mizuho, and Dome Fuji.

Dr. S.P. Adhikary, Dir. General	Mary R. Albert	Prof. Richard Alley

Dept. of Hydrology & Meteorology	U.S.A. CRREL	The Pennsylvania State University

P.O. Box No. 406	72 Lyme Road	Earth Systems Science Center

Babarmahal, Kathmandu	Hanover, NH  03755-1290	306 Deike Building

NEPAL	University Park, PA  16802

Dr. I. Allison	Dr. Sridhar Anandakrishnan	Dr. Jean-Claude Andre

Antarctic CRC	The Pennsylvania State University	Centre National de Recherches

University of Tasmania	Earth Systems Science Center	Meteorologiques

Hobart, Tasmania  7001	540 Deike Building	42 Avenue G. Coriolis

AUSTRALIA	University Park, PA  16802	F-31057 Toulouse Cedex

FRANCE

Dr. Gail Ashley	Liana Bacigalupi	Dr. Karen S. Baker

Rutgers University	Library	Scripps Institute of Oceanography

Dept. of Geological Sciences	ENEA Progetto Antartide	UCSD - A - 018

Piscataway, NJ  08854	Via Anguillarese, 301	La Jolla, CA  92093

00060 S. Maria di Galeria (Roma)

ITALY

Prof. Roger Bales	Dr. L.A. Barrie	Ian J. Barton

University of Arizona	Atmospheric Environment Service	CSIRO Marine Laboratories

Dept. of Hydrology & Water Resources	4905 Dufferin Street	GPO Box 1538

Tuscon, AZ  85721	North York, Ontario	Hobart, Tasmania 7001

M3H 5T4	AUSTRALIA

CANADA

Rabindra Basnyat	Rob Bauer	Ms. Judy Bausch

General Post Office	NSIDC/CIRES	Yerkes Observatory

P.O. Box #6644	University of Colorado	P.O. Box 258

Kathmandu	Boulder, CO  80309-0449	Williams Bay, WI  53191

NEPAL

Dr. John Bengston	Prof. Charles R. Bentley	Biblioteca Y Centro de Documentacion

NOAA/NMFS	University of Wisconsin	Dir. Meteorologica de Chile

National Marine Mammal Lab.	Geophys. & Polar Research Cen.	Casilla 717

7600 Sand Pt. Way N.E.	1215 W. Dayton St.	Santiago

Seattle, WA  98115-0070	Madison, WI  53706	CHILE

Dr. Robert Bindshadler	Prof. Krzystztof Birkenmajer, Chrm.	Dr. George Blaisdell

NASA	Com. On Polar Res., Polish Academy of 	National Science Foundation

Goddard Space Flight Center	Sciences	Office of Polar Programs

Code 971	Dept. of Dynamic Geology	Room 755

Greenbelt, MD  20771	Instit. of Geological Sciences	4201 Wilson Blvd.

ul. Senacka 1-3 31-002 Krakow	Arlingtion, VA 22230

POLAND

Mr. Greg Bodeker	Dr. John Bolzan	Dr. Scott Borg

University of Natal	Ohio State University	National Science Foundation

Space Physics Research Instit.	Byrd Polar Research Center	Office of Polar Programs

King George V. Ave.	1090 Carmack Road	Room 755

Durban 4001	Columbus, OH  43210-1002	4201 Wilson Blvd.

SOUTH AFRICA	Arlington, VA  22230

Irina I. Borzenkova	Prof. David Braaten	John W. Briggs, Engineer

State Hydrological Institute	University of Kansas	Yerkes Observatory

23 Second Line	Dept. of Physics and Astronomy	373 West Geneva Street

199053, St. Petersburg	Lawrence, KS  66045	Williams Bay, WI  53191-0258

RUSSIA

Dr. David Bromwich	Neva Brown	Giancarlo Caimi

Ohio State University	30 Lantern Lane	Via S. Benelli 8

Byrd Polar Research Center	Wrentham, MA  02093	20151

1090 Carmack Road	Milano

Columbus, OH  43210-1002	ITALY

Prof. Azzio Caneva	Captain, R/V Lawrence G. Gould	Captain, R/V Nathaniel B. Palmer

Universita di Genova	c/o AGUNSA	c/o AGUNSA

Stazione Geofisica	Deposito Franco Antarctico	Deposito Franco Antarctico

12080 Roburent	Av. Independencia 772	Av. Independencia 772

Cuneo	P.O. Box 60-D Punta Arenas	P.O. Box 60-D Punta Arenas

ITALY	CHILE	CHILE

Dr. A.M. Carleton	Prof. John Carroll	I.F.S. Cavalacanti

Dept. of Geography	University of California	Depto Meteorologia, INPE

The Pennsylvania State University	Dept. LAWR	Caixu Postal 515

0314 Walker Building	Davis, CA  95616	12 200 Sao Jose dos Campos

University Park, PA  16802	Sao Paulo

BRAZIL

Art Cayette	Roberto Cervellati	Prof. Chen Liqi, Director

SPAWAR System Center	ENEA Pas Progetto Antarctide	Chinese Antarctic Administration

P.O. Box 190022	S.S. Anguillarese 301	No. 1 Fuxingmenwai Street

North Charleston, NC  29419-9022	Roma	Beijing

ITALY	CHINA

Dr. K.Y. Chen	Mr. Erick Chiang	Pecheng Chu

University of Florida	National Science Foundation	Naval Postgrad. School, Code OC/CU

Dept. of Astronomy	Office of Polar Programs	Dept. of Oceanography

211 Space Sci. Research Building	Room 755	Monterey, CA  93943

Gainesville, FL  32611	4201 Wilson Blvd.

Arlington, VA  22230

Dr. Josefino C. Comiso	The Committee on Antarctic Research	Richard Cram

NASA/Goddard Space Flight Center	ul. Vavilova, 44	Federal Building

Code 971	Building 2	National Climatic Data Center

Greenbelt, MD  20771	Moscow 117333	Asheville, NC  28801

RUSSIA

Frederick E. Crory	Dick Crouthamel	CRREL-TL

US Army CRREL	National Weather Service	Dept. of the Army Corps of Engineers

72 Lyme Road	Mail Code W/OM31	Hanover, NH  03755-1290

Hanover, NH  03755-1290	1325 E-W Highway

Silver Spring, MD  20910

W. Darnell	Dr. Thomas DeFelice	Rear Ar. Fernando M. Diegues, Sec.

NASA, Langley Research Center	University of Wisconsin	Comm. Interministerial para os Recursos

Mail Stop 185	Dept. of Geoscience, Lapham Hall	do Mar (CIRM)

Hampton, VA  23665-5225	P.O. Box 413	Ministerio da Marinha

Milwaukee, WI  53201	40 ANDAR

70055 DF-Brasilia

BRAZIL

Debra Diemand	Director	Dr. David J. Drewry

CRREL-TL	Cold And Arid Regions	Vice-Chancellor

Dept. of the Army Corps of Engineers	Environ. and Engineering Res. Institute	University of Hull

Hanover, NH  03755-1290	Academia Sinica	Cottingham Road

Lanzhou 730000	Hull, Yorksire HU6 7RX

CHINA	UNITED KINGDOM

Mr. Michel Engler, Director	Dr. Hiroyuki Enomoto	Dr. Karl Erb

Expeditions Polaires Francaises	Kitami Institute of Technology	National Science Foundation

47 Ave. du Marechal Fayolle	Koencho 165	Office of Polar Programs

75116 Paris	Kitami, Hokkaido	Room 755

FRANCE	090-8507  JAPAN	4201 Wilson Blvd.

Arlington, VA  22230

Edelberto R. Farias, Director	Jane Ferrigno	Michel Fily

Dir. Meteorologica de Chile	USGS National Center	Lab. de Glaciologie et Geophys. de

Casilla 717-STGO	Mail Stop 927	L'Environnement

CHILE	Reston, VA  22092	BP 96

38402 Saint-Martin-d'Heres

FRANCE

Margaret Finger, Librarian	Dr. Joan Fitzpatrick	Dr. Joseph Fletcher

University of Melbourne	Federal Center	NOAA/ERL R/E

Dept. of Meteorology	U.S. Geological Survey-Denver	325 Broadway

Parkville, Victoria  3052	P.O. Box 25046	Boulder, CO 80303

AUSTRALIA	Denver, CO  80225

Dr. R.J. Flint	Dr. Benson Fogle	Prof. Theodore D. Foster

185 Bear Gulch Road	7 Park Overlook Court	Scripps Institute of Oceanography

Woodside, CA  94062	Bethesda, MD  20817	Mail Code 0240

University of California, San Diego

LaJolla, CA  92093-0240

Andrew G. Fountain	E. Imre Friedmann	Prof. H. Fukunishi

Dept. of Geology	Polar Desert Research Center	Tohoku University

Portland State University	Dept. of Biological Science (B-142)	Upper Atmos. & Space Research

P.O. Box 751	Florida State University	Sendai 980-0845

Portland, OR 97207-0751	Tallahassee, FL  32306-2043	JAPAN

Gen. Ricardo Galarza	Christophe Genthon	Abbate Giulia

Uruguayan Antarctic Institute	Lab. de Glaciol. et Geophys. de l'Env.	ENEA Casaccia

Buenos Aires 350	54 Rue Moliere, DU BP 96	Italian Antarctic Project

Montevideo	F-38402 Saint Martin d'Heres	Via Anguillarese 301

URUGUAY	Cedex	00100 Roma

FRANCE	ITALY

Hector R. Godoy	Goldthwait Polar Library	Robert Grumbine

Servicio Meteorologico National	Byrd Polar Research Center	5200 Auth Rd., Room 204

Depto Antarctico	Ohio State University	W/NMC21

25 de Mayo 658	1090 Carmack Road	Camp Springs, MD 20746

Capital Federal	Columbus, OH  43210-1002

REPUBLICA ARGENTINA

Anselm Haanen	Ritva Hanninen, Librarian	Inger Hanssen-Bauer

IEIS Prog. Consultant, c/-DSIR Antarctic	Finnish Meteorological Institute	Norwegian Meteorological Institute

Orchard Road	Box 503	P.O. Box 43 - Blindem

Christchurch International Airport	SF-00101 Helsinki 10	0313 Oslo 3

P.O. Box 14-091, Christchurch	FINLAND	NORWAY

NEW ZEALAND

Ross Harp, Jr.	Ralph Harvey	G. Hernandez

P.O. Box 1717	Dept. Geological Sciences	University of Washington

Boulder, CO 80306	Case Western Reserve University	Geophysics Program

112 A.W. Smith	AK-50

10900 Euclid Ave.	Seattle, WA  98195

Cleveland, OH  44106-7216

Dr. Roger Hewitt	Ms. Kathie Hill	Dr. Austin Hogan

Southwest Fisheries Center	Raytheon Polar Services	P.O. Box 21

Antarctic Ecosys. Research Gr.	7400 South Tuscon Way	Piermont, NH  03779-0021

P.O. Box 271	Centennial, CO 80112-3938

La Jolla, CA  92038

Hugh Hutchinson	Lloyd E. Irwin	T.H. Jacka

Bureau of Meteorology	NOAA, NWS	Antarctic CRC

20 Ellerslie Rd.	SSMC2, Station 5118	Box 252C

Hobart, TAS  7000	1325 East-West Highway	Hobart, Tasmania

AUSTRALIA	Silver Spring, MD  20910	AUSTRALIA 7001

Dr. Stanley Jacobs	Martin Jeffries	Dr. Ken Jezek

Columbia University	University of Alaska	Ohio State University

Lamont Observatory	Geophysical Institute	Byrd Polar Research Center

Palisades, NY  10964	903 Koyukuk Ave.	1090 Carmack Road

Fairbanks, AK  99705-7320	Columbus, OH  43210-1002

Prof. Jia Pengqun	P.D. Jones	Prof. Anders Karlqvist, Director

Polar Met. Lab., Acad. of Met. Sci.	University of East Anglia	Royal Swedish Academy of Science

State Meteorological Admin.	Climate Research Unit	Polar Research Secretariat

Baishiqiaolu No. 46, West Suburb	Norwich NR4 7TJ	Box 50005

Beijing,  100081	UNITED KINGDOM	10405 Stockholm

CHINA	SWEDEN

Stephen J. Katz	Chris Kidd	Dr. Tokio Kikuchi

Colorado School of Mines	School of Geography	Kochi University

Arthur Lakes Library/Serials	University of Birmingham	Dept. of Physics, Faculty of Sci.

Golden, CO  80401	Edgbaston, Birmingham	2-5-1, Akebono-cho

B15 2TT	Kochi 780-8072

UNITED KINGDOM	JAPAN

Dr. John C. King	Akio Kitoh	Dr. Yuji Kodama

British Antarctic Survey	Meteorological Research Institute	Hokkaido University

High Cross, Madingley Road	Climate Research Division	Institute of Low Temp. Science

Cambridge CB3 OET	Tsukuba, Ibaraki 305-0052	Sapporo 060-0819

UNITED KINGDOM	JAPAN	JAPAN

Dr. K.Ya Kondratyev	Dr. Els Kooij-Connally	Prof. Dr. Christopher Kottmeier

The USSR Acad. Inst. for Lake Res.	Library ECMWF	Universitat Karlsruhe

Sevastyanov Str., 9	Shinfield Park	Institut fur Meteorologie und

St. Petersburg,  196199	Reading, Berkshire RG2 9AX	Kaiserstr. 12 (Physikhochhaus)

RUSSIA	UNITED KINGDOM	76128 Karlsruhe

GERMANY

Dr. Steve Kottmeier	Prof. Dr. H. Kraus	Dr. Monica Kristensen

Raytheon Polar Services	Met. Institut der Universitat Bonn	Norwegian Meteorological Institute

7400 South Tuscon Way	D-5300 Bonn 1	Postboks Box 43 Blindem

Centennial, CO  80112-3938	FRG Auf Dem Hugel 20	0313 Oslo 3

GERMANY	NORWAY

Arlin Krueger	Dr. B.L. Krutskikh	Prof. M. Kuhn

NASA/Goddard Space Flight Center	Arctic & Antarc. Research Institute	Universitat Innsbruck

Code 616	St. Petersburg,  191104	Institut fur Met. & Geophysik

Greenbelt, MD  20771	RUSSIA	Innrain 52

A-6020 Innsbruck

AUSTRIA

Dr. Ron Kwok	Dr. Victor Lagun	Claude M. Laird

Senior Research Scientist	Air-Sea Interaction Dept.	University of Kansas

Jet Propulsion Laboratory	The Arctic & Antarc. Scientific Res. Instit.	2291 Irving Hill Drive

California Institute of Technology	38 Bering Street	Campus West

4800 Oak Grove Dr.	199 397 St. Petersburg	Lawrence, KS  66045

Pasadena, CA 91109	RUSSIAN FEDERATION

Jouko Launiainen	Anthony Lazzara	Brig. Gen. Jorge E. Leal

Finnish Instit. of Marine Research	110 Cherry Street	Dir. Nacional del Antarctico

P.O. Box 33	Wrentham, MA  02093	Cerrito 1248

00931 Helsinki	1010 Buenos Aires

FINLAND	ARGENTINA

Bang Yong Lee	Denyse Lemaire-Ronveaux	Dr. Marc Lessard

Korea Ocean Research & Dev. Instit.	Dept. des Sci. de la terre et de l'environ.	Thayer School of Engineering

Polar Research Center	Faculte des Sciences	Dartmouth College

Ansan P.O. Box 29	CP 160 Ave F.D. Roosevelt, 50	8000 Cummings

Seoul 425-600	1050 Brussels	Hanover, NH  03755-8000

SOUTH KOREA	BELGIUM

Dr. Bernhard Lettau	Librarian	Librarian

National Science Foundation	Australian Antarctic Division	British Antarctic Survey

Office of Polar Programs	Channel Highway	High Cross, Madingley Road

Room 755	Kingston, Tasmania  7050	Cambridge CB3 OET

4201 Wilson Blvd. Arlington, VA  22230	AUSTRALIA	UNITED KINGDOM

Library	Library	Library

Alfred Wegener Inst. fur Polar und 	Antarctica New Zealand	Climatic Research Unit

Meeresforschung	Private Bag 4745	University of East Anglia

Forschungsstelle Potsdam	Christchurch	Norwich NR4 7TJ

Telegraphenberg A 43	NEW ZEALAND	UNITED KINGDOM

14473 Potsdam

GERMANY

Library	Keith B. Mather Library	Library

Finnish Inst. of Marine Research	Geophysical Institute/IARC	National Inst. of Polar Research

Box 33	903 Koyukuk	9-10, Kaga 1-chome

SF 00931	Fairbanks, AK 99775	Itabashi-ku

Helsinki	Tokyo 173-8515

FINLAND	JAPAN

Library	Dr. David Limbert	Nancy Liston, Librarian

Tasmania/Antarctica Reg. Office	2 Topcliffe Way	U.S. Army CRREL

Bureau of Meteorology	Cambridge, CB1 4SH	Attn: Library

GPO Box 727G	UNITED KINGDOM	72 Lyme Road

Hobart, Tasmania 7001	Hanover, NH  03755-1290

AUSTRALIA

Michael P. Lizotte	Dr. C. Lorius	Dr. Douglas H. Lowenthal

800 Algoma Blvd.	C.R.N.S., Lab. de Glaciol. et Geophys. de 	Desert Research Institute

Oshkosh, WI  54901	L'Environ.	2215 Raggio Parkway

rue Moliere, BP 96	Reno, NV  89512-1095

38 402 Saint Martin D'Heres

Cedex

FRANCE

Prof. Lu Longhua	Daren Lu	Dan Lubin

Polar Meteorology Research Lab.	Institute of Atmospheric Physics	UCSD/SIO

Acad. of Met. Sci., State Meteorological 	CAS	La Jolla, CA  92093-0221

Admin.	Beijing 100029

Baishiqiaolu #46, West Suburb	CHINA

Beijing

CHINA  100081

Kirk A. Maasch	Doug MacAyeal	Sammantha Magsino, SOAR

Quaternary Institute	Dept. of Geophysical Sciences	Institute for Geophysics

University of Maine	5734 S. Ellis Ave.	University of Texas at Austin

5711 Boardman Hall	Chicago, IL  60637	4412 Spicewood Springs Road

Orono, ME  04469-5711	Bldg. 600

Austin, TX  78759-8500

Dr. Pentti Malkki	Dr. M.J. Manton	Dominique Marbouty

Finnish Instit. of Marine Research	Bureau of Meteorology Research Cen.	European Cent. for Med Range Weather

P.O. Box 33	GPO Box 1289K	Forecasts

SF-00931 Helsinki	Melbourne, Victoria 3001	Shinfield Park, Reading

FINLAND	AUSTRALIA	Berkshire RG2 9AX

UNITED KINGDOM

Cristine Coutinho Marcial	Alberto Marenga	Roberta Marinello

Sec. Da Com. Intermin., Para Recursos do 	Polar Documentation Centre	National Science Foundation

Mar.	P.O. Box 14302	Office of Polar Programs

Min. da Marinha, SECIRM-40 ANDAR	00149 Roma, (Trullo)	Room 755

Cep: 70055-Brasilia-D.F.	ITALY	4201 Wilson Blvd.

BRAZIL	Arlington, VA  22230

Paul Mayewski	Viktor L. Mazo	Thomas McIntyre

Dept. of Geological Sciences	USSR Academy of Sciences	F/PR, NOAA, NMFS

University of Maine	Institute of Geography	1335 East West Highway

5790 Bryand Global Sciences Center	29 Staromonetny	Silver Spring, MD  20910

Orono, ME  04469-5790	109017 Moscow

RUSSIA

Christopher P. McKay	Thomas K. Mefford	Meteorology Department Library

NASA, Solar Systems Explor. Branch	NOAA/CMDL	The Pennsylvania State University

Ames Research Center	325 Broadway	503 Walker Bldg.

Moffett Field, CA  94034	R/CMDL1	University Park, PA  16802

Boulder, CO  80305

Meteorology Office	Tony Meunier	Dr. Kingtse C. Mo

McMurdo Station, Antarctica	USGS MS 515	NOAA/NWS

PSC 469 Box 700	Reston, VA  20192	Nat. Meteorological Center

APO AP  96599-5700	Washington, D.C.  20233

Al Mongeon	Prof. Joseph M. Moran	Dr. Richard E. Moritz

National Weather Service	UW-Green Bay	University of Washington

WMO Representative	Dept. of Earth Science	Polar Science Center

W/OSO 242	Green Bay, WI  54311	1013 N.E. 40th Street

1325 East West Highway	Seattle, WA  98015

Silver Spring, MD  20910

E.M. Morris	Robert Morse	Dr. Ellen Mosley-Thompson

British Antarctic Survey	UW-Madison	Ohio State University

High Cross, Madingley Road	Dept. of Physics	Byrd Polar Research Center

Cambridge CB3 OET	Room 3406 Sterling Hall	1090 Carmack Road

UNITED KINGDOM	475 N. Charter Street	Columbus, OH  43210-1002

Madison, WI  53706

Mr. G.B. Mukherji, Director	Prof. Frank J. Murcray	John Nairn

Dept. of Ocean Development	University of Denver	Bureau of Meteorology

Block 12 C.G.O. Complex	Dept. of Physics	Antarctic Met. Center

Lodi Road	Denver, CO  80208-0202	Project Development Center

New Delhi  110003	P.O. Box 1289K, Melbourne 3001 Victoria

INDIA	AUSTRALIA

M.R. Nayak	National Center for Atmospheric Research	William Neff

National Institute of Oceanography	NCAR Library - Serials	NOAA/ERL/WPL, R/E/WP7

Marine Instrumentation Div.	P.O. Box 3000	325 Broadway

N.I.O. Dona Paula	Boulder, CO  80307	Boulder, CO  80303

Goa-403 004

INDIA

NOAA/MASC Library	NSF Representative	NSF Representative

Acquisitions MC5	McMurdo Station	New Zealand

325 Broadway	PSC 469 Box 700	PSC 467 Box 296

Boulder, CO  80303	APO AP 96599-5700	APO AP 96531-2000

Prof. H.J. Paech	Kathy Pagan	Dr. Julie Palais

Academy of Sciences of the GDR	NASA/AMES	National Science Foundation

Central Earth Physics Instit.	Mail Stop 245-5	Office of Polar Programs

Telegrafenberg A 43	Moffett Field, CA  94035	Room 755

E-1561 Potsdam	4201 Wilson Blvd.

GERMANY	Arlington, VA  22230

Prof. Thomas Parish	Dr. Byong Kwon Park, Director	Carol H. Pease

University of Wyoming	Korea Ocean Res. & Dev. Institute	PMEL NOAA-Building 3

Dept. of Atmospheric Sciences	An San P.O. Box 29	7600 Sand Point Way NE

P.O. Box 3038, Univ. Station	Seoul 425-600	Seattle, WA  98115-0070

Laramie, WY  82071	SOUTH KOREA

Dr. Andrea Pellegrini	Dr. Polly Penhale	Polar Research Center

PAS/Progetto Antarctide	National Science Foundation	Korean Ocean Research and Dev. Inst.

ENEA-Casaccia	Office of Polar Programs	Ansan P.O. Box 29

C.P. 2400	Room 755	Seoul 425-600

00100 Roma A.D.	4201 Wilson Blvd.	SOUTH KOREA

ITALY	Arlington, VA  22230

Michael Pook	Joseph Prejs	Dr. Tony Press, Director

Antarctic CRC	1501 S. Clifton Ave.	Antarctic Division

GPO Box 252-80	Bloomington, IN  47401-6649	Channel Highway

Hobart, Tasmania 7001	Kingston, Tasmania 7050

AUSTRALIA	AUSTRALIA

Linda Preston, Librarian	Rajmund Przybylak	Dominique Raynaud

Federal Bldg.-Library	Nicholas Copernicus University	Laboratoire de Glaciol. et Geophys. de

Nat. Climatic Data Center/NOAA	Dept. of Climatology	l'Environ.

Asheville, NC  28801-2696	ul. Danielwskiego 6	BP 96 38402 St. Martin d'Heres

87-100 Torun	Cedex

POLAND	FRANCE

Margaret N. Rees	Ms. Winifred Reuning	David Reusch

University of Nevada	National Science Foundation	517 Deike Building

Dept. of Geophysics	Office of Polar Programs	Earth and Environmental Systems Institute

Las Vegas, NV  89154	Room 755	Pennsylvania State University

4201 Wilson Blvd.	University Park, PA  16802

Arlington, VA  22230

Dr. Carlos A. Rinaldi, Director	Dr. Ted J. Rosenburg	Dr. Thomas F. Ross

Instituto Antarctico	University of Maryland	NOAA/NCDC

Cerrito 1248	Inst. for Phys. Sci. & Tech.	National Climatic Data Center

1010 Buenos Aires	College Park, MD  20742	Room 121

ARGENTINA	Federal Building

Asheville, NC  28801-2696

Comodoro Enrique Saavedra	V.K. Saxena	Akhil Saxena, Section Officer

Servicio Meteorologico Nacional	North Carolina State University	Government of India

25 de Mayo 658	Dept. of Marine, Earth & Atmos. Science	Dept. of Ocean Development

1002 Buenos Aires	Box 8208	Mahasagar Bhavan, Block-12

REPUBLICA ARGENTINA	Raleigh, NC  27695-8208	CGO Complex, Lodi Road New Delhi

INDIA

Ted Scambos	Greg Scharfen	Guy Schayes

National Snow and Ice Data Center	National Snow and Ice Data Center	UCL-Institut d'Astron. et de Geophys.

University of Colorado	University of Colorado	Chemin du Cyclotron 2

Campus Box 449	Campus Box 449	B-1348 Louvain-La-Neuve

Boulder, CO  80309-0449	Boulder, CO  80309-0449	BELGIUM

Thomas W. Schmidlin	Schwerdtfeger Library	Science & Technology Corp.

Kent State University	Space Science and Engineering Center	101 Research Drive

Dept. of Geography	1225 W. Dayton St., Room-317	Hampton, VA  23666

Kent, OH  44242	Madison, WI 53706	ATTN: C.A. Keen

Science Library	Science Library	Science Library

Amundsen-Scott South Pole Station, 	Crary Laboratory	Palmer Station, Antarctica

Antarctica	McMurdo Station, Antarctica	c/o Supervisor, Lab Operations

c/o Inland Stations Manager	c/o Laboratory Manager	7400 South Tuscon Way

7400 South Tuscon Way	7400 South Tuscon Way Centennial, CO 	Centennial, CO 80112-3938

Centennial, CO 80112-3938	80112-3938

Service Central d'Explication Meteorologia	Dr. Alberto Setzer	Jonathan Shanklin

Division Climatologie	Instituto de Pesquisas Espaciais	British Antarctic Survey

2, Division Rapp	Dept. de Meteorologia e Oceanografia	High Cross, Madingley Road

75340 Paris Cedex 07	Caixa Postal 515	Cambridge CB3 OET

FRANCE	12201-Sao Jose dos Campos-SP	UNITED KINGDOM

BRAZIL

Dr. Hayley Shen	Dr. Christopher A. Shuman	August L. Shumbera

Dept. of Civil and Environmental Engineering	NASA/GFSC	Federal Building

Clarkson University	Code 971	National Climatic Data Center

Potsdam, NY  13699-5710	Greenbelt, MD 20771	Asheville, NC  28801

Kim Silverman	Prof. Ian Simmonds	Prof. Raymond Smith

National Science Foundation	School of Earth Sciences	University of CA-Santa Barbara

Office of Polar Programs	University of Melbourne	Computer Systems Laboratory

Room 755	Parkville, Victoria	Santa Barbara, CA  93106

4201 Wilson Blvd.	AUSTRALIA 3052

Arlington, VA  22230

Dr. Philip M. Solomon	Dr. Susan Solomon	Dr. Roland Souchez

SUNY Stoneybrook	NOAA/Aeronomy Laboratory	Faculte des Sciences

Dept. of Physics	325 Broadway	Dept. des Sci. de la terre et de l'environ.

Stoneybrook, NY  11794	Boulder, CO  80303	CP 160 Ave. F.D. Roosevelt, 50

1050 Brussels

BELGIUM

South Pole Station	Todd Sowers	Sharon Stammerjohn

PSC 468 Box 400	University of Rhode Island	Institute for Computational Earth System

APO AP 96598-5400	URI/GSO S. Ferry Road	Science

Narragansett, RI  02882-1197	UCSD

Sanata Barbara, CA 93106-3060

Station Manager	Towanda Street	Dr. N.A. Streten

Palmer Station, Antarctica	National Ice Center	Bureau of Meteorology

c/o AGUNSA	FOB#4, Room 2301	Box 1289 K, Melbourne 3000

Deposito Franco Antarctico	4251 Suitland Rd.	Victoria

Av. Independencia 772, P.O. Box 60-D 	Washington, DC  20395	AUSTRALIA

Punta Arenas

CHILE

Dr. A.P. Sturman	Al Sutherland	Swedish Polar Research Secretariat, Library

University of Canterbury	National Science Foundation	Box 50005

Dept. of Geography	Office of Polar Programs	S10405 Stockholm

Christchurch 1	Room 755	SWEDEN

NEW ZEALAND	4201 Wilson Blvd.

Arlington, VA  22230

Dr. Charles Swithinbank	Tasmanian/Antarctic Reg. Office	Dr. Kendall Taylor

Home End Fulbourn	Bureau of Meteorology	Desert Research Institute

Cambridge CB1 5BS	Hobart	P.O. Box 60220

UNITED KINGDOM	AUSTRALIA	5625 Fox Ave.

Reno, NV  89506-0220

Dr. Calvin Teague	Martha B. Thayer	Geza T. Thuronyi

Center for Radar Astronomy	NOAA Library	Library of Congress

Dept. of Electrical Engineering	7600 Sand Point Way NE Bldg. 3	Science & Technology Division

Stanford University	Bin C-15700	Washington, D.C.  20540

Stanford, CA  94305	Seattle, WA  98115

Dr. Brian A. Tinsley, MSF022	Dr. Adrian Tuck	John Turner

University of Texas-Dallas	R/E/ALG	British Antarctic Survey

Box 830688	NOAA/Aeronomy Lab	High Cross, Madingley Road

Richardson, TX  75083-0688	325 Broadway	Cambridge CB3 OET

Boulder, CO  80303	UNITED KINGDOM

Dr. Rein Vaikmae	Luca Valenziano	Rodolfo del Valle

Estonian Academy of Science	CNR-TESRE	Instituto Antarctico Argentino

Institute of Geology	via P. Gobetti 101	Cerrito 1248-1010

7 Estonia Puiestee	I-40129	Buenos Aires

200101 Tallinn	Bologna	ARGENTINA

ESTONIA	ITALY

Michiel van den Broeke	Dr. Harry van Loon	Michael Van Woert

IMAU	NCAR	National Science Foundation

P.O. Box 80005	P.O. Box 3000	Office of Polar Programs

3508 TA Utrecht	Boulder, CO  80307	Room 755

THE NETHERLANDS	4201 Wilson Blvd.

Arlington, VA  22230

E.D. Waddington	Tom Wagner	Von Walden

University of Washington	National Science Foundation	Dept. of Geography

Geophysics AK-50	Office of Polar Programs	University of Idaho

Seattle, WA  98195-1650	Room 755	McClure Building, Room 203

4201 Wilson Blvd.	Moscow, ID  83844-3021

Arlington, VA  22230

John E. Walsh	Stephen G. Warren	Dr. Gerd Wendler

University of Illinois	University of Washington	University of Alaska

Dept. of Atmospheric Sciences	Dept. of Atmospheric Sciences	Geophysical Institute

1101 W. Springfield Ave.	AK-40	903 Koyukuk Ave.

Urbana, IL  61801	Seattle, WA  98195	Fairbanks, AK  99701

Prof. G. Wojcik	Dr. Anthony Worby	World Data Center-A

Nicholas Copernicus University	GPO Box 252-80	Glaciology (Snow and Ice)

Inst. Of Geography, Dept. of Climatology	Hobart, Tasmania 7001	CIRES/University of Colorado

ul. Danielewskiego 6	AUSTRALIA	Boulder, CO  80309

87-100 Torun

POLAND

World Data Center-A	World Data Center-A: Meteorology	World Data Center-A: Oceanography

Marine Geology & Geophysics	NOAA, E/CC	NOAA, E/OC53

NOAA, E/GC3	Federal Building	Silver Spring, MD 20910-3282

325 Broadway	Asheville, NC  28801

Boulder, CO  80303

Steven Worley  (2)	Mrs. Xu Pei-Min	Dr.Takashi Yamanouchi

NCAR/SCD	Polar Information Data Center	National Instit. of  Polar Research

P.O. Box 3000	Polar Res. Institute of China	1-9-10 Kaga, Itabashi-ku

Boulder, CO 80307	Jinqiao Road, Pudong 451	Tokyo, 173-8515

Shanghai 200129	JAPAN

CHINA

Hengchun Ye	Zhang Yinsheng	Dr. I.A. Zotikov

California State University, Los Angeles	Cold And Arid Regions	USSR Academy of Sciences

Dept. of Geography and Urban Analysis	Environ. and Engineering Res. Institute	Institute of Geography

Los Angeles, CA  90032-8222	Academia Sinica	29 Staromonetny

Lanzhou 730000	Moscow, 10917

CHINA	RUSSIA

Dr. Mario Zucchelli, Manager	Dr. H.J. Zwally

Antarctic Project, ENEA PAS	NASA/Goddard Space Flight Center

C.R.E. CASACCIA P.O. Box 2400	Code 971

Via Anguillarese, 301	Greenbelt, MD  20771

00100 Roma A.D.

ITALY