### Antarctic Automatic Weather Station Data

for the calendar year 2002

by

- Linda M. Keller

George A. Weidner

Charles R. Stearns

Jonathan E. Thom

Matthew A. Lazzara

Space Science and Engineering Center

University of Wisconsin

1225 W. Dayton St.

Madison, Wisconsin

December, 2008

- Antarctic Automatic Weather Station Data

- For The Calendar Year

2002

by

- Linda M. Keller

- George A. Weidner

- Charles R. Stearns

- Jonathan E. Thom

Matthew A. Lazzara

- Space Science and Engineering Center

- University of Wisconsin

- 1225 W. Dayton St.

- Madison, Wisconsin

- December, 2008

- TABLE OF CONTENTS

- Page

1.  Introduction.........................................................................................................................	  1

2.  Data Transmission..............................................................................................................   	  1

3.  AWS Identification and Location......................................................................................... 	  1

4.  AWS Data Summaries........................................................................................................   	 10

4.1. Monthly Data Summaries..............................................................................................  	 10

D-10..............................................................................................................................	 11

D-47..............................................................................................................................	 11

Dome C II..................................................................................................................... 	 11

Port Martin…………………………………………………………………………………….	 12

Cape Denison…………………………………………………………………………………	 12

Penguin Point............................................................................................................... 	 12

Byrd……………………………………………………………………………………………      13

Mount Siple...................................................................................................................  	 13                    Theresa........................................................................................................................ 	 13

Elizabeth....................................................................................................................... 	 13

Brianna…………………………………………………………………………………………     14

Harry……………………………………………………………………………………………	 14

Erin............................................................................................................................... 	 14

Siple Dome................................................................................................................... 	 15

Swithinbank................................................................................................................... 	 15

Marble Point..................................................................................................................	 15

Ferrell............................................................................................................................ 	 16

Pegasus North...............................................................................................................	 16

Pegasus South.............................................................................................................. 	 16

Minna Bluff.................................................................................................................... 	 17

Linda............................................................................................................................. 	 17

Willie Field....................................................................................................................	 17

Windless Bight..............................................................................................................	 18

Herbie Alley……………………………………………………………………………………	 18

Cape Spencer…………………………………………………………………………………	 18

Cape Bird....................................................................................................................... 	 19

Laurie II……..................................................................................................................	 19

Whitlock……………………………………………………………………………………….      19

Possession Island..........................................................................................................	 20

Manuela.........................................................................................................................	 20

Marilyn...........................................................................................................................	 20

Schwerdtfeger................................................................................................................	 21

Gill.................................................................................................................................	 21

Lettau............................................................................................................................	 21

Elaine............................................................................................................................	 22

Larsen Ice Shelf.............................................................................................................	 22

Butler Island................................................................................................................... 	 22

Uranus Glacier…………………………………………………………………………………    23

Limbert.......................................................................................................................... 	 23

Racer Rock.................................................................................................................... 	 23

Bonaparte Point.............................................................................................................	 24

Sky-Blu............................................................................................................................ 	 24

Kirkwood Island............................................................................................................      24

Dismal Island………………………………………………………………………………….     25

Clean Air (South Pole)................................................................................................... 	 25

Henry............................................................................................................................. 	 25

Nico............................................................................................................................... 	 26

Relay Station.................................................................................................................	 26

Dome Fuji………………………………………………………………………………………    26

Mizuho………………………………………………………………………………………….     27

4.2. Three Hourly Data Summaries....................................................................................... 	 28

5.   AWS Calibration................................................................................................................. 	 28

5.1. Temperature................................................................................................................. 	 28

5.2. Pressure....................................................................................................................... 	 28

5.3. Wind Direction and Speed............................................................................................ 	 28

5.4. Relative Humidity......................................................................................................... 	 29

5.5. Vertical Air Temperature Difference.............................................................................. 	 29

6.   AWS Operations Summary for 2001...................................................................................	 29

6.1. AWS Performance........................................................................................................ 	 29

6.2. AWS Antarctic Field Activities....................................................................................... 	 31

7.   Global Telecommunications System.................................................................................... 	 32

8.   Data Availability................................................................................................................... 	 33

9.   Acknowledgments................................................................................................................ 	 34

- 1.  INTRODUCTION

Automatic weather station (AWS) units are deployed to collect Antarctic surface weather observations in support of specific meteorological research projects as well as operational activities in Antarctica.  The 2002 network consisted of 52 installed AWS units providing observations on the Ross Ice Shelf, east of the Transantarctic Mountains and north of McMurdo to the Adelie Coast, along the Antarctic Peninsula, West Antarctica, East Antarctic, and climatological locations such as the South Pole.  Each unit measures air temperature, wind speed, and wind direction at the top of the unit's tower at a nominal height of three meters and air pressure at the electronics enclosure (Figure 1).  Some AWS units also measure the relative humidity at three meters and vertical air temperature difference between 0.5 and 3 meters.  Measurement heights relative to the actual surface at the site are nominal due to snow accumulation around the AWS unit.

- 2.  DATA TRANSMISSION

The transmitted AWS data are received and stored by the Data Collection System on the NOAA series of polar orbiting satellites.  The data are retransmitted by the satellite for use in the High Resolution Picture Transmission (HRPT) broadcast at McMurdo, Antarctica.  The data are processed into scientific units and are available for local use.  The complete data set is received at the University of Wisconsin-Madison from CLS America (Service ARGOS), Largo, Maryland, for processing and distribution to the users.

- 3.  AWS IDENTIFICATION AND LOCATION

Site location is defined by the latitude and longitude which is determined by various methods: sun shots, angles to geographical features, aircraft data, ice breaker data, the platform location system of CLS America (Service ARGOS), and the Global Positioning System.  AWS elevation is obtained by barometry and Global Positioning System (GPS) and should be correct to within +/- 5 meters.  Site names were introduced for convenience. Table 3.1 lists the site name, ARGOS identification number, latitude, longitude, elevation, start date for the site, and the World Meteorological Organization (WMO) number for the site.  Figures 2, 3, 4, and 5 show the locations of the AWS units in the Antarctic for 2002.

The ARGOS identification number (ID) is used to identify the data sets distributed to the users.  AWS units are sometimes moved from one location to another, and as a result, the ID at a given site may change from year to year. The site name does not change. Table 3.2 lists the site name with the ARGOS ID, the site start date, and the ID start and stop dates.

Figure 1.  Layout of the AWS unit used in the Antarctic.  The installed AWS unit has a 3-meter tower with a horizontal boom supporting the antenna, aerovane for measuring wind speed and direction, air temperature resistance thermometer, upper thermopile for measuring vertical air temperature difference, and the relative humidity sensor.  The electronics enclosure is mounted at the midpoint of the tower.  The gel cell batteries are placed at the tower base.  The solar panel, located near the tower top, faces north. The Acoustic Depth Gauge (ADG) is installed on some of the AWS units to measure snow depth for precipitation studies.

# Table 3.1

AWS site name, geographic location and elevation, site start date, and WMO number for 2002.

| Site Name | ARGOS ID | Lat. (deg ) | Long. (deg) | Elev. (m) | Site Start Date | WMO No. |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | ## Adelie Land |  |  |  |
| D-10 | 8914 | 66.71oS | 139.83oE | 243 | 08 Jan 80 | 89832 |
| D-47 | 8986 | 67.397oS | 138.726oE | 1560 | 24 Jan 83 | 89834 |
| Dome C II | 8989 | 75.121oS | 123.374oE | 3250 | 12 Dec 95 | 89828 |
| Port Martin | 8909 | 66.82oS | 141.39oE | 39 | 19 Jan 90 |  |
| Cape Denison | 8988 | 67.009oS | 142.664oE | 31 | 20 Jan 90 |  |
| Penguin Point | 8910 | 67.617oS | 146.180oE | 30 | 24 Dec 92 | 89847 |
|  |  |  | ## West Antarctica |  |  |  |
| Byrd Station | 8903 | 80.007oS | 119.404oW | 1530 | 05 Feb 80 | 89324 |
| Mount Siple | 8981 | 73.198oS | 127.052oW | 230 | 20 Feb 92 | 89327 |
| Theresa | 21358 | 84.599oS | 115.811oW | 1463 | 29 Nov 94 | 89314 |
| Elizabeth | 21361 | 82.607oS | 137.078oW | 519 | 30 Nov 94 | 89332 |
| Brianna | #8931 | 83.889oS | 134.154oW | 525 | 30 Nov 94 |  |
| Harry | 8900 | 83.003S | 121.393W | 945 | 29 Nov 94 |  |
| Erin | 21363 | 84.904oS | 128.828oW | 990 | 29 Nov 94 |  |
| Siple Dome | 8938 | 81.656oS | 148.773oW | 668 | 21 Jan 97 | 89345 |
| Swithinbank | 21355 | 81.201oS | 126.177oW | 959 | 18 Jan 97 |  |
|  |  |  | ## Ross Island Region |  |  |  |
| Marble Point | 8906 | 77.439oS | 163.754oE | 108 | 05 Feb 80 | 89866 |
| Ferrell | 8929 | 77.910oS | 170.817oE | 45 | 10 Dec 80 | 89872 |
| Pegasus North | 21357 | 77.952oS | 166.500oE | 8 | 23 Jan 90 | 89667 |
| Pegasus South | 8937 | 77.99oS | 166.576oE | 10 | 14 Jan 91 |  |
| Minna Bluff | 8935 | 78.554oS | 166.691oE | 895 | 22 Jan 91 | 89768 |
| Linda | 8919 | 78.464oS | 168.382oE | 47 | 21 Jan 91 | 89769 |
| Willie Field | 21364 | 77.865oS | 167.017oE | 40 | 25 Jan 92 |  |
| Windless Bight | 8927 | 77.728oS | 167.703oE | 61 | 09 Dec 98 |  |
| Herbie Alley | 8697 | 78.10oS | 166.67oE | 30 | 11 Jan 99 |  |
| Cape Spencer | #8695 | 77.97oS | 167.55oE | 30 | 11 Jan 99 |  |
| Cape Bird | 8901 | 77.224oS | 166.440oE | 42 | 28 Jan 99 |  |
| Laurie II | 21360 | 77.549oS | 170.817oE | 30 | 01 Feb 00 |  |
|  |  |  | ## Ocean Islands |  |  |  |
| Whitlock | 8907 | 76.144oS | 168.392oE | 274 | 01 Jan 82 | 89865 |
| Possession Is. | 8984 | 71.891oS | 171.210oE | 30 | 29 Dec 92 | 89879 |
| Manuela | 8905 | 74.946oS | 163.687oE | 78 | 06 Feb 84 | 89864 |
|  |  |  | ## Ross Ice Shelf |  |  |  |
| Marilyn | 8934 | 79.954oS | 165.130oE | 75 | 16 Jan 84 | 89869 |
| Schwerdtfeger | 8913 | 79.904oS | 169.973oE | 60 | 24 Jan 85 | 89868 |
| Gill | 8911 | 79.985oS | 178.611oW | 55 | 24 Jan 85 | 89376 |
| Lettau | 8908 | 82.518oS | 174.452oW | 55 | 29 Jan 86 | 89377 |
| Elaine | 8915 | 83.134oS | 174.169oE | 60 | 28 Jan 86 | 89873 |

| Site Name | ARGOS ID | Lat. (deg) | Long. (deg) | Elev. (m) | Site Start Date | WMO No. |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | ## Antarctic Peninsula |  |  |  |
| Larsen Ice | 8926 | 66.949oS | 60.897oW | 17 | 07 Feb 83 | 89262 |
| Butler Island | 8902 | 72.207oS | 60.160oW | 91 | 01 Mar 86 | 89266 |
| Uranus Glacier | 8920 | 71.43oS | 68.93oW | 780 | 06 Mar 86 | 89264 |
| Limbert | 8925 | 75.422oS | 59.851oW | 40 | 30 Nov 95 | 89257 |
| Racer Rock | 8947 | 64.067oS | 61.613oW | 17 | 15 Oct 89 | 89261 |
| Bonaparte Pt. | 8923 | 64.778oS | 64.067oW | 8 | 05 Jan 92 | 89269 |
| Sky-Blu | 8917 | 74.792oS | 71.488oW | 1556 | 07 Feb 99 | 89272 |
| Kirkwood Island | 8930 | 68.340oS | 69.007oW | 30 | 26 May 01 |  |
| Dismal Island | 8932 | 68.087oS | 68.825oW | 10 | 27 May 01 |  |
|  |  |  | ## High Polar Plateau |  |  |  |
| Clean Air | 8987 | 90.00oS |  | 2835 | 29 Jan 86 | 89208 |
| Henry | 8985 | 89.011oS | 1.025oW | 2755 | 26 Jan 93 | 89108 |
| Nico | 8924 | 89.000oS | 89.669oE | 2935 | 26 Jan 93 | 89799 |
| Relay Station | 8918 | 74.017oS | 43.062oE | 3353 | 01 Feb 95 | 89744 |
| Dome Fuji | 8904 | 77.31oS | 39.70oE | 3810 | 04 Feb 97 | 89734 |
| Mizuho | 21359 | 70.70oS | 44.29oE | 2260 | 07 Oct 00 |  |

* New sites started during 2002

# New ARGOS ID for 2002 at the site

Figure 2.  Antarctic automatic weather station locations during 2002 identified by the site name. Area      around Ross Island is shown in Figure 3. Adelie Coast area is shown in Figure 4. Antarctic Peninsula is shown in Figure 5.

Figure 3.  Location of Antarctic automatic weather stations in the vicinity of Ross Island, Antarctica during 2002.

Figure 4.  Location of Antarctic automatic weather stations along the Adelie Coast during 2002.

- Figure 5.  Location of Antarctic automatic weather stations along the Antarctic Peninsula during 2002.

- Table 3.2

2002 Antarctic AWS site name, ARGOS identification number (ID), site start date, ID start date, and ID stop date if occurring in 2002.

| Site | ARGOS ID | Site Start Date | ID Start Date | ID Stop Date |
| --- | --- | --- | --- | --- |
| D-10 | 8914 | 08 Jan 80 | 28 Jan 98 |  |
| D-47 | 8986 | 24 Nov 83 | 11 Feb 96 |  |
| Dome C II | 8989 | 12 Dec 95 | 12 Dec 95 |  |
| Port Martin | 8909 | 19 Jan 90 | 18 Dec 99 |  |
| Cape Denison | 8988 | 20 Jan 90 | 17 Dec 99 |  |
| Penguin Point | 8910 | 24 Dec 92 | 21 Dec 98 |  |
| Byrd Station | 8903 | 05 Feb 80 | 05 Feb 80 |  |
| Mount Siple | 8981 | 20 Feb 92 | 20 Feb 92 |  |
| Theresa | 21358 | 29 Nov 94 | 29 Nov 94 |  |
| Doug | 8922 | 29 Nov 94 | 20 Jan 97 |  |
| Brianna | 21362 | 30 Nov 94 | 30 Nov 94 | 15 Jan 02 |
|  | 8931 |  | 15 Jan 02 |  |
| Harry | 8900 | 29 Nov 94 | 26 Jan 99 |  |
| Elizabeth | 21361 | 30 Nov 94 | 17 Jan 96 |  |
| Erin | 21363 | 29 Nov 94 | 18 Jan 96 |  |
| Siple Dome | 8938 | 21 Jan 97 | 24 Jan 99 |  |
| Swithinbank | 21356 | 18 Jan 97 | 18 Jan 97 | 13 Jan 02 |
|  | 21355 |  | 13 Jan 02 |  |
| Marble Point | 8906 | 05 Feb 80 | 05 Feb 80 |  |
| Ferrell | 8929 | 10 Dec 80 | 09 Jan 01 |  |
| Pegasus North | 21357 | 23 Jan 90 | 03 Feb 00 |  |
| Pegasus South | 8937 | 14 Jan 91 | 14 Jan 91 |  |
| Minna Bluff | 8935 | 22 Jan 91 | 25 Jan 99 |  |
| Linda | 8919 | 21 Jan 91 | 15 Jan 98 |  |
| Willie Field | 21364 | 25 Jan 92 | 29 Jan 01 |  |
| Windless Bight | 8927 | 09 Dec 98 | 25 Jan 99 |  |
| Herbie Alley | 8697 | 11 Jan 99 | 11 Jan 99 |  |
| Cape Spencer | 8722 | 11 Jan 99 | 11 Jan 99 | 29 Jan 02 |
|  | 8695 |  | 29 Jan 02 |  |
| Cape Bird | 8901 | 28 Jan 99 | 28 Jan 99 |  |
| Laurie II | 21360 | 01 Feb 00 | 16 Jan 01 |  |
| Whitlock | 8907 | 01 Jan 82 | 07 Feb 01 |  |
| Possession Island | 8984 | 29 Dec 92 | 29 Dec 92 |  |
| Marilyn | 8934 | 16 Jan 84 | 30 Jan 01 |  |
| Schwerdtfeger | 8913 | 24 Jan 85 | 22 Jan 93 |  |
| Gill | 8911 | 24 Jan 85 | 25 Jan 91 |  |
| Elaine | 8915 | 28 Jan 86 | 02 Jan 97 |  |
| Lettau | 8908 | 29 Jan 86 | 29 Jan 86 |  |
| Manuela | 8905 | 06 Feb 84 | 15 Feb 87 |  |
| Larsen Ice Shelf | 8926 | 07 Feb 83 | 01 Jan 86 |  |
| Butler Island | 8902 | 01 Mar 86 | 01 Mar 86 |  |
| Uranus Glacier | 8920 | 06 Mar 86 | 06 Mar 86 |  |
| Limbert | 8925 | 30 Nov 95 | 30 Nov 95 |  |
| Racer Rock | 8947 | 15 Oct 89 | 08 Dec 91 |  |
| Bonaparte Point | 8923 | 05 Jan 92 | 23 Dec 96 |  |
| Sky-Blu | 8917 | 07 Feb 99 | 07 Feb 99 |  |
| Kirkwood Is. | 8930 | 26 may 01 | 26 May 01 |  |
| Dismal Is. | 8932 | 27 May 01 | 27 May 01 |  |
| Clean Air | 8987 | 29 Jan 86 | 25 Jan 94 |  |
| Henry | 8985 | 26 Jan 93 | 26 Jan 93 |  |
| Nico | 8924 | 26 Jan 93 | 26 Jan 93 |  |
| Relay Station | 8918 | 01 Feb 95 | 01 Feb 95 |  |
| Dome Fuji | 8904 | 08 Feb 95 | 04 Feb 97 |  |
| Mizuho | 21359 | 07 Oct 00 | 07 Oct 00 |  |

4. AWS DATA SUMMARIES

The data received by the University of Wisconsin, Space Science and Engineering Center, contain all the information received by the ARGOS system including duplicate and erroneous transmissions.  Invalid data are eliminated during a quality check, and the valid data are converted to scientific units producing the complete data set.  Data selected at three hourly intervals, plus or minus forty minutes, produce a three hourly data set for each AWS unit month.  Section 6.1, AWS Performance, provides some explanations for missing and invalid data.

Use of the 2002 Antarctic AWS data for publication should acknowledge the support of NSF-OPP Grant 9726040 or reference this publication.

- 4.1.  Monthly Data Summaries

The monthly summaries consist of the monthly means, from the three hourly data set, and the extremes, from the complete data set.  For monthly values to be included, 25% of the three hourly observations must be available.  Months with 50-75% of data missing occur most often when a station is started or stopped in the middle of the month.  This can cause a bias in the monthly mean, especially during seasons when parameters such as temperature change rapidly.  Annual means are calculated only when twelve months of data are available.  The data are presented in the same order as the sites listed in Table 3.1.  Definitions of the monthly data summary headings are listed below.

Heading                                                Definition

Mean air temperature, oC.	Mean value for the month.

Percent of monthly data missing.        Ratio of the number of missing observations to the number of

possible observations X 100.

Maximum air temperature, oC.	Maximum value for the month.

Minimum air temperature, oC.	Minimum value for the month.

Mean wind speed, m/s.             	Mean value for the month.

Percent of monthly data missing.	Ratio of the number of missing observations to the number of

possible observations X 100.

Resultant wind speed, dir/vv.	Resultant speed and direction for the month.

Constancy.                  	Ratio of the monthly resultant to the monthly mean wind speed.

Maximum wind speed, dir/vv.	Maximum wind speed and direction for the month.

Mean air pressure, mb.	Mean value for the month.

Percent of monthly data missing         Ratio of the number of missing observations to the number of

possible observations X 100.

Maximum air pressure, mb.	Maximum value for the month.

Minimum air pressure, mb.	Minimum value for the month.

Potential temperature, K                     Mean value for the month.

4.2. Three Hourly Data Summaries

After the data are received from Service ARGOS, ten minute interval data are created for each AWS unit. The data are calibrated for the individual station instruments, but no other corrections are made. This data set is created for those users who need fairly current information. These data are available by anonymous FTP (see Section 8).

The 10 minute data set for each AWS unit for the month is scanned to pick out the nearest observation within forty minutes of the UTC hours 00, 03, 06, 09, 12, 15, 18, and 21 to produce the three hourly data set. If valid data are not available within forty minutes of the three hourly time interval, then the entry is left blank to indicate missing data. The means, standard deviations, resultant wind speed and direction, the distribution of temperature, and wind speed with wind direction are determined from the three hourly observations and are presented as a monthly summary at the bottom of each page. A wind direction value of zero indicates a wind speed less than 0.50 m s-1.  North is indicated by a value of 360 degrees. The maximum and minimum values are taken from the complete 10 minute data set, not the three hourly data set. The appropriate monthly data from the three hourly data set are used for the monthly summaries presented in Section 4.1. In the presence of sunlight the air temperatures are questionable if the wind speed is less than 1 m s-1. These summaries are available by anonymous FTP (see Section 8).  If you are unable to access the Internet, we will send the information either on diskettes or paper. Please contact us for further information (the address is at end of Section 8).

5.  AWS CALIBRATION

- 5.1. Temperature

The external and internal temperatures are calibrated using a 1000 ohm 0.05% resistor in place of the platinum resistance thermometers with 1000 ohms resistance at 0oC.  Because the other resistances in the temperature circuit are known only to 1%, the temperature calibration will vary from one electronic unit to another.  The correction factor determined from the calibration resistor is programmed into the read-only-memories for each unit.  After the correction factors have been programmed into the AWS, a calibration box with 0.1% resistors is used to check the temperature calibration.

- 5.2. Pressure

The atmospheric pressure transducer is a Parascientific model 215A Digiquartz pressure gauge.  The transducer frequency changes from a nominal 40 kHz at zero pressure to a nominal 36 kHz at 1000 hPa. The pressure resolution is about 0.05 hPa.

Paulin aneroid barometers calibrated against a mercury barometer of 10 mm bore are used to check the pressure gauge calibration.  Comparisons are made between AWS units, a Parascientific Model 760-16B accurate to +/- 0.1 hPa, and with the mercury barometers at Scott Base, Antarctica.  The calibrations should be within +/- 0.2 hPa.

The reference vacuum on the older pressure transducers can degrade with time with a maximum observed 4 hPa shift to lower pressure after fifteen years. Recalibration of each pressure transducer would be desirable when each unit is serviced.

- 5.3. Wind direction and Speed

The Belfort model 123 aerovane measures wind direction and speed.  The aerovane rotates a potentiometer wiper, and the fraction of full scale of the potentiometer is measured.  The wind direction is checked by positioning the aerovane to the cardinal directions relative to the boom supporting the aerovane.  North or the potentiometer zero is towards the antenna on the boom and has a maximum dead zone of 3o.  During the field installation, the boom is usually aligned along the north-south line as determined from the sun's azimuth, longitude, and Greenwich Mean Time.  In some cases the 180o end of the boom may point in a direction other than south.  At Manuela site, the 180o end of the boom points up the glacier and a correction is added to the data during processing.  At Byrd site the wind is usually out of the north so the boom was rotated 120o and the correction added during the data processing.  The wind speed is determined from the aerovane tachometer voltage output as 0.0472 volt per meter per second.  The aerovane tachometers are spun at 1800 rpm with a load of 1071.5 ohms, and the output should be 9.20 +/-0.05 vdc.

Additional wind sensors were used with AWS units for 2000.  These were the R.M. Young wind monitor model 05103 and the Hydro-Tech WS-3 anemometer with the WP-3 aerovane.  The Hydro-Tech system was used for measuring wind speed in the Adelie Coast area. The Hydro-Tech WS-3 is a disk rotor, 3 in. high and 12 in. overall diameter, with radial cups, and the threshold sensitivity is 3 mph.  The anemometer utilizes a commercial dc tachometer generator.  Output is 0 to +5 vdc (and 0 to 1 ma) over the desired full scale wind speed of 85 m/s.  Accuracy is +/- 2%.

The R.M. Young monitor 05103 also used a 10,000 ohm potentiometer with a 3o dead zone so that the wind direction was recorded identically with the Belfort/Bendix aerovanes.  The wind speed was from the range of 0 to 1.0 volt full scale corresponding to 50 m/s.  Thus the calibration for wind speed was a nominal .195 m/s/bit for the R.M. Young with +/- 1% up to 50 m/s.

- 5.4. Relative Humidity

The Vaisala HMP-35A/45A humidity sensor output voltage varies linearly with relative humidity (U).  The sensor is calibrated by placing it over saturated salt solutions with known relative humidities at room temperature:  sodium chloride (U=75%), and lithium chloride (U=12%) are used.  In addition, a dry inert gas, forced past the sensor, gives a 0% U, and the sensor output can be zeroed.  Then, the gain setting can be set directly using a salt solution with a high relative humidity, such as sodium chloride.  The resolution of the humidity sensor is about 1% and the drift is 2 to 3% per year in the field.  The relative humidity data are not included on the summary pages but are included in the 3 hourly data sets.

- 5.5. Vertical Air Temperature Difference

Two junction thermocouples are used to measure the air temperature difference between 3 m and 0.5 m on the tower.  The output is about 78 microvolts for 1oC temperature difference between the junctions at 0.0oC, dropping to 60 microvolts at -80oC.  Zero output is adjusted to 0.4 volts, so that 0 to 1 volt corresponds to a -6oC to +9oC range of air temperature differences between 3 m and 0.5 m.  The resolution is 0.05oC.  Calibration of the individual systems is done by applying known voltages to the amplifier input. The vertical temperature difference data are not included on the summary pages but are included in the 3 hourly data sets.

- 6.  AWS OPERATIONS SUMMARY FOR 2002

- 6.1.  AWS Performance

Forty-seven AWS units were installed at the start of 2002 and 51 were installed by the end of 2002.  Based on the installation months the AWS units delivered 85% of the temperature data, 82% of the pressure data and 72% of the wind data during 2002. Complete data sets were received from 12 AWS units and 38 AWS units operated for the installed period. Eleven AWS units were not received for one month or more during the year or stopped during the year.  Some of the stations were not received during the winter months due to low battery voltage.

The wind system has the poorest performance. If the wind speed is zero or the wind direction is constant for extended periods (days to months) then the data is considered invalid. The reason for this behavior is not known but is believed to be due to the build up of frost on the wind system. This usually occurs in the winter season and at several AWS sites. The wind speed is most frequently zero when the wind direction is constant.

Site	Performance

D-10 			The station transmitted erratically from July through the beginning of October. The relative humidity sensor and delta-T sensor were not functioning.

D-47			Temperature sensor not functioning. Station stopped transmitting 4 January and began again 27 February. Station transmitted erratically from April through early August with another break the first half of September. No relative humidity or delta-T sensors.

Dome C II		OK.

Port Martin		The aerovane was not functioning. No delta-T or extra high wind speed sensor. Pressure corrected for high wind speed conditions.

Cape Denison		Station stopped transmitting 29 April and resumed 27 May. Several transmission gaps of a week or more in April, May, and September. Pressure corrected for high wind speed conditions.

Penguin Point		The station stopped transmitting from 15 May to 21 June. Pressure corrected for high wind speed conditions.

Byrd			Station transmitted from 3 to 8 October only.

Mount Siple		Site has a "dog house" AWS without wind speed and direction. Pressure sensor 					not functioning correctly January and February.

Theresa			Delta-T sensor buried.

Doug			Aerovane not functioning in July. Relative humidity sensor not functioning. Station transmitted erratically in September and stopped transmitting on 8 October.

Elizabeth			Aerovane "frozen" the last part of September.

Brianna			The relative humidity sensor functioned erratically after June. The station stopped transmitting 3 November.

Harry			The delta-T sensor is not functioning. The aerovane was "frozen" occasionally from June through November.

Erin			The aerovane was "frozen" occasionally from May through July.

Siple Dome		Aerovane “frozen” occasionally from April through November. Delta-T sensor not installed.

Swithinbank		Station stopped transmitting 8 October. Delta-T sensor not functioning.

Marble Point		OK.

Ferrell			New station installed 10 January.

Pegasus North		Station stopped transmitting on 25 January. Delta-T not functioning. Station resumed transmitting 4 November.

Pegasus South		Station stopped transmitting on 6 May and resumed on 24 June. Relative humidity sensor not functioning. Aerovane “frozen” from June through October. Delta-T erratic all year.

Minna Bluff		Station started transmitting 10 July. Delta-T sensor not functioning. Aerovane occasionally “frozen” July through November. Station stopped transmitting 13 December.

Linda			Aerovane occasionally "frozen" April through October.

Willie Field		Station replaced 29 January.

Windless Bight		Station stopped transmitting 12 December due to low battery voltage.

Herbie Alley		Pressure functioned erratically August,  October, and November.

Cape Spencer		Station stopped transmitting 25 December.

Cape Bird			OK.

Laurie II			Station replaced 17 January. Delta-T sensor not functioning. Aerovane occasionally "frozen" in September.

Whitlock			Station replaced 7 February. Delta-T sensor not functioning. Aerovane “frozen”  most of April and May and parts of June and July.

Possession Island		Site has a "dog house" AWS without wind speed and direction.

Manuela			Aerovane broken.

Marilyn			Station replaced 30 January. Aerovane occasionally "frozen" May through November.

Schwerdtfeger		Relative humidity sensor not functioning from mid February through November Aerovane “frozen” occasionally in July.

Gill			Relative humidity sensor not functioning. Aerovane “frozen” occasionally in October.

Lettau			Transmitting with many gaps in January, November, and December. Aerovane “frozen” occasionally from March through December.

Elaine			OK.

Larsen Ice Shelf		Aerovane “frozen” occasionally in July.

Butler Island		Aerovane not functioning beginning of January and “frozen” occasionally from May through November. Pressure continues to need correction due to the failure of the precision time-based correction to the system clock.

Uranus Glacier		Large gaps in transmission in March, November, and some of December.

Limbert			Delta-T sensor not functioning. Aerovane not functioning. Station stopped transmitting in July and resumed in September due to low battery voltage.

Racer Rock		Station transmitted erratically from the end of May to October due to low battery voltage. Station stopped transmitting 30 November.

Bonaparte Point		Relative humidity and delta-T sensor not functioning. Aerovane not functioning January through May, July, and October through December. Pressure is too low and has been removed.

Sky Blu			Aerovane “frozen” occasionally in September.

Kirkwood Island		New station installed 21 May. Aerovane not functioning properly in May.

Dismal Island		New station installed 22 May. Aerovane not functioning properly in May.

Clean Air			Occasional problems with relative humidity sensor. Pressure functioning erratically February through November.

Henry			Aerovane occasionally "frozen" June through September. Station stopped transmitting 20 September and resumed transmitting 30 October as the battery recharged in the austral spring.

Nico			Aerovane occasionally "frozen" in August and September.

Relay Station		OK.

Dome Fuji		Repaired station installed on 22 December.

Mizuho			No relative humidity or delta-T sensors. Occasional gaps in transmission during winter months.

- 6.2.  AWS Antarctic Field Activities

# McMurdo area

On 1 February, Ferrell site was visited by Jason Vandervest. A completely new station was installed (AWS 21355) with help from the Twin Otter crew. Later on 1 February, a new station site was establish near Cape Crozier (AWS 21364). This site will be called Laurie II.

On 3 February, Pegasus North and Pegasus South (AWS 8937) sites were visited by Dr. John Cassano and Jason Vandervest. AWS 8928 at Pegasus North was removed and a new station (AWS 21357) was installed. At Pegasus South the aerovane was replaced.

# West Antarctica

# Dr. John Cassano, Dr. Chris Shuman, and Jason Vandervest flew to Siple Dome on 15 January to conduct AWS field work in West Antarctica. GPS coordinates were recorded for Siple Dome (8938) site, and it was noted that the boom was not aligned with north. On 18 January, the team went to Brianna (21361) site. Two battery boxed and 1 battery cable were replaced, and GPS coordinates were recorded. The team moved on to Elizabeth (21361) site where two new battery boxes and cables were installed, and GPS coordinates were recorded. On 19 January, the team went to JC (21357) site. The site was anchored loosely, and the antenna was broken. They were unable to replace the antenna, so the entire site was removed. On to Erin (21363) site, the team recorded GPS coordinates. They installed 2 new battery boxes and cables, raised the lower delta T boom because the probe was covered with snow, and replaced the Bendix/Belfort aerovane. Finally, the team installed a new AWS (8936) at Noel/ITASE site. GPS coordinates were recorded.

# Polar Plateau

Dr. John Cassano reinstalled the Clean Air AWS (8987) at the new site at South Pole station on 26 January.

Mizuho AWS unit (21359) was installed at Mizuho Station on 7 October by members of JARE.

# Adelie Coast

Rob Flint met the USGS Polar Sea in Hobart for the trip along the Adelie Coast. On 18 December Rob Flint and Blake Moore went to Port Martin (8909). The solar panel had been smashed by an impact. Some of the connectors and the junction box were so corroded by salt that it was impossible to do any work. A return visit with better tools and equipment was made on 20 December. New batteries, solar panel and junction box were installed. Also on 18 December, Gerd Wendler and Drew Egressey went to Sutton (8939). The station was removed and returned to the ship.

On 19 December, Rob Flint and Rachel Smith went to Penguin Point. They replaced the batteries and tightened the guys. A visual inspection showed the station to be in good condition with no evidence of salt corrosion.  On 20 December, Rob Flint and LCDR Jackson went to Cape Denison. The batteries were replaced, and no evidence of salt deposition was found on the station. Installation of Cape Webb was scrapped due to transportation problems.

# Antarctic Peninsula

The British Antarctic Survey serviced several of the units on the Antarctic Peninsula. Limbert (8925) site had stopped transmitting. The batteries were charged, so the unit was moved to Rothera on 14 January for further testing. A fault was found on the CPU board, and the unit was replaced on 10 February. A new tower section was added, and the boom was replaced. Uranus Glacier (8920) was visited on 7 January.  The tower was raised, three new guys and deadmen were placed, and two new batteries and the junction box were replaced. The unit was removed to Rothera on 19 January because the transmissions were garbled. The 50 MHz oscillator was retuned, and the unit replaced on 24 January.

AWS unit 8917 (Sky-Blu) was revisited on 8 and 9 February. The tower was raised by two sections. Larsen Ice (8926) site was also visited on 9 February. Two new batteries were added and the guys were tightened. On 11 February, Butler Island (8902) site was visited. The tower was raised and new battery boxes added. The wind propeller was loose, and there is evidence of salt corrosion at the site.

7.  GLOBAL TELECOMMUNICATIONS SYSTEM

The data from 31 Antarctic AWS units were entered into the Global Telecommunications System (GTS) during 2000. The data are collected by Service ARGOS. As soon as the data are received, Service ARGOS processes them and sends them on to the National Weather Service which distributes the data to the GTS.The data headers are:

- SMAA14 KARS YYGGgg

SIAA14 KARS YYGGgg

SNAA14 KARS YYGGgg

where S indicates surface, M is main observations (at 00, 06, 12, and 18 UT), I is intermediate observations (at 03, 09, 15, and 21 UT), and N is any other time. AA14 is for Antarctica, and KARS stands for the Largo receiving center (backup is LFPW for the center in Toulouse, France). YY indicates the day in the month, GG is the hour, and gg is the minutes. Table 3.1 contains the WMO identification number used by the GTS grouped according to their purpose and proximity where possible.

The University of Wisconsin-Madison is responsible for obtaining WMO numbers for AWS sites and for providing Service ARGOS with calibration information for processing the data. The main reason for getting the AWS data into the GTS is to make sure that the data are available in near real time for all organizations operating in Antarctica.

- 8.  DATA AVAILABILITY

The data from our Automatic Weather Stations are available by anonymous FTP.  The IP address and domain name are 128.104.109.33 (ice.ssec.wisc.edu).  The login is "anonymous" (do not use the quotation marks), and the password is your email address.  Once you have logged in, change to the pub/aws subdirectory.  A listing of our station locations, names, and ARGOS ID numbers is located in the file "biglist" in this subdirectory.  It is meant to serve as a guide to our stations as their ID numbers sometimes change. A complete guide for navigating the site may be found in the file “readme.faq”.

Our three-hourly interval data for Antarctica in ASCII text format are contained in the year subdirectories of pub/aws/antrdr.  The data have been corrected, i.e. an effort has been made to remove the bad data points.  These data take longer to process, so the data for recent months are not available in this format.  Within each of the year subdirectories of pub/aws/antrdr, there are text files named "3hrlist??" (where ?? indicates the last two digits of the year). These files list what station's data are contained in which files. The files “readme.updates?” in pub/aws/antrdr contain information on updates and/or corrections to the data, and the file “readme.3format” contains file name construction information and format of the three-hourly data. The file “readme.mailinglist” contains information on joining a mailing list which distributes information on data updates and changes. To subscribe, send email to aws@ssec.wisc.edu requesting to be added to our mailing list.

The directory pub/aws/summary/monthly contains ASCII printable text files of the paper data summary sheets.  The format of the files can be found in the file "readme.sum" while updates and corrections to the data are located in "readme.sumupdates".  The data are located in year subdirectories of pub/summary/monthly.

For those users who need more current information, we have created 10 minute interval data in ASCII text format for each station.  These data are located in year subdirectories of pub/aws/10min/rdr.  The data have been calibrated for the individual station instruments, but no other corrections have been made.  The data are generally available up to and including the last full month of this year.  The year subdirectories also contain a text file named "namelist??" (where ?? indicates the last two digits of the year in question).  These files list what station's data are contained in which files.

Several important readme files are located in pub/aws/10min/rdr. The file "readme.format" contains information on filename construction of the data, as well as well as file content and is a must for those unfamiliar with the data. The files "readme.updates?" contain important information on changes/additions to the data.

Our site is available 24 hours a day, 7 days a week.  If you have questions or problems, send email to aws@ssec.wisc.edu.  We can also be reached by phone at (608) 265-2209 or (608) 262-0436 or fax at (608) 262-5947.

By mail, please contact:

Linda M. Keller

University of Wisconsin

Department of Atmospheric and Oceanic Sciences

1225 W. Dayton St.

Madison, WI  53706

USA

- 9.  ACKNOWLEDGMENTS

This work is supported by the National Science Foundation, Office of Polar Programs, Grant 9726040 under the management of Dr. Bernhard Lettau. Assistance from the following countries is gratefully acknowledged. Expeditions Polaires Francaises installs and maintains the AWS units from the Adelie Coast to Dome C II.  The British Antarctic Survey maintains the AWS units on the east side of the Antarctic Peninsula and south of Adelaide Island. The Japanese Antarctic Research Expedition (JARE) installs and maintains the AWS units at Relay Station, Mizuho, and Dome Fuji.

Mary R. Albert	Prof. Richard Alley	Dr. I. Allison

U.S.A. CRREL	The Pennsylvania State University	Antarctic CRC

72 Lyme Road	Earth Systems Science Center	University of Tasmania

Hanover, NH  03755-1290	517 Deike Building	Hobart, Tasmania  7001

University Park, PA  16802	AUSTRALIA

Dr. Sridhar Anandakrishnan	Dr. Jean-Claude Andre	Stefania Argentini

The Pennsylvania State University	Centre National de Recherches	Consiglio Nazionale Delle Ricerche

Earth Systems Science Center	Meteorologiques	IFA/CNR Via G. Galilei, c.p. 27

540 Deike Building	42 Avenue G. Coriolis	00044 Frascati (Roma)

University Park, PA  16802	F-31057 Toulouse Cedex	ITALY

FRANCE

Dr. Gail Ashley	Liana Bacigalupi	Dr. Karen S. Baker

Rutgers University	Library	Scripps Institute of Oceanography

Dept. of Geological Sciences	ENEA Progetto Antartide	UCSD - A - 018

Piscataway, NJ  08854	Via Anguillarese, 301	La Jolla, CA  92093

00060 S. Maria di Galeria (Roma)

ITALY

Prof. Roger Bales	Dr. L.A. Barrie	Ian J. Barton

University of Arizona	Atmospheric Environment Service	CSIRO Marine Laboratories

Dept. of Hydrology & Water Resources	4905 Dufferin Street	GPO Box 1538

Tuscon, AZ  85721	North York, Ontario	Hobart, Tasmania 7001

M3H 5T4	AUSTRALIA

CANADA

Rabindra Basnyat	Rob Bauer	Ms. Judy Bausch

General Post Office	NSIDC/CIRES	Yerkes Observatory

P.O. Box #6644	University of Colorado	P.O. Box 258

Kathmandu	Boulder, CO  80309-0449	Williams Bay, WI  53191

NEPAL

Dr. John Bengston	Prof. Charles R. Bentley	Dr. Robert Bindshadler

NOAA/NMFS	University of Wisconsin	NASA

National Marine Mammal Lab.	Geophys. & Polar Research Cen.	Goddard Space Flight Center

7600 Sand Pt. Way N.E.	1215 W. Dayton St.	Code 971

Seattle, WA  98115-0070	Madison, WI  53706	Greenbelt, MD  20771

Prof. Krzystztof Birkenmajer, Chrm.	Dr. George Blaisdell	Mr. Greg Bodeker

Com. On Polar Res., Polish Academy of 	National Science Foundation	University of Natal

Sciences	Office of Polar Programs	Space Physics Research Instit.

Dept. of Dynamic Geology	Room 755	King George V. Ave.

Instit. of Geological Sciences	4201 Wilson Blvd.	Durban 4001

ul. Senacka 1-3	Arlington, VA  22230	SOUTH AFRICA

31-002 Krakow

POLAND

Dr. John Bolzan	Dr. Scott Borg	Irina I. Borzenkova

Ohio State University	National Science Foundation	State Hydrological Institute

Byrd Polar Research Center	Office of Polar Programs	23 Second Line

1090 Carmack Road	Room 755	199053, St. Petersburg

Columbus, OH  43210-1002	4201 Wilson Blvd.	RUSSIA

Arlington, VA  22230

John W. Briggs, Engineer	Dr. David Bromwich	Neva Brown

Yerkes Observatory	Ohio State University	30 Lantern Lane

373 West Geneva Street	Byrd Polar Research Center	Wrentham, MA 02093

Williams Bay, WI  53191-0258	1090 Carmack Road

Columbus, OH  43210-1002

Giancarlo Caimi	Michael Carmody	Prof. Azzio Caneva

Via S. Benelli 8	Raytheon Polar Services	Universita di Genova

20151	7400 South Tuscon Way	Stazione Geofisica

Milano	Centennial, CO 80112-3938	12080 Roburent

ITALY	Cuneo

ITALY

Captain, R/V Lawrence G. Gould	Captain, R/V Nathaniel B. Palmer	Dr. A.M. Carleton

c/o AGUNSA	c/o AGUNSA	Dept. of Geography

Deposito Franco Antarctico	Deposito Franco Antarctico	The Pennsylvania State University

Av. Independencia 772	Av. Independencia 772	0314 Walker Building

P.O. Box 60-D	P.O. Box 60-D	University Park, PA  16802

Punta Arenas	Punta Arenas

CHILE	CHILE

Prof. John Carroll	I.F.S. Cavalacanti	Art Cayette

University of California	Depto Meteorologia, INPE	SPAWAR System Center

Dept. LAWR	Caixu Postal 515	P.O. Box 190022

Davis, CA  95616	12 200 Sao Jose dos Campos	North Charleston, NC  29419-9022

Sao Paulo

BRAZIL

Roberto Cervellati	Prof. Chen Liqi, Director	Dr. TaeJin Choi

ENEA Pas Progetto Antarctide	Chinese Antarctic Administration	Korea Polar Research Institute, KORDI

S.S. Anguillarese 301	No. 1 Fuxingmenwai Street	Songdo Techno Park, 7-50,

Roma	Beijing	Songdo-dong, Yeonsu-gu,

ITALY	CHINA	Incheon 406-840

KOREA

Pecheng Chu	Dr. Josefino C. Comiso	The Committee on Antarctic Research

Naval Postgrad. School, Code OC/CU	NASA/Goddard Space Flight Center	ul. Vavilova, 44

Dept. of Oceanography	Code 971	Building 2

Monterey, CA  93943	Greenbelt, MD  20771	Moscow 117333

RUSSIA

Frederick E. Crory	Dick Crouthamel	CRREL-TL

US Army CRREL	National Weather Service	Dept. of the Army Corps of Engineers

72 Lyme Road	Mail Code W/OM31	Hanover, NH  03755-1290

Hanover, NH  03755-1290	1325 E-W Highway

Silver Spring, MD  20910

W. Darnell	Dr. Thomas DeFelice	Rear Ar. Fernando M. Diegues, Sec.

NASA, Langley Research Center	University of Wisconsin	Comm. Interministerial para os Recursos do

Mail Stop 185	Dept. of Geoscience, Lapham Hall	Mar (CIRM)

Hampton, VA  23665-5225	P.O. Box 413	Ministerio da Marinha

Milwaukee, WI  53201	40 ANDAR

70055 DF-Brasilia

BRAZIL

Debra Diemand	Director	Dr. David J. Drewry

CRREL-TL	Cold And Arid Regions	Vice-Chancellor

Dept. of the Army Corps of Engineers	Environ. and Engineering Res. Institute	University of Hull

Hanover, NH  03755-1290	Academia Sinica	Cottingham Road

Lanzhou 730000	Hull, Yorksire HU6 7RX

CHINA	UNITED KINGDOM

Mr. Michel Engler, Director	Dr. Hiroyuki Enomoto	Dr. Karl Erb

Expeditions Polaires Francaises	Kitami Institute of Technology	National Science Foundation

47 Ave. du Marechal Fayolle	Koencho 165	Office of Polar Programs

75116 Paris	Kitami, Hokkaido	Room 755

FRANCE	090-8507  JAPAN	4201 Wilson Blvd.

Arlington, VA  22230

Jane Ferrigno	Michel Fily	Margaret Finger, Librarian

USGS National Center	Lab. de Glaciologie et Geophys. de 	University of Melbourne

Mail Stop 927	L'Environnement	Dept. of Meteorology

Reston, VA  22092	BP 96	Parkville, Victoria  3052

38402 Saint-Martin-d'Heres	AUSTRALIA

FRANCE

Dr. Joan Fitzpatrick	Dr. R.J. Flint	Dr. Benson Fogle

Federal Center	185 Bear Gulch Road	7 Park Overlook Court

U.S. Geological Survey-Denver	Woodside, CA  94062	Bethesda, MD  20817

P.O. Box 25046

Denver, CO  80225

Prof. Theodore D. Foster	Andrew G. Fountain	E. Imre Friedmann

Scripps Institute of Oceanography	Dept. of Geology	Polar Desert Research Center

Mail Code 0240	Portland State University	Dept. of Biological Science (B-142)

University of California, San Diego	P.O. Box 751	Florida State University

LaJolla, CA  92093-0240	Portland, OR 97207-0751	Tallahassee, FL  32306-2043

Prof. H. Fukunishi	Gen. Ricardo Galarza	Christophe Genthon

Tohoku University	Uruguayan Antarctic Institute	Lab. de Glaciol. et Geophys. de l'Env.

Upper Atmos. & Space Research	Buenos Aires 350	54 Rue Moliere, DU BP 96

Laboratory	Montevideo	F-38402 Saint Martin d'Heres

Sendai 980-0845	URUGUAY	Cedex

JAPAN	FRANCE

Abbate Giulia	Hector R. Godoy	Goldthwait Polar Library

ENEA Casaccia	Servicio Meteorologico National	Byrd Polar Research Center

Italian Antarctic Project	Depto Antarctico	Ohio State University

Via Anguillarese 301	25 de Mayo 658	1090 Carmack Road

00100 Roma	Capital Federal	Columbus, OH  43210-1002

ITALY	REPUBLICA ARGENTINA

Robert Grumbine	Anselm Haanen	Ritva Hanninen, Librarian

5200 Auth Rd., Room 204	IEIS Prog. Consultant, c/-DSIR Antarctic	Finnish Meteorological Institute

W/NMC21	Orchard Road	Box 503

Camp Springs, MD 20746	Christchurch International Airport	SF-00101 Helsinki 10

P.O. Box 14-091, Christchurch	FINLAND

NEW ZEALAND

Inger Hanssen-Bauer	Ross Harp, Jr.	Ralph Harvey

Norwegian Meteorological Institute	P.O. Box 1717	Dept. Geological Sciences

P.O. Box 43 - Blindem	Boulder, CO  80306	Case Western Reserve University

0313 Oslo 3	112 A.W. Smith

NORWAY	10900 Euclid Ave.

Cleveland, OH  44106-7216

G. Hernandez	Dr. Roger Hewitt	Dr. Austin Hogan

University of Washington	Southwest Fisheries Center	P.O. Box 21

Geophysics Program	Antarctic Ecosys. Research Gr.	Piermont, NH  03779-0021

AK-50	P.O. Box 271

Seattle, WA  98195	La Jolla, CA  92038

Hugh Hutchinson	T.H. Jacka	Dr. Stanley Jacobs

Bureau of Meteorology	Antarctic CRC	Columbia University

20 Ellerslie Rd.	Box 252C	Lamont Observatory

Hobart, TAS  7000	Hobart, Tasmania	Palisades, NY  10964

AUSTRALIA	AUSTRALIA 7001

Martin Jeffries	Dr. Ken Jezek	Prof. Jia Pengqun

National Science Foundation	Ohio State University	Polar Met. Lab., Acad. of Met. Sci.

Office of Polar Programs	Byrd Polar Research Center	State Meteorological Admin.

Room 755	1090 Carmack Road	Baishiqiaolu No. 46, West Suburb

4201 Wilson Blvd.	Columbus, OH  43210-1002	Beijing,  100081

Arlington, VA  22230	CHINA

P.D. Jones	Dr. Takao Kameda	Prof. Anders Karlqvist, Director

University of East Anglia	Snow and Ice Research Laboratory	Royal Swedish Academy of Science

Climate Research Unit	Dept. of Civil & Environmental Engineering	Polar Research Secretariat

Norwich NR4 7TJ	Kitami Institute of Technology	Box 50005

UNITED KINGDOM	165, Koencho, Kitami,	10405 Stockholm

Hokkaido 090-8507	SWEDEN

JAPAN

Stephen J. Katz	Chris Kidd	Dr. Tokio Kikuchi

Colorado School of Mines	School of Geography	Kochi University

Arthur Lakes Library/Serials	University of Birmingham	Dept. of Physics, Faculty of Sci.

Golden, CO  80401	Edgbaston, Birmingham	2-5-1, Akebono-cho

B15 2TT	Kochi 780-8072

UNITED KINGDOM	JAPAN

Dr. John C. King	Akio Kitoh	Dr. Yuji Kodama

British Antarctic Survey	Meteorological Research Institute	Hokkaido University

High Cross, Madingley Road	Climate Research Division	Institute of Low Temp. Science

Cambridge CB3 OET	Tsukuba, Ibaraki 305-0052	Sapporo 060-0819

UNITED KINGDOM	JAPAN	JAPAN

Dr. K.Ya Kondratyev	Dr. Els Kooij-Connally	Prof. Dr. Christopher Kottmeier

The Russian Acad. Inst. for Lake Res.	Library ECMWF	Universitat Karlsruhe

Sevastyanov Str., 9	Shinfield Park	Institut fur Meteorologie und

St. Petersburg,  196199	Reading, Berkshire RG2 9AX		          Klimaforschung

RUSSIA	UNITED KINGDOM	Kaiserstr. 12 (Physikhochhaus)

76128 Karlsruhe

GERMANY

Dr. Steve Kottmeier	Prof. Dr. H. Kraus	Arlin Krueger

Raytheon Polar Services	Met. Institut der Universitat Bonn	NASA/Goddard Space Flight Center

7400 South Tuscon Way	D-5300 Bonn 1	Code 616

Centennial, CO  80112-3938	FRG Auf Dem Hugel 20	Greenbelt, MD  20771

GERMANY

Dr. B.L. Krutskikh	Prof. M. Kuhn	Dr. Ron Kwok

Arctic & Antarc. Research Institute	Universitat Innsbruck	Senior Research Scientist

St. Petersburg,  191104	Institut fur Met. & Geophysik	Jet Propulsion Laboratory

RUSSIA	Innrain 52	California Institute of Technology

A-6020 Innsbruck	4800 Oak Grove Dr.

AUSTRIA	Pasadena, CA 91109

Dr. Victor Lagun	Claude M. Laird	Jouko Launiainen

Air-Sea Interaction Dept.	University of Kansas	Finnish Instit. of Marine Research

The Arctic & Antarc. Scientific Res. Instit.	2291 Irving Hill Drive	P.O. Box 33

38 Bering Street	Campus West	00931 Helsinki

199 397 St. Petersburg	Lawrence, KS  66045	FINLAND

RUSSIAN FEDERATION

Anthony Lazzara	Brig. Gen. Jorge E. Leal	Bang Yong Lee

P.O. Box 72	Dir. Nacional del Antarctico	Korea Ocean Research & Dev. Instit.

Morgan, VT  05853	Cerrito 1248	Polar Research Center

1010 Buenos Aires	Ansan P.O. Box 29

ARGENTINA	Seoul 425-600

SOUTH KOREA

Denyse Lemaire-Ronveaux	Dr. Marc Lessard	Librarian

Dept. des Sci. de la terre et de l'environ.	Thayer School of Engineering	Australian Antarctic Division

Faculte des Sciences	Dartmouth College	Channel Highway

CP 160 Ave F.D. Roosevelt, 50	8000 Cummings	Kingston, Tasmania  7050

1050 Brussels	Hanover, NH  03755-8000	AUSTRALIA

BELGIUM

Librarian	Library	Library

British Antarctic Survey	Alfred Wegener Inst. fur Polar und 	Antarctica New Zealand

High Cross, Madingley Road	Meeresforschung	Private Bag 4745

Cambridge CB3 OET	Forschungsstelle Potsdam	Christchurch

UNITED KINGDOM	Telegraphenberg A 43	NEW ZEALAND

14473 Potsdam

GERMANY

Library	Library	Library

Climatic Research Unit	Finnish Inst. of Marine Research	Instituto Antarctico Chileno

University of East Anglia	Box 33	Luis Thayer Ojeda 814

Norwich NR4 7TJ	SF 00931	Casilla 16521 Correo 9

UNITED KINGDOM	Helsinki	Santiago

FINLAND	CHILE

Keith B. Mayer Library	Library	Library

Geophysical Institute/IARC	National Inst. of Polar Research	Tasmania/Antarctica Reg. Office

903 Koyukuk	9-10, Kaga 1-chome	Bureau of Meteorology

Fairbanks, AK  99775	Itabashi-ku	GPO Box 727G

Tokyo 173-8515	Hobart, Tasmania 7001

JAPAN	AUSTRALIA

Dr. David Limbert	Nancy Liston, Librarian	Michael P. Lizotte

2 Topcliffe Way	U.S. Army CRREL	800 Algoma Blvd.

Cambridge, CB1 4SH	Attn: Library	Oshkosh, WI  54901

UNITED KINGDOM	72 Lyme Road

Hanover, NH  03755-1290

Dr. C. Lorius	Dr. Douglas H. Lowenthal	Daren Lu

C.R.N.S., Lab. de Glaciol. et Geophys. de 	Desert Research Institute	Institute of Atmospheric Physics

L'Environ.	2215 Raggio Parkway	CAS

rue Moliere, BP 96	Reno, NV  89512-1095	Beijing 100029

38 402 Saint Martin D'Heres	CHINA

Cedex

FRANCE

Dan Lubin	Prof. Lu Longhua	Kirk A. Maasch

National Science Foundation	Polar Meteorology Research Lab.	Quaternary Institute

Office of Polar Programs	Acad. of Met. Sci., State Meteorological 	University of Maine

Room 755	Admin.	5711 Boardman Hall

4201 Wilson Blvd.	Baishiqiaolu #46, West Suburb	Orono, ME  04469-5711

Arlington, VA  22230	Beijing

CHINA  100081

Doug MacAyeal	Sammantha Magsino, SOAR	Dr. Pentti Malkki

Dept. of Geophysical Sciences	Institute for Geophysics	Finnish Instit. of Marine Research

5734 S. Ellis Ave.	University of Texas at Austin	P.O. Box 33

Chicago, IL  60637	4412 Spicewood Springs Road	SF-00931 Helsinki

Bldg. 600	FINLAND

Austin, TX  78759-8500

Dr. M.J. Manton	Dominique Marbouty	Cristine Coutinho Marcial

Bureau of Meteorology Research Cen.	European Cent. for Med Range Weather 	Sec. Da Com. Intermin., Para

GPO Box 1289K	Forecasts	Recursos do Mar.

Melbourne, Victoria 3001	Shinfield Park, Reading	Min. da Marinha, SECIRM-40 ANDAR

AUSTRALIA	Berkshire RG2 9AX	Cep: 70055-Brasilia-D.F.

UNITED KINGDOM	BRAZIL

Alberto Marenga	Roberta Marinello	Paul Mayewski

Polar Documentation Centre	National Science Foundation	Dept. of Geological Sciences

P.O. Box 14302	Office of Polar Programs	University of Maine

00149 Roma, (Trullo)	Room 755	5790 Bryand Global Sciences Center

ITALY	4201 Wilson Blvd.	Orono, ME  04469-5790

Arlington, VA  22230

Viktor L. Mazo	Joe McConnell	Christopher P. McKay

Russian Academy of Sciences	University of Arizona	NASA, Solar Systems Explor. Branch

Institute of Geography	Dept. of Hydrology and Water Resources	Ames Research Center

29 Staromonetny	Tucson, AZ  85721	Moffett Field, CA  94034

119017 Moscow

RUSSIA

Thomas K. Mefford	Meteorology Department Library	Meteorology Office

NOAA/CMDL	The Pennsylvania State University	McMurdo Station, Antarctica

325 Broadway	503 Walker Bldg.	PSC 469 Box 700

R/CMDL1	University Park, PA  16802	APO AP  96599-5700

Boulder, CO  80305

Tony Meunier	Dr. Peter Milne	Al Mongeon

USGS MS 515	National Science Foundation	National Weather Service

Reston, VA  20192	Office of Polar Programs	WMO Representative

Room 755	W/OSO 242

4201 Wilson Blvd.	1325 East West Highway

Arlington, VA  22230	Silver Spring, MD  20910

Prof. Joseph M. Moran	Dr. Richard E. Moritz	E.M. Morris

UW-Green Bay	University of Washington	British Antarctic Survey

Dept. of Earth Science	Polar Science Center	High Cross, Madingley Road

Green Bay, WI  54311	1013 N.E. 40th Street	Cambridge CB3 OET

Seattle, WA  98015	UNITED KINGDOM

Robert Morse	Dr. Ellen Mosley-Thompson	Mr. G.B. Mukherji, Director

UW-Madison	Ohio State University	Dept. of Ocean Development

Dept. of Physics	Byrd Polar Research Center	Block 12 C.G.O. Complex

Room 3406 Sterling Hall	1090 Carmack Road	Lodi Road

475 N. Charter Street	Columbus, OH  43210-1002	New Delhi  110003

Madison, WI  53706	INDIA

Prof. Frank J. Murcray	John Nairn	M.R. Nayak

University of Denver	Bureau of Meteorology	National Institute of Oceanography

Dept. of Physics	Antarctic Met. Center	Marine Instrumentation Div.

Denver, CO  80208-0202	Project Development Center	N.I.O. Dona Paula

P.O. Box 1289K, Melbourne 3001	Goa-403 004

Victoria	INDIA

AUSTRALIA

National Center for Atmospheric Research	William Neff	NOAA/MASC Library

NCAR Library - Serials	NOAA/ERL/WPL, R/E/WP7	Acquisitions MC5

P.O. Box 3000	325 Broadway	325 Broadway

Boulder, CO  80307	Boulder, CO  80303	Boulder, CO  80303

NSF Representative	NSF Representative	Kathy Pagan

McMurdo Station	New Zealand	NASA/AMES

PSC 469 Box 700	PSC 467 Box 296	Mail Stop 245-5

APO AP 96599-5700	APO AP 96531-2000	Moffett Field, CA  94035

Dr. Julie Palais	Prof. Thomas Parish	Dr. Byong Kwon Park, Director

National Science Foundation	University of Wyoming	Korea Ocean Res. & Dev. Institute

Office of Polar Programs	Dept. of Atmospheric Sciences	An San P.O. Box 29

Room 755	P.O. Box 3038, Univ. Station	Seoul 425-600

4201 Wilson Blvd.	Laramie, WY  82071	SOUTH KOREA

Arlington, VA  22230

Dr. Andrea Pellegrini	Dr. Polly Penhale	Polar Research Center

PAS/Progetto Antarctide	National Science Foundation	Korean Ocean Research and Dev. Inst.

ENEA-Casaccia	Office of Polar Programs	Ansan P.O. Box 29

C.P. 2400	Room 755	Seoul 425-600

00100 Roma A.D.	4201 Wilson Blvd.	SOUTH KOREA

ITALY	Arlington, VA  22230

Michael Pook	Joseph Prejs	Dr. Tony Press, Director

Antarctic CRC	1501 S. Clifton Ave.	Antarctic Division

GPO Box 252-80	Bloomington, IN  47401-6649	Channel Highway

Hobart, Tasmania 7001	Kingston, Tasmania 7050

AUSTRALIA	AUSTRALIA

Linda Preston, Librarian	Rajmund Przybylak	Dominique Raynaud

Federal Bldg.-Library	Nicholas Copernicus University	Laboratoire de Glaciol. et Geophys. de

Nat. Climatic Data Center/NOAA	Dept. of Climatology	l'Environ.

Asheville, NC  28801-2696	ul. Danielwskiego 6	BP 96 38402 St. Martin d'Heres

87-100 Torun	Cedex

POLAND	FRANCE

Margaret N. Rees	Ms. Winifred Reuning	Dr. David Reusch

University of Nevada	National Science Foundation	Earth and Environmental Systems Institute

Dept. of Geophysics	Office of Polar Programs	517 Deike Building

Las Vegas, NV  89154	Room 755	Pennsylvania State University

4201 Wilson Blvd.	University Park, PA  16802

Arlington, VA  22230

Dr. Carlos A. Rinaldi, Director	Dr. Ted J. Rosenburg	Dr. Thomas F. Ross

Instituto Antarctico	University of Maryland	NOAA/NCDC

Cerrito 1248	Inst. for Phys. Sci. & Tech.	National Climatic Data Center

1010 Buenos Aires	College Park, MD  20742	Room 121

ARGENTINA	Federal Building

Asheville, NC  28801-2696

Comodoro Enrique Saavedra	V.K. Saxena	Akhil Saxena, Section Officer

Servicio Meteorologico Nacional	North Carolina State University	Government of India

25 de Mayo 658	Dept. of Marine, Earth & Atmos. Science	Dept. of Ocean Development

1002 Buenos Aires	Box 8208	Mahasagar Bhavan, Block-12

REPUBLICA ARGENTINA	Raleigh, NC  27695-8208	CGO Complex, Lodi Road

New Delhi 110003

INDIA

Ted Scambos	Greg Scharfen	Guy Schayes

National Snow and Ice Data Center	National Snow and Ice Data Center	UCL-Institut d'Astron. et de Geophys.

University of Colorado	University of Colorado	Chemin du Cyclotron 2

Campus Box 449	Campus Box 449	B-1348 Louvain-La-Neuve

Boulder, CO  80309-0449	Boulder, CO  80309-0449	BELGIUM

Thomas W. Schmidlin	Schwerdtfeger Library	Science & Technology Corp.

Kent State University	Space Science and Engineering Center	101 Research Drive

Dept. of Geography	1225 W. Dayton St., Room-317	Hampton, VA  23666

Kent, OH  44242	Madison, WI 53706	ATTN: C.A. Keen

Science Library	Science Library	Science Library

Amundsen-Scott South Pole Station,	Crary Laboratory	Palmer Station, Antarctica

Antarctica	McMurdo Station, Antarctica	c/o Supervisor, Lab Operations

c/o Inland Stations Manager	c/o Laboratory Manager	61 Inverness Dr. East, Suite 300

7400 South Tuscon Way	61 Inverness Dr. East, Suite 300	Englewood, CO  80112

Centennial, CO 80112-3938	Englewood, CO  80112

Service Central d'Explication Meteorologia	Dr. Alberto Setzer	Jonathan Shanklin

Division Climatologie	Instituto de Pesquisas Espaciais	British Antarctic Survey

2, Division Rapp	Dept. de Meteorologia e Oceanografia	High Cross, Madingley Road

75340 Paris Cedex 07	Caixa Postal 515	Cambridge CB3 OET

FRANCE	12201-Sao Jose dos Campos-SP	UNITED KINGDOM

BRAZIL

Dr. Hayley Shen	Dr. Madan L. Shrestha, Dir. General	Dr. Christopher A. Shuman

Dept. of Civil and Environmental	Dept. of Hydrology & Meteorology	NASA/GFSC

Engineering	P.O. Box No. 406	Code 971

Clarkson University	Babarmahal, Kathmandu	Greenbelt, MD  20771

Potsdam, NY  13699-5710	NEPAL

August L. Shumbera	Prof. Ian Simmonds	Prof. Raymond Smith

Federal Building	School of Earth Sciences	University of CA-Santa Barbara

National Climatic Data Center	University of Melbourne	Computer Systems Laboratory

Asheville, NC  28801	Parkville, Victoria	Santa Barbara, CA  93106

AUSTRALIA 3052

Dr. Philip M. Solomon	Dr. Susan Solomon	Dr. Roland Souchez

SUNY Stoneybrook	NOAA DSRC	Faculte des Sciences

Dept. of Physics	325 Broadway	Dept. des Sci. de la terre et de l'environ.

Stoneybrook, NY  11794	Boulder, CO  80305	CP 160 Ave. F.D. Roosevelt, 50

1050 Brussels

BELGIUM

South Pole Station	Todd Sowers	Sharon Stammerjohn

PSC 468 Box 400	University of Rhode Island	Institute for Computational Earth System

APO AP 96598-5400	URI/GSO S. Ferry Road	Science

Narragansett, RI  02882-1197	UCSD

Sanata Barbara, CA 93106-3060

Station Manager	Mr. Brian Stone	Towanda Street

Palmer Station, Antarctica	National Science Foundation	National Ice Center

c/o AGUNSA	Office of Polar Programs	FOB#4, Room 2301

Deposito Franco Antarctico	Room 755	4251 Suitland Rd.

Av. Independencia 772, P.O. Box 60-D	4201 Wilson Blvd.	Washington, DC  20395

Punta Arenas	Arlington, VA  22230

CHILE

Dr. N.A. Streten	Dr. A.P. Sturman	Mike Scheuermann

Bureau of Meteorology	University of Canterbury	National Science Foundation

Box 1289 K, Melbourne 3000	Dept. of Geography	Office of Polar Programs

Victoria	Christchurch 1	Room 755

AUSTRALIA	NEW ZEALAND	4201 Wilson Blvd.

Arlington, VA  22230

Swedish Polar Research Secretariat,	Dr. Charles Swithinbank	Prof. Masaki Takematsu

Library	Home End Fulbourn	Kyushu University

Box 50005	Cambridge CB1 5BS	Res. Inst. for Applied Mechanics

S10405 Stockholm	UNITED KINGDOM	Kasuga 816

SWEDEN	JAPAN

Tasmanian/Antarctic Reg. Office	Dr. Kendall Taylor	Dr. Calvin Teague

Bureau of Meteorology	Desert Research Institute	Center for Radar Astronomy

Hobart	P.O. Box 60220	Dept. of Electrical Engineering

AUSTRALIA	5625 Fox Ave.	Stanford University

Reno, NV  89506-0220	Stanford, CA  94305

Martha B. Thayer	Geza T. Thuronyi	Dr. Brian A. Tinsley, MSF022

NOAA Library	Library of Congress	University of Texas-Dallas

7600 Sand Point Way NE Bldg. 3	Science & Technology Division	Box 830688

Bin C-15700	Washington, D.C.  20540	Richardson, TX  75083-0688

Seattle, WA  98115

Dr. Adrian Tuck	John Turner	Dr. Rein Vaikmae

R/E/ALG	British Antarctic Survey	Estonian Academy of Science

NOAA/Aeronomy Lab	High Cross, Madingley Road	Institute of Geology

325 Broadway	Cambridge CB3 OET	7 Estonia Puiestee

Boulder, CO  80303	UNITED KINGDOM	200101 Tallinn

ESTONIA

Luca Valenziano	Rodolfo del Valle	Michiel van den Broeke

CNR-TESRE	Instituto Antarctico Argentino	IMAU

via P. Gobetti 101	Cerrito 1248-1010	P.O. Box 80005

I-40129	Buenos Aires	3508 TA Utrecht

Bologna	ARGENTINA	THE NETHERLANDS

ITALY

Dr. Harry van Loon	Michael Van Woert	E.D. Waddington

NCAR	National Science Foundation	University of Washington

P.O. Box 3000	Office of Polar Programs	Geophysics AK-50

Boulder, CO  80307	Room 755	Seattle, WA  98195-1650

4201 Wilson Blvd.

Arlington, VA  22230

Von Walden	John E. Walsh	Stephen G. Warren

Dept. of Geography	University of Illinois	University of Washington

University of Idaho	Dept. of Atmospheric Sciences	Dept. of Atmospheric Sciences

McClure Building, Room 203	1101 W. Springfield Ave.	AK-40

Moscow, ID  83844-3021	Urbana, IL  61801	Seattle, WA  98195

Dr. Gunter Weller	Dr. Gerd Wendler	Prof. G. Wojcik

University of Alaska	University of Alaska	Nicholas Copernicus University

Geophysical Institute	P.O. Box 757320	Inst. Of Geography, Dept. of Climatology

903 Koyukuk Ave.	Fairbanks, AK  99775-7320	ul. Danielewskiego 6

Fairbanks, AK  99701	87-100 Torun

POLAND

Dr. Anthony Worby	World Data Center-A	World Data Center-A

GPO Box 252-80	Glaciology (Snow and Ice)	Marine Geology & Geophysics

Hobart, Tasmania 7001	CIRES/University of Colorado	NOAA, E/GC3

AUSTRALIA	Boulder, CO  80309	325 Broadway

Boulder, CO  80303

World Data Center-A: Meteorology	World Data Center-A: Oceanography	Steven Worley  (2)

NOAA, E/CC	NOAA, E/OC53	NCAR/SCD

Federal Building	Silver Spring, MD 20910-3282	P.O. Box 3000

Asheville, NC  28801	Boulder, CO  80307

Mrs. Xu Pei-Min	Dr.Takashi Yamanouchi	Hengchun Ye

Polar Information Data Center	National Instit. of  Polar Research	California State University, Los Angeles

Polar Res. Institute of China	1-9-10 Kaga, Itabashi-ku	Dept. of Geography and Urban Analysis

Jinqiao Road, Pudong 451	Tokyo, 173-8515	Los Angeles, CA  90032-8222

Shanghai 200129	JAPAN

CHINA

Zhang Yinsheng	Dr. I.A. Zotikov	Dr. Mario Zucchelli, Manager

Cold And Arid Regions	Russian Academy of Sciences	Antarctic Project, ENEA PAS

Environ. and Engineering Res. Institute	Institute of Geography	C.R.E. CASACCIA P.O. Box 2400

Academia Sinica	29 Staromonetny	Via Anguillarese, 301

Lanzhou 730000	Moscow, 119017	00100 Roma A.D.

CHINA	RUSSIA	ITALY

Dr. H.J. Zwally

NASA/Goddard Space Flight Center

Code 971

Greenbelt, MD  20771