### Antarctic Automatic Weather Station Data

for the calendar year 1998

by

- Linda M. Keller

George A. Weidner

Charles R. Stearns

Matthew T. Whittaker

Robert E. Holmes

Space Science and Engineering Center

University of Wisconsin

1225 W. Dayton St.

Madison, Wisconsin

July 2002

- Antarctic Automatic Weather Station Data

- For The Calendar Year

1998

by

- Linda M. Keller

- George A. Weidner

- Charles R. Stearns

- Matthew T. Whittaker

- Robert E. Holmes

- Space Science and Engineering Center

- University of Wisconsin

- 1225 W. Dayton St.

- Madison, Wisconsin

July 2002

- TABLE OF CONTENTS

- Page

1.  Introduction.........................................................................................................................	  1

2.  Data Transmission..............................................................................................................   	  1

3.  AWS Identification and Location......................................................................................... 	  1

4.  AWS Data Summaries........................................................................................................   	  9

4.1. Monthly Data Summaries..............................................................................................  	  9

D-10..............................................................................................................................	 10

D-47..............................................................................................................................	 10

Dome C.II..................................................................................................................... 	 10

Penguin Point............................................................................................................... 	 11

Sutton........................................................................................................................... 	 11

Byrd Station.................................................................................................................. 	 11

Elizabeth....................................................................................................................... 	 12

Erin............................................................................................................................... 	 12

Theresa........................................................................................................................ 	 12

Doug............................................................................................................................. 	 13

Mount Siple...................................................................................................................  	 13

Siple Dome................................................................................................................... 	 13

Swithinbank................................................................................................................... 	 14

Marble Point..................................................................................................................	 14

Ferrell............................................................................................................................ 	 14

Pegasus North...............................................................................................................	 15

Pegasus South.............................................................................................................. 	 15

Minna Bluff.................................................................................................................... 	 15

Linda............................................................................................................................. 	 16

Willie Field....................................................................................................................	 16

Windless Bight..............................................................................................................	 16

White Out......................................................................................................................	 16

White Island..................................................................................................................	 16

Whitlock........................................................................................................................ 	 17

Scott Island...................................................................................................................	 17

Possession Island..........................................................................................................	 17

Manuela.........................................................................................................................	 18

Marilyn...........................................................................................................................	 18

Schwerdtfeger................................................................................................................	 18

Gill.................................................................................................................................	 19

Elaine............................................................................................................................	 19

Lettau............................................................................................................................	 19

Larsen Ice Shelf.............................................................................................................	 20

Butler Island................................................................................................................... 	 20

Uranus Glacier...............................................................................................................	 20

Limbert.......................................................................................................................... 	 21

Racer Rock.................................................................................................................... 	 21

Bonaparte Point.............................................................................................................	 21

Ski-Hi............................................................................................................................. 	 22

Santa Claus Island......................................................................................................... 	 22

Clean Air (South Pole)................................................................................................... 	 22

Henry............................................................................................................................. 	 23

Nico............................................................................................................................... 	 23

Relay Station.................................................................................................................	 23

Dome Fuji...................................................................................................................... 	 24

4.2. Three Hourly Data Summaries....................................................................................... 	 25

5.   AWS Calibration................................................................................................................. 	 26

5.1. Temperature................................................................................................................. 	 26

5.2. Pressure....................................................................................................................... 	 26

5.3. Wind Direction and Speed............................................................................................ 	 26

5.4. Relative Humidity......................................................................................................... 	 27

5.5. Vertical Air Temperature Difference.............................................................................. 	 27

6.   AWS Operations Summary for 1998...................................................................................	 27

6.1. AWS Performance........................................................................................................ 	 27

6.2. AWS Antarctic Field Activities....................................................................................... 	 29

7.   Global Telecommunications System.................................................................................... 	 31

8.   Data Availability................................................................................................................... 	 31

9.   Acknowledgments................................................................................................................ 	 32

- 1.  INTRODUCTION

A network of automatic weather station (AWS) units is deployed to collect Antarctic surface weather observations in support of specific meteorological research projects as well as operational activities at McMurdo, Antarctica.  The 1998 network consisted of 53 installed AWS units providing observations on the Ross Ice Shelf, east of the Transantarctic Mountains and north of McMurdo to the Adelie Coast, along the Antarctic Peninsula, West Antarctica, and climatological locations such as the South Pole.  Each unit measures air temperature, wind speed, and wind direction at the top of the unit's tower at a nominal height of three meters and air pressure at the electronics enclosure (Figure 1).  Some AWS units also measure the relative humidity at three meters and vertical air temperature difference between 0.5 and 3 meters.  Measurement heights relative to the actual surface at the site are nominal due to snow accumulation around the AWS unit.

- 2.  DATA TRANSMISSION

The transmitted AWS data are received and stored by the ARGOS data collection system on the NOAA series of polar orbiting satellites.  The data are retransmitted by the satellite for reception by a local user terminal (LUT) as at McMurdo, Antarctica.  The data are processed into scientific units and are available for local use.  The complete data set is received at Madison, Wisconsin, from Service ARGOS, Largo, Maryland, for processing and distribution to the users.

- 3.  AWS IDENTIFICATION AND LOCATION

Site location is defined by the latitude and longitude which is determined by various methods: sun shots, angles to geographical features, aircraft data, ice breaker data, the platform location system of Service ARGOS, and the Global Positioning System.  AWS elevation is obtained by barometry and should be correct to within +/- 5 meters.  Site names were introduced for convenience.  Table 3.1 lists the site name, ARGOS identification number, latitude, longitude, elevation, start date for the site, and the World Meteorological Organization (WMO) number for the site.  Figures 2, 3, and 4 show the locations of the AWS units in the Antarctic for 1998.

The ARGOS identification number (ID) is used to identify the data sets distributed to the users.  AWS units are sometimes moved from one location to another, and as a result, the ID at a given site may change from year to year. The site name does not change. Table 3.2 lists the site name with the ARGOS ID, the site start date, and the ID start and stop dates.

Figure 1.  Layout of the AWS unit used in the Antarctic.  The installed AWS unit has a 3-meter tower with a horizontal boom supporting the antenna, aerovane for measuring wind speed and direction, air temperature resistance thermometer, upper thermopile for measuring vertical air temperature difference, and the relative humidity sensor.  The electronics enclosure is mounted at the midpoint of the tower.  The gel cell batteries are placed at the tower base.  The solar panel, located near the tower top, faces north.

# Table 3.1

AWS site name, geographic location and elevation, site start date, and WMO number for 1998.

| Site Name | ARGOS ID | Lat. (deg ) | Long. (deg) | Elev. (m) | Site Start Date | WMO No. |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | ## Adelie Land |  |  |  |
| D-10 | 8914 | 66.71S | 139.83E | 243 | 08 Jan 80 | 89832 |
| D-47 | 8986 | 67.397S | 138.726E | 1560 | 24 Nov 82 | 89834 |
| Dome C II | 8989 | 75.121oS | 123.374oE | 3250 | 12 Dec 95 | 89828 |
| Port Martin | 8930 | 66.82oS | 141.40oE | 39 | 19 Jan 90 |  |
| Cape Denison | 8907 | 67.009oS | 142.664oE | 31 | 20 Jan 90 |  |
| Penguin Point | #8910 | 67.617oS | 146.18oE | 30 | 24 Dec 92 | 89847 |
| Sutton | 8939 | 67.08oS | 141.37oE | 871 | 26 Dec 94 |  |
| Cape Webb | 8933 | 67.934oS | 146.824oE | 37 | 28 Dec 94 |  |
|  |  |  | ## West Antarctica |  |  |  |
| Byrd Station | 8903 | 80.007oS | 119.404oW | 1530 | 05 Feb 80 | 89324 |
| Mount Siple | 8981 | 73.198oS | 127.052oW | 230 | 20 Feb 92 | 89327 |
| J.C. | 21357 | 85.07oS | 135.516oW | 549 | 29 Nov 94 |  |
| Theresa | 21358 | 84.599oS | 115.811oW | 1463 | 29 Nov 94 | 89314 |
| Doug | 8922 | 82.315oS | 113.24oW | 1433 | 29 Nov 94 |  |
| Elizabeth | 21361 | 82.606oS | 137.082oW | 549 | 30 Nov 94 | 89332 |
| Brianna | 21362 | 83.887oS | 134.145oW | 549 | 30 Nov 94 |  |
| Harry | 21355 | 83.00S | 121.69W | 945 | 29 Nov 94 |  |
| Erin | 21363 | 84.901oS | 128.81oW | 1006 | 29 Nov 94 |  |
| Siple Dome* | 8900 | 81.656oS | 148.773oW | 620 | 21 Jan 97 | 89345 |
| Swithinbank* | 21356 | 81.20oS | 126.174oW | 945 | 18 Jan 97 |  |
|  |  |  | ## Ross Island Region |  |  |  |
| Marble Point | 8906 | 77.439oS | 163.759oE | 120 | 05 Feb 80 | 89866 |
| Ferrell | 8934 | 77.928oS | 170.82oE | 45 | 10 Dec 80 | 89872 |
| Pegasus North | 8927 | 77.952oS | 166.505oE | 10 | 23 Jan 90 | 89667 |
| Pegasus South | 8937 | 77.99oS | 166.576oE | 10 | 14 Jan 91 |  |
| Minna Bluff | #21360 | 78.554oS | 166.656oE | 920 | 22 Jan 91 | 89768 |
| Linda | #8919 | 78.48oS | 168.375oE | 50 | 21 Jan 91 | 89769 |
| Willie Field | 8901 | 77.865oS | 167.017oE | 40 | 25 Jan 92 |  |
| Windless Bight* | 8901 | 77.728S | 167.703E | 61 | 09 Dec 98 |  |
| White Out* | 8697 | 77.87S | 168.16E | 30 | 01 Dec 98 |  |
| White Island* | 8722 | 78.09S | 168.01E | 30 | 01 Dec 98 |  |
|  |  |  | ## Ocean Islands |  |  |  |
| Whitlock | 8921 | 76.144oS | 168.392oE | 274 | 23 Jan 82 | 89865 |
| Scott Island | 8983 | 67.37oS | 179.97oW | 30 | 25 Dec 87 | 89371 |
| Young Island | 8980 | 66.229oS | 162.275oE | 30 | 01 Jan 91 | 89660 |
| Possession Is. | 8984 | 71.891oS | 171.21oE | 30 | 29 Dec 92 | 89879 |
| Manuela | 8905 | 74.946oS | 163.687oE | 80 | 06 Feb 84 | 89864 |
|  |  |  | ## Ross Ice Shelf |  |  |  |
| Marilyn | 8931 | 79.954oS | 165.13oE | 75 | 16 Jan 84 | 89869 |
| Schwerdtfeger | 8913 | 79.904oS | 169.973oE | 60 | 24 Jan 85 | 89868 |
| Gill | 8911 | 79.985oS | 178.611oW | 55 | 24 Jan 85 | 89376 |
| Lettau | 8908 | 82.518oS | 174.452oW | 55 | 29 Jan 86 | 89377 |
| Elaine | #8915 | 83.134oS | 174.169oE | 60 | 28 Jan 86 | 89873 |

| Site Name | ARGOS ID | Lat. (deg) | Long. (deg) | Elev. (m) | Site Start Date | WMO No. |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | ## Antarctic Peninsula |  |  |  |
| Larsen Ice | 8926 | 66.949oS | 60.914oW | 17 | 21 Oct 85 | 89262 |
| Butler Island | 8902 | 72.207oS | 60.171oW | 91 | 01 Mar 86 | 89266 |
| Uranus Glacier | 8920 | 71.43oS | 68.93oW | 780 | 06 Mar 86 | 89264 |
| Limbert | 8925 | 75.422oS | 59.948oW | 40 | 30 Nov 95 |  |
| Racer Rock | 8947 | 64.067oS | 61.613oW | 17 | 15 Oct 89 | 89261 |
| Bonaparte Pt. | 8923 | 64.778oS | 64.067oW | 8 | 05 Jan 92 | 89269 |
| AGO-A84 | 8932 | 84.36oS | 23.86oW | 2103 | 09 Jan 96 |  |
| Ski-Hi | 8917 | 74.975oS | 70.766oW | 1395 | 21 Feb 94 | 89272 |
| Santa Claus Is. | 21364 | 64.964oS | 65.67oW | 25 | 10 Dec 94 |  |
|  |  |  | ## High Polar Plateau |  |  |  |
| Clean Air | 8987 | 90.00oS |  | 2835 | 29 Jan 86 | 89208 |
| Henry | 8985 | 89.011oS | 1.025oW | 2755 | 26 Jan 93 | 89108 |
| Nico | 8924 | 89.00oS | 89.669oE | 2935 | 26 Jan 93 | 89799 |
| Relay Station | 8918 | 74.017oS | 43.062oE | 3353 | 01 Feb 95 | 89744 |
| Dome Fuji | 8982 | 77.31 | 39.70oE | 3810 | 08 Feb 95 | 89734 |

* New sites started during 1998

# New ARGOS ID for 1998 at the site

Figure 2.  Antarctic automatic weather station locations during 1998 identified by the site name. Area around Ross Island is shown in Figure 3. Adelie Coast area is shown in Figure 4.

Figure 3.  Location of Antarctic automatic weather stations in the vicinity of Ross Island, Antarctica during 1998.

Figure 4.  Location of Antarctic automatic weather stations along the Adelie Coast during 1998.

- Table 3.2

1998 Antarctic AWS site name, ARGOS identification number (ID), site start date, ID start date, and ID stop date if occurring in 1998.

| Site | ARGOS ID | Site Start Date | ID Start Date | ID Stop Date |
| --- | --- | --- | --- | --- |
| D-10 | 8914 | 08 Jan 80 | 28 Jan 98 |  |
| D-47 | 8986 | 24 Nov 82 | 11 Feb 96 |  |
| Dome C II | 8989 | 12 Dec 95 | 12 Dec 95 |  |
| Port Martin | 8930 | 19 Jan 90 | 23 Dec 92 |  |
| Cape Denison | 8907 | 20 Jan 90 | 27 Dec 94 |  |
| Penguin Point | 8929 | 24 Dec 92 | 24 Dec 92 | 21 Dec 98 |
|  | 8910 |  | 21 Dec 98 |  |
| Sutton | 8939 | 26 Dec 94 | 26 Dec 94 |  |
| Cape Webb | 8933 | 28 Dec 94 | 28 Dec 94 |  |
| Byrd Station | 8903 | 05 Feb 80 | 05 Feb 80 |  |
| Mount Siple | 8981 | 20 Feb 92 | 20 Feb 92 |  |
| J.C. | 21357 | 29 Nov 94 | 29 Nov 94 |  |
| Theresa | 21358 | 29 Nov 94 | 29 Nov 94 |  |
| Doug | 8922 | 29 Nov 94 | 20 Jan 97 |  |
| Harry | 21355 | 29 Nov 94 | 29 Nov 94 |  |
| Elizabeth | 21361 | 30 Nov 94 | 17 Jan 96 |  |
| Brianna | 21362 | 30 Nov 94 | 30 Nov 94 |  |
| Erin | 21363 | 29 Nov 94 | 18 Jan 96 |  |
| Siple Dome | 8900 | 21 Jan 97 | 21 Jan 97 |  |
| Swithinbank | 21356 | 18 Jan 97 | 18 Jan 97 |  |
| Marble Point | 8906 | 05 Feb 80 | 05 Feb 80 |  |
| Ferrell | 8934 | 10 Dec 80 | 13 Jan 93 |  |
| Pegasus North | 8927 | 23 Jan 90 | 23 Jan 90 |  |
| Pegasus South | 8937 | 14 Jan 91 | 14 Jan 91 |  |
| Minna Bluff | 8988 | 22 Jan 91 | 12 Jan 94 | 26 Jan 98 |
|  | 21360 |  | 26 Jan 98 |  |
| Linda | 8909 | 21 Jan 91 | 24 Jan 95 | 15 Jan 98 |
|  | 8919 |  | 15 Jan 98 |  |
| Willie Field | 8901 | 25 Jan 92 | 25 Jan 92 | 04 Dec 98 |
| Windless Bight | 8901 | 09 Dec 98 | 09 Dec 98 |  |
| White Out | 8697 | 01 Dec 98 | 01 Dec 98 |  |
| White Island | 8722 | 01 Dec 98 | 01 Dec 98 |  |
| Whitlock | 8921 | 23 Jan 82 | 23 Feb 94 |  |
| Scott Island | 8983 | 25 Dec 87 | 27 Dec 92 |  |
| Young Island | 8980 | 01 Jan 91 | 01 Jan 91 |  |
| Possession Island | 8984 | 29 Dec 92 | 29 Dec 92 |  |
| Marilyn | 8931 | 16 Jan 84 | 18 Jan 91 |  |
| Schwerdtfeger | 8913 | 24 Jan 85 | 22 Jan 93 |  |
| Gill | 8911 | 24 Jan 85 | 25 Jan 91 |  |
| Elaine | 8915 | 23 Jan 93 | 02 Jan 97 |  |
| Lettau | 8908 | 29 Jan 86 | 29 Jan 86 |  |
| Manuela | 8905 | 06 Feb 84 | 15 Feb 87 |  |
| Lynn | 8935 | 19 Jan 88 | 23 Jan 92 | 05 Jan 98 |
| Larsen Ice Shelf | 8926 | 21 Oct 85 | 01 Jan 86 |  |
| Butler Island | 8902 | 01 Mar 86 | 01 Mar 86 |  |
| Uranus Glacier | 8920 | 06 Mar 86 | 24 Jan 92 |  |
| Limbert | 8925 | 30 Nov 95 | 30 Nov 95 |  |
| Racer Rock | 8947 | 15 Oct 89 | 08 Dec 91 |  |
| Bonaparte Point | 8923 | 05 Jan 92 | 23 Dec 96 | 23 Dec 96 |
| AGO-A84 | 8932 | 09 Jan 96 | 09 Jan 96 |  |
| Ski-Hi | 8917 | 21 Feb 94 | 21 Feb 94 |  |
| Santa Claus Is. | 21364 | 10 Dec 94 | 26 Dec 96 |  |
| Clean Air | 8987 | 29 Jan 86 | 25 Jan 94 |  |
| Henry | 8985 | 26 Jan 93 | 26 Jan 93 |  |
| Nico | 8924 | 26 Jan 93 | 26 Jan 93 |  |
| Relay Station | 8918 | 01 Feb 95 | 01 Feb 95 |  |
| Dome Fuji | 8982 | 08 Feb 95 | 08 Feb 95 |  |

4. AWS DATA SUMMARIES

The data received by the University of Wisconsin, Space Science and Engineering Center, contain all the information received by the ARGOS system including duplicate and erroneous transmissions.  Invalid data are eliminated during a quality check, and the valid data are converted to scientific units producing the complete data set.  Data selected at three hourly intervals, plus or minus forty minutes, produce a three hourly data set for each AWS unit month.  Section 6.1, AWS Performance, provides some explanations for missing and invalid data.

Use of the 1998 Antarctic AWS data for publication should acknowledge the support of NSF-OPP Grant 9726040 or reference this publication.

- 4.1.  Monthly Data Summaries

The monthly summaries consist of the monthly means, from the three hourly data set, and the extremes, from the complete data set.  For monthly values to be included, 25% of the three hourly observations must be available.  Months with 50-75% of data missing occur most often when a station is started or stopped in the middle of the month.  This can cause a bias in the monthly mean, especially during seasons when parameters such as temperature change rapidly.  Annual means are calculated only when twelve months of data are available.  The data are presented in the same order as the sites listed in Table 3.1.  Definitions of the monthly data summary headings are listed below.

Heading                                                Definition

Mean air temperature, oC.	Mean value for the month.

Percent of monthly data missing. 	Ratio of the number of missing observations to the number of

possible observations X 100.

Maximum air temperature, oC.	Maximum value for the month.

Minimum air temperature, oC.	Minimum value for the month.

Mean wind speed, m/s. 		            Mean value for the month.

Percent of monthly data missing.	Ratio of the number of missing observations to the number of

possible observations X 100.

Resultant wind speed, dir/vv.	Resultant speed and direction for the month.

Constancy.                  		Ratio of the monthly resultant to the monthly mean wind speed.

Maximum wind speed, dir/vv.	Maximum wind speed and direction for the month.

Mean air pressure, mb.		Mean value for the month.

Percent of monthly data missing.	Ratio of the number of missing observations to the number of

possible observations X 100.

Maximum air pressure, mb.	Maximum value for the month.

Minimum air pressure, mb.	Minimum value for the month.

|  | Mean | % of |  |  | Mean | % of |  |  |  |  |  | Mean | % of |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Air | Mon | Max Air | Min Air | Wind | Mon |  |  |  | Max |  | Air | Mon | Max Air | Min Air |
|  | Temp | Data | Temp | Temp | Speed | Data | Result |  |  | Wind |  | Press | Data | Press | Press |
| Mon | (C) | Abs | (C) | (C) | (m/s) | Abs | Wind (dir | vv) | Con | (dir | vv) | (mb) | Abs | (mb) | (mb) |
| D-10 (8914) |  |  |  | 66.71S |  |  |  | 139.83E |  |  |  | 243 M |  |  |  |  |
| Feb | -5.4 | 09 | 1.9 | -13.4 | 11.6 | 08 | 146 | 11.0 | 0.95 | 147 | 35 | 946.7 | 08 | 961.9 | 923.1 |
| Mar | -13.0 | 07 | -3.8 | -22.6 | 11.2 | 07 | 159 | 10.6 | 0.95 | 153 | 29 | 946.9 | 07 | 958.8 | 933.2 |
| Apr | -16.5 | 08 | -5.2 | -25.5 | 13.9 | 25 | 158 | 13.3 | 0.96 | 160 | 32 | 942.7 | 08 | 962.7 | 916.0 |
| May | -17.4 | 09 | -4.0 | -27.0 | 9.8 | 46 | 159 | 8.5 | 0.87 | 168 | 32 | 953.5 | 09 | 970.8 | 923.4 |
| Jun | -19.3 | 20 | -10.6 | -29.0 | 10.0 | 42 | 154 | 9.3 | 0.93 | 151 | 30 | 948.9 | 20 | 966.5 | 931.8 |
| Jul |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Aug | -20.5 | 57 | -12.4 | -30.5 | 9.0 | 68 | 166 | 8.4 | 0.94 | 196 | 28 | 950.1 | 57 | 970.2 | 934.8 |
| Sep | -18.0 | 10 | -10.8 | -28.5 | 10.6 | 29 | 154 | 10.5 | 0.99 | 153 | 32 | 945.2 | 10 | 967.1 | 926.8 |
| Oct | -16.2 | 54 | -10.8 | -25.9 | 10.1 | 54 | 159 | 9.6 | 0.95 | 153 | 39 | 947.5 | 54 | 960.7 | 928.1 |
| Nov |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Dec | -2.6 | 66 | 3.6 | -8.5 | 12.3 | 66 | 133 | 11.8 | 0.96 | 112 | 26 | 953.8 | 66 | 966.8 | 942.1 |
| D-47 (8986) |  |  |  | 67.40S |  |  |  | 138.73E |  |  |  | 1560 M |  |  |  |  |
| Feb |  |  |  |  | 13.3 | 18 | 046 | 12.8 | 0.96 | 012 | 25 | 804.1 | 18 | 816.7 | 783.9 |
| Mar |  |  |  |  | 12.1 | 04 | 064 | 11.5 | 0.95 | 053 | 20 | 800.8 | 04 | 814.2 | 790.9 |
| Apr |  |  |  |  | 12.9 | 10 | 059 | 12.4 | 0.96 | 037 | 24 | 797.0 | 10 | 814.2 | 779.6 |
| May |  |  |  |  | 11.1 | 06 | 072 | 10.0 | 0.90 | 037 | 25 | 805.4 | 06 | 818.2 | 784.0 |
| Jun |  |  |  |  | 12.3 | 18 | 073 | 11.8 | 0.96 | 037 | 27 | 800.2 | 18 | 817.1 | 787.3 |
| Jul |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Aug |  |  |  |  | 12.3 | 55 | 076 | 11.0 | 0.89 | 066 | 23 | 802.8 | 55 | 831.6 | 770.0 |
| Sep |  |  |  |  | 13.1 | 49 | 047 | 12.6 | 0.96 | 028 | 24 | 797.4 | 49 | 815.0 | 782.8 |
| Oct |  |  |  |  | 12.0 | 10 | 047 | 11.2 | 0.94 | 029 | 25 | 801.4 | 07 | 811.1 | 786.9 |
| Nov |  |  |  |  | 10.5 | 05 | 071 | 10.1 | 0.97 | 077 | 23 | 804.6 | 05 | 827.9 | 786.5 |
| Dec |  |  |  |  | 11.3 | 11 | 039 | 10.2 | 0.91 | 019 | 27 | 806.4 | 11 | 822.0 | 790.7 |
| Dome C II (8989) |  |  |  |  | 75.12S |  |  |  | 123.37E |  |  |  | 3250 M |  |  |  |  |
| Jan | -30.4 | 00 | -20.8 | -46.2 | 2.7 | 00 | 221 | 1.7 | 0.63 | 192 | 10 | 649.6 | 00 | 657.9 | 643.2 |
| Feb | -38.1 | 00 | -24.6 | -57.8 | 3.0 | 00 | 219 | 1.3 | 0.42 | 279 | 11 | 651.3 | 00 | 657.8 | 642.4 |
| Mar | -56.9 | 00 | -40.4 | -70.6 | 3.3 | 00 | 187 | 2.4 | 0.74 | 178 | 13 | 644.1 | 00 | 659.8 | 634.1 |
| Apr | -60.0 | 00 | -35.0 | -71.2 | 3.6 | 00 | 275 | 0.9 | 0.25 | 023 | 15 | 640.4 | 00 | 654.6 | 631.2 |
| May | -64.0 | 00 | -41.9 | -76.2 | 2.7 | 00 | 218 | 1.1 | 0.41 | 080 | 14 | 647.7 | 00 | 657.9 | 636.5 |
| Jun | -64.7 | 01 | -47.8 | -77.1 | 2.3 | 01 | 181 | 1.6 | 0.71 | 195 | 8 | 648.3 | 01 | 663.4 | 640.1 |
| Jul | -63.0 | 00 | -40.5 | -76.8 | 4.9 | 00 | 188 | 2.9 | 0.58 | 092 | 13 | 646.0 | 00 | 661.9 | 632.0 |
| Aug | -62.4 | 00 | -29.8 | -75.5 | 3.8 | 00 | 225 | 1.9 | 0.50 | 285 | 14 | 645.7 | 00 | 667.0 | 630.9 |
| Sep | -62.4 | 02 | -43.0 | -80.1 | 3.7 | 02 | 175 | 2.0 | 0.55 | 158 | 12 | 646.3 | 02 | 658.2 | 632.0 |
| Oct | -50.2 | 00 | -29.2 | -69.4 | 3.5 | 00 | 154 | 0.9 | 0.24 | 201 | 10 | 646.2 | 00 | 664.4 | 633.4 |
| Nov | -38.6 | 00 | -20.9 | -58.1 | 3.9 | 00 | 227 | 2.6 | 0.68 | 215 | 13 | 649.4 | 00 | 669.8 | 635.6 |
| Dec | -27.9 | 00 | -15.6 | -38.5 | 3.0 | 00 | 153 | 0.9 | 0.31 | 111 | 11 | 653.3 | 00 | 668.9 | 643.3 |
| MEAN | -51.6 |  |  |  | 3.4 |  | 201 | 1.5 | 0.50 |  |  | 647.4 |  |  |  |

|  | Mean | % of |  |  | Mean | % of |  |  |  |  |  | Mean | % of |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Air | Mon | Max Air | Min Air | Wind | Mon |  |  |  | Max |  | Air | Mon | Max Air | Min Air |
|  | Temp | Data | Temp | Temp | Speed | Data | Result |  |  | Wind |  | Press | Data | Press | Press |
| Mon | (C) | Abs | (C) | (C) | (m/s) | Abs | Wind (dir | vv) | Con | (dir | vv) | (mb) | Abs | (mb) | (mb) |
| Penguin Point (8910) |  |  |  |  | 67.62S |  |  |  | 146.18E |  |  |  | 30 M |  |  |  |  |
| Dec | -1.0 | 66 | 4.2 | -5.2 | 10.3 | 68 | 148 | 9.6 | 0.93 | 147 | 23 | 980.7 | 66 | 990.5 | 970.9 |
| Sutton (8939) |  |  |  | 67.08S |  |  |  | 141.37E |  |  |  | 871 M |  |  |  |  |
| Jan | -10.7 | 06 | -3.6 | -19.5 |  |  |  |  |  |  |  | 882.8 | 06 | 891.8 | 862.8 |
| Feb | -12.5 | 06 | -4.5 | -20.4 |  |  |  |  |  |  |  | 877.4 | 06 | 891.5 | 854.5 |
| Mar | -21.5 | 07 | -10.1 | -31.0 |  |  |  |  |  |  |  | 875.3 | 07 | 888.3 | 863.4 |
| Apr | -24.1 | 08 | -11.8 | -32.0 |  |  |  |  |  |  |  | 871.5 | 08 | 891.2 | 852.7 |
| May |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Jun |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Jul |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Aug |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sep | -24.3 | 24 | -16.0 | -36.4 |  |  |  |  |  |  |  | 874.3 | 24 | 891.4 | 855.2 |
| Oct |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Nov |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Dec | -10.4 | 46 | -4.1 | -19.2 |  |  |  |  |  |  |  | 875.3 | 46 | 891.3 | 863.2 |
| Byrd (8903) |  |  |  | 80.01S |  |  |  | 119.40W |  |  |  | 1530 M |  |  |  |  |
| Jan | -14.0 | 00 | -3.9 | -29.4 | 3.2 | 02 | 010 | 2.4 | 0.75 | 028 | 15 | 815.4 | 00 | 821.3 | 801.5 |
| Feb | -16.1 | 00 | -6.4 | -31.1 | 5.6 | 00 | 008 | 5.3 | 0.95 | 024 | 21 | 813.3 | 00 | 823.0 | 801.2 |
| Mar | -28.0 | 00 | -14.2 | -41.2 | 5.6 | 00 | 003 | 4.6 | 0.82 | 003 | 13 | 808.4 | 00 | 820.4 | 787.9 |
| Apr | -34.1 | 00 | -18.7 | -52.4 | 7.0 | 03 | 008 | 6.4 | 0.92 | 017 | 22 | 799.2 | 00 | 814.5 | 776.4 |
| May | -24.8 | 00 | -8.1 | -56.3 | 9.3 | 11 | 348 | 7.3 | 0.78 | 021 | 23 | 811.3 | 00 | 829.5 | 790.7 |
| Jun | -26.7 | 00 | -12.3 | -43.8 | 8.6 | 03 | 018 | 8.2 | 0.96 | 015 | 22 | 807.0 | 00 | 822.8 | 778.4 |
| Jul | -41.4 | 00 | -25.4 | -55.1 | 8.1 | 43 | 015 | 8.0 | 0.99 | 021 | 27 | 801.0 | 00 | 813.6 | 785.9 |
| Aug | -35.9 | 00 | -21.6 | -57.0 | 9.8 | 66 | 009 | 9.6 | 0.98 | 013 | 22 | 800.0 | 00 | 816.6 | 781.5 |
| Sep | -36.8 | 00 | -15.2 | -59.1 | 9.0 | 29 | 010 | 8.7 | 0.97 | 017 | 20 | 807.3 | 00 | 826.1 | 787.1 |
| Oct | -33.0 | 00 | -13.6 | -51.4 |  |  |  |  |  |  |  | 804.7 | 00 | 819.4 | 788.2 |
| Nov | -23.3 | 00 | -11.1 | -45.5 | 10.5 | 72 | 023 | 9.8 | 0.93 | 031 | 22 | 801.1 | 00 | 813.6 | 784.0 |
| Dec | -15.0 | 00 | -5.5 | -25.0 | 4.1 | 23 | 004 | 3.8 | 0.92 | 011 | 13 | 812.5 | 00 | 822.5 | 797.8 |
| MEAN | -27.4 |  |  |  |  |  |  |  |  |  |  | 806.8 |  |  |  |

|  | Mean | % of |  |  | Mean | % of |  |  |  |  |  | Mean | % of |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Air | Mon | Max Air | Min Air | Wind | Mon |  |  |  | Max |  | Air | Mon | Max Air | Min Air |
|  | Temp | Data | Temp | Temp | Speed | Data | Result |  |  | Wind |  | Press | Data | Press | Press |
| Mon | (C) | Abs | (C) | (C) | (m/s) | Abs | Wind (dir | vv) | Con | (dir | vv) | (mb) | Abs | (mb) | (mb) |
| Elizabeth (21361) |  |  |  |  | 82.61S |  |  |  | 137.08W |  |  |  | 549 M |  |  |  |  |
| Jan | -11.0 | 21 | -2.1 | -24.4 | 3.2 | 19 | 029 | 1.8 | 0.56 | 343 | 11 | 925.7 | 19 | 932.4 | 913.9 |
| Feb | -15.2 | 00 | -0.6 | -30.4 | 4.9 | 00 | 040 | 3.7 | 0.76 | 081 | 13 | 920.4 | 00 | 935.9 | 898.9 |
| Mar | -26.3 | 00 | -9.5 | -40.5 | 5.7 | 00 | 047 | 4.9 | 0.86 | 360 | 14 | 918.6 | 00 | 934.8 | 904.2 |
| Apr | -32.4 | 00 | -8.4 | -45.1 | 5.3 | 00 | 044 | 4.6 | 0.87 | 068 | 14 | 909.7 | 00 | 932.6 | 879.6 |
| May | -27.2 | 00 | -4.8 | -45.4 | 7.2 | 54 | 014 | 5.6 | 0.79 | 043 | 23 | 919.4 | 00 | 938.3 | 897.8 |
| Jun | -25.9 | 00 | -7.9 | -42.8 | 6.6 | 49 | 049 | 6.0 | 0.91 | 045 | 23 | 914.0 | 00 | 930.1 | 888.5 |
| Jul | -36.7 | 00 | -20.6 | -47.8 | 6.2 | 00 | 046 | 5.7 | 0.91 | 078 | 15 | 912.8 | 00 | 931.0 | 898.5 |
| Aug |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sep |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Oct | -28.7 | 04 | -7.6 | -42.4 | 6.4 | 04 | 051 | 5.7 | 0.88 | 057 | 18 | 916.2 | 04 | 930.1 | 892.9 |
| Nov | -18.2 | 00 | -7.0 | -42.4 | 5.3 | 00 | 060 | 4.3 | 0.81 | 081 | 16 | 910.2 | 00 | 927.4 | 886.7 |
| Dec | -10.8 | 01 | -2.0 | -17.1 | 3.3 | 01 | 044 | 2.0 | 0.60 | 015 | 9 | 921.2 | 01 | 932.9 | 907.0 |
| Erin (21363) |  |  |  | 84.90S |  |  |  | 128.81W |  |  |  | 1006 M |  |  |  |  |
| Jan | -10.9 | 00 | -4.5 | -22.2 |  |  |  |  |  |  |  | 878.6 | 00 | 885.7 | 866.6 |
| Feb | -13.1 | 00 | -5.4 | -24.5 |  |  |  |  |  |  |  | 875.4 | 00 | 886.2 | 861.9 |
| Mar | -24.6 | 00 | -12.9 | -36.8 |  |  |  |  |  |  |  | 873.0 | 00 | 884.4 | 853.4 |
| Apr | -29.6 | 00 | -16.8 | -40.5 |  |  |  |  |  |  |  | 863.8 | 00 | 879.7 | 844.5 |
| May | -25.0 | 01 | -7.4 | -44.2 |  |  |  |  |  |  |  | 872.6 | 01 | 887.5 | 857.4 |
| Jun | -20.6 | 00 | -9.4 | -27.9 |  |  |  |  |  |  |  | 871.7 | 00 | 886.5 | 849.9 |
| Jul | -30.0 | 41 | -18.8 | -38.5 |  |  |  |  |  |  |  | 866.7 | 41 | 875.5 | 853.3 |
| Aug |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sep |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Oct | -26.9 | 40 | -8.4 | -40.4 |  |  |  |  |  |  |  | 871.4 | 40 | 888.6 | 856.3 |
| Nov | -17.8 | 00 | -9.0 | -30.9 |  |  |  |  |  |  |  | 867.0 | 00 | 879.9 | 850.1 |
| Dec | -10.8 | 00 | -5.9 | -16.2 |  |  |  |  |  |  |  | 875.3 | 00 | 887.5 | 859.9 |
| Theresa (21358) |  |  |  |  | 84.60S |  |  |  | 115.81W |  |  |  | 1463 M |  |  |  |  |
| Jan | -11.6 | 00 | -2.9 | -20.4 | 6.5 | 00 | 072 | 6.0 | 0.92 | 091 | 15 | 821.2 | 00 | 829.4 | 809.1 |
| Feb | -14.3 | 00 | -8.0 | -25.0 | 8.5 | 00 | 080 | 8.2 | 0.96 | 090 | 22 | 818.9 | 00 | 827.5 | 809.8 |
| Mar | -24.9 | 00 | -15.0 | -37.4 | 10.4 | 00 | 067 | 9.7 | 0.93 | 045 | 21 | 814.3 | 00 | 825.0 | 794.0 |
| Apr | -30.5 | 00 | -20.4 | -45.4 | 11.2 | 00 | 081 | 10.9 | 0.97 | 095 | 29 | 805.2 | 00 | 816.9 | 790.4 |
| May | -24.9 | 00 | -7.2 | -47.2 | 9.2 | 00 | 065 | 7.5 | 0.82 | 081 | 27 | 814.2 | 00 | 828.3 | 800.8 |
| Jun | -21.0 | 00 | -9.6 | -30.6 | 10.6 | 00 | 076 | 9.5 | 0.90 | 085 | 22 | 815.0 | 00 | 829.4 | 791.8 |
| Jul | -30.0 | 00 | -19.0 | -42.4 | 10.9 | 00 | 069 | 9.8 | 0.90 | 060 | 19 | 808.9 | 00 | 821.2 | 792.7 |
| Aug | -28.4 | 00 | -18.4 | -51.1 | 11.1 | 00 | 079 | 10.4 | 0.94 | 085 | 26 | 807.4 | 00 | 821.9 | 793.2 |
| Sep | -31.1 | 00 | -14.2 | -49.4 | 10.1 | 00 | 079 | 9.7 | 0.96 | 068 | 24 | 813.6 | 00 | 831.6 | 796.6 |
| Oct | -25.9 | 00 | -10.4 | -41.2 | 9.7 | 00 | 067 | 8.7 | 0.89 | 054 | 27 | 812.2 | 00 | 831.2 | 799.7 |
| Nov | -19.7 | 00 | -10.8 | -31.5 | 11.2 | 00 | 080 | 10.8 | 0.96 | 082 | 28 | 809.4 | 00 | 820.7 | 794.7 |
| Dec | -12.1 | 00 | -6.8 | -20.6 | 6.8 | 00 | 071 | 6.3 | 0.93 | 081 | 15 | 818.3 | 00 | 829.7 | 804.2 |
| MEAN | -22.9 |  |  |  | 9.7 |  | 074 | 8.9 | 0.92 |  |  | 813.2 |  |  |  |

|  | Mean | % of |  |  | Mean | % of |  |  |  |  |  | Mean | % of |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Air | Mon | Max Air | Min Air | Wind | Mon |  |  |  | Max |  | Air | Mon | Max Air | Min Air |
|  | Temp | Data | Temp | Temp | Speed | Data | Result |  |  | Wind |  | Press | Data | Press | Press |
| Mon | (C) | Abs | (C) | (C) | (m/s) | Abs | Wind (dir | vv) | Con | (dir | vv) | (mb) | Abs | (mb) | (mb) |
| Doug (8922) |  |  |  | 82.32S |  |  |  | 113.24W |  |  |  | 1433 M |  |  |  |  |
| Oct | -31.2 | 34 | -12.6 | -46.4 | 9.2 | 34 | 075 | 8.0 | 0.87 | 080 | 27 | 807.8 | 34 | 823.3 | 793.6 |
| Nov | -21.9 | 02 | -11.4 | -39.1 | 9.3 | 02 | 091 | 8.5 | 0.92 | 358 | 22 | 803.6 | 02 | 815.2 | 788.0 |
| Dec | -13.6 | 09 | -4.2 | -19.1 | 5.6 | 09 | 076 | 5.0 | 0.89 | 087 | 13 | 813.3 | 09 | 824.6 | 800.2 |
| Mount Siple (8981) |  |  |  |  | 73.20S |  |  |  | 127.05W |  |  |  | 230 M |  |  |  |  |
| Jan | -1.9 | 00 | 4.6 | -7.0 |  |  |  |  |  |  |  |  |  |  |  |
| Feb | -3.2 | 00 | 1.8 | -8.6 |  |  |  |  |  |  |  |  |  |  |  |
| Mar | -8.7 | 00 | -1.2 | -16.1 |  |  |  |  |  |  |  | 953.6 | 01 | 972.5 | 923.8 |
| Apr | -13.2 | 00 | -6.5 | -24.2 |  |  |  |  |  |  |  | 943.9 | 00 | 962.6 | 909.9 |
| May | -12.1 | 00 | -0.8 | -29.9 |  |  |  |  |  |  |  | 962.2 | 02 | 987.8 | 934.3 |
| Jun | -12.6 | 00 | -2.2 | -26.1 |  |  |  |  |  |  |  | 946.2 | 00 | 972.5 | 921.7 |
| Jul | -19.3 | 01 | -9.4 | -31.6 |  |  |  |  |  |  |  | 948.0 | 01 | 966.3 | 917.2 |
| Aug | -13.8 | 00 | -6.1 | -22.8 |  |  |  |  |  |  |  | 940.0 | 00 | 975.1 | 907.8 |
| Sep | -14.8 | 01 | -2.5 | -25.1 |  |  |  |  |  |  |  | 951.9 | 01 | 975.0 | 913.2 |
| Oct | -12.2 | 00 | -2.2 | -22.5 |  |  |  |  |  |  |  | 947.6 | 01 | 967.7 | 916.8 |
| Nov | -8.8 | 01 | 0.9 | -17.5 |  |  |  |  |  |  |  | 942.7 | 04 | 962.6 | 915.6 |
| Dec | -3.0 | 01 | 2.9 | -7.6 |  |  |  |  |  |  |  | 954.2 | 27 | 964.7 | 936.0 |
| MEAN | -10.3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Siple Dome (8900) |  |  |  |  | 81.66S |  |  |  | 148.77W |  |  |  | 620 M |  |  |  |  |
| Jan | -11.7 | 00 | -1.4 | -19.8 | 3.7 | 03 | 022 | 1.2 | 0.32 | 161 | 11 | 907.3 | 00 | 913.9 | 895.6 |
| Feb | -13.7 | 00 | -2.2 | -25.1 | 4.0 | 00 | 033 | 0.8 | 0.21 | 002 | 20 | 901.6 | 00 | 916.8 | 878.2 |
| Mar | -20.8 | 00 | -10.2 | -37.5 | 4.5 | 33 | 032 | 1.9 | 0.41 | 354 | 15 | 898.9 | 00 | 915.7 | 882.3 |
| Apr | -28.1 | 00 | -10.6 | -46.2 |  |  |  |  |  |  |  | 890.0 | 00 | 914.2 | 859.9 |
| May | -26.6 | 00 | -8.1 | -40.9 |  |  |  |  |  |  |  | 899.2 | 00 | 916.8 | 876.9 |
| Jun | -23.6 | 00 | -8.0 | -40.6 |  |  |  |  |  |  |  | 893.9 | 00 | 911.1 | 872.2 |
| Jul | -31.1 | 00 | -16.0 | -44.6 |  |  |  |  |  |  |  | 892.9 | 00 | 910.7 | 878.9 |
| Aug | -27.9 | 00 | -17.1 | -46.9 |  |  |  |  |  |  |  | 890.4 | 00 | 919.1 | 866.7 |
| Sep | -31.3 | 00 | -13.5 | -48.0 |  |  |  |  |  |  |  | 899.8 | 00 | 919.6 | 879.0 |
| Oct | -24.7 | 00 | -5.8 | -39.2 | 5.1 | 51 | 026 | 1.4 | 0.28 | 074 | 18 | 895.9 | 00 | 909.0 | 872.2 |
| Nov | -20.4 | 00 | -8.0 | -38.8 | 5.0 | 00 | 043 | 1.6 | 0.31 | 009 | 16 | 890.4 | 00 | 908.6 | 866.8 |
| Dec | -11.9 | 00 | -2.9 | -20.8 | 3.5 | 00 | 043 | 0.9 | 0.25 | 336 | 12 | 902.8 | 00 | 914.2 | 890.2 |
| MEAN | -22.7 |  |  |  |  |  |  |  |  |  |  | 896.9 |  |  |  |

|  | Mean | % of |  |  | Mean | % of |  |  |  |  |  | Mean | % of |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Air | Mon | Max Air | Min Air | Wind | Mon |  |  |  | Max |  | Air | Mon | Max Air | Min Air |
|  | Temp | Data | Temp | Temp | Speed | Data | Result |  |  | Wind |  | Press | Data | Press | Press |
| Mon | (C) | Abs | (C) | (C) | (m/s) | Abs | Wind (dir | vv) | Con | (dir | vv) | (mb) | Abs | (mb) | (mb) |
| Swithinbank (21356) |  |  |  |  | 81.20S |  |  |  | 126.17W |  |  |  | 945 M |  |  |  |  |
| Jan | -10.2 | 00 | 3.4 | -25.1 | 4.0 | 00 | 354 | 3.1 | 0.76 | 008 | 12 | 874.8 | 00 | 880.3 | 861.1 |
| Feb | -11.6 | 00 | 0.0 | -26.6 | 7.0 | 00 | 357 | 6.6 | 0.94 | 050 | 20 | 870.6 | 00 | 883.2 | 853.9 |
| Mar | -22.5 | 00 | -9.8 | -34.8 | 7.6 | 00 | 359 | 7.1 | 0.93 | 005 | 20 | 867.5 | 00 | 882.4 | 850.0 |
| Apr | -27.8 | 00 | -10.6 | -43.2 | 8.0 | 00 | 359 | 7.6 | 0.95 | 360 | 19 | 858.3 | 00 | 878.7 | 831.4 |
| May | -20.9 | 00 | -4.6 | -41.1 | 9.2 | 00 | 344 | 7.8 | 0.85 | 360 | 23 | 869.0 | 00 | 887.1 | 847.0 |
| Jun | -21.3 | 00 | -7.2 | -34.2 | 11.2 | 00 | 005 | 10.9 | 0.97 | 016 | 23 | 864.0 | 00 | 879.9 | 836.3 |
| Jul | -33.2 | 00 | -18.8 | -45.4 | 10.5 | 00 | 002 | 10.3 | 0.98 | 004 | 27 | 860.8 | 00 | 875.7 | 847.0 |
| Aug | -30.8 | 00 | -18.4 | -48.9 | 10.6 | 00 | 005 | 10.3 | 0.97 | 360 | 27 | 858.8 | 00 | 882.6 | 838.3 |
| Sep | -30.5 | 00 | -10.2 | -47.8 | 9.4 | 00 | 360 | 9.0 | 0.96 | 005 | 23 | 867.4 | 00 | 884.5 | 844.4 |
| Oct | -26.1 | 00 | -8.4 | -40.5 | 9.2 | 00 | 004 | 8.6 | 0.93 | 360 | 25 | 864.5 | 00 | 879.2 | 842.8 |
| Nov | -17.2 | 00 | -6.8 | -36.5 | 7.5 | 00 | 003 | 6.8 | 0.91 | 012 | 20 | 859.4 | 00 | 874.4 | 839.7 |
| Dec | -9.7 | 00 | -2.4 | -16.4 | 5.4 | 00 | 353 | 5.0 | 0.93 | 008 | 13 | 870.5 | 00 | 881.5 | 854.2 |
| MEAN | -21.8 |  |  |  | 8.3 |  | 360 | 7.7 | 0.92 |  |  | 865.5 |  |  |  |
| Marble Point (8906) |  |  |  |  | 77.44S |  |  |  | 163.76E |  |  |  | 120 M |  |  |  |  |
| Jan | -4.1 | 00 | 0.8 | -10.6 | 4.3 | 00 | 132 | 1.7 | 0.41 | 146 | 12 | 977.5 | 00 | 985.7 | 969.1 |
| Feb | -7.6 | 00 | 1.2 | -16.4 | 4.4 | 00 | 152 | 3.8 | 0.85 | 123 | 17 | 973.6 | 00 | 985.6 | 942.8 |
| Mar | -17.3 | 00 | -8.5 | -27.2 | 3.2 | 00 | 166 | 2.2 | 0.71 | 111 | 16 | 975.3 | 00 | 996.1 | 958.2 |
| Apr | -26.3 | 00 | -9.9 | -38.6 | 2.8 | 00 | 170 | 1.9 | 0.68 | 151 | 22 | 968.9 | 00 | 990.4 | 941.8 |
| May | -22.8 | 11 | -4.0 | -35.2 | 4.2 | 11 | 159 | 3.2 | 0.75 | 109 | 25 | 975.6 | 11 | 993.0 | 948.4 |
| Ferrell (8934) |  |  |  | 77.93S |  |  |  | 170.82E |  |  |  | 45 M |  |  |  |  |
| Jan | -7.5 | 01 | 2.9 | -20.9 | 3.1 | 00 | 171 | 1.3 | 0.44 | 201 | 12 | 985.3 | 00 | 993.1 | 977.1 |
| Feb | -11.5 | 00 | -3.6 | -23.5 | 5.0 | 00 | 204 | 3.8 | 0.76 | 225 | 19 | 980.6 | 00 | 992.3 | 948.8 |
| Mar | -24.2 | 00 | -14.8 | -43.2 | 5.3 | 00 | 219 | 4.3 | 0.81 | 202 | 18 | 982.1 | 00 | 1003.7 | 964.9 |
| Apr | -35.7 | 00 | -15.9 | -52.5 | 3.7 | 00 | 213 | 2.9 | 0.79 | 205 | 21 | 976.1 | 00 | 998.0 | 947.6 |
| May | -29.9 | 00 | -7.8 | -44.0 | 6.8 | 12 | 218 | 5.4 | 0.79 | 219 | 24 | 982.7 | 00 | 999.7 | 954.6 |
| Jun | -23.9 | 00 | -11.4 | -43.9 | 7.3 | 28 | 220 | 7.0 | 0.95 | 232 | 21 | 977.5 | 00 | 992.9 | 958.4 |
| Jul | -34.5 | 00 | -16.8 | -53.5 | 5.2 | 15 | 220 | 4.7 | 0.89 | 198 | 19 | 980.7 | 00 | 998.3 | 963.9 |
| Aug | -34.7 | 21 | -16.1 | -50.8 | 6.4 | 43 | 220 | 6.1 | 0.95 | 203 | 21 | 980.8 | 21 | 1003.6 | 964.5 |

|  | Mean | % of |  |  | Mean | % of |  |  |  |  |  | Mean | % of |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Air | Mon | Max Air | Min Air | Wind | Mon |  |  |  | Max |  | Air | Mon | Max Air | Min Air |
|  | Temp | Data | Temp | Temp | Speed | Data | Result |  |  | Wind |  | Press | Data | Press | Press |
| Mon | (C) | Abs | (C) | (C) | (m/s) | Abs | Wind (dir | vv) | Con | (dir | vv) | (mb) | Abs | (mb) | (mb) |
| Pegasus North (8927) |  |  |  |  | 77.95S |  |  |  | 166.51E |  |  |  | 10 M |  |  |  |  |
| Jan | -5.9 | 00 | 3.6 | -21.2 |  |  |  |  |  |  |  | 993.2 | 00 | 1001.8 | 984.7 |
| Feb | -9.5 | 00 | -0.6 | -21.6 | 4.3 | 00 | 094 | 2.8 | 0.64 | 185 | 21 | 989.7 | 00 | 1002.4 | 954.5 |
| Mar | -20.7 | 00 | -9.1 | -38.9 | 4.7 | 00 | 098 | 2.1 | 0.45 | 184 | 21 | 992.0 | 00 | 1013.4 | 974.5 |
| Apr | -32.0 | 00 | -12.2 | -52.0 | 3.0 | 00 | 094 | 1.5 | 0.50 | 171 | 33 | 986.2 | 00 | 1007.4 | 956.0 |
| May | -26.7 | 00 | -5.9 | -41.5 | 5.6 | 00 | 127 | 3.1 | 0.56 | 185 | 38 | 992.8 | 00 | 1011.1 | 963.3 |
| Jun | -20.7 | 00 | -4.8 | -37.1 | 4.9 | 00 | 135 | 2.2 | 0.45 | 189 | 29 | 987.3 | 00 | 1002.4 | 968.3 |
| Jul | -29.1 | 00 | -10.4 | -46.1 | 4.8 | 00 | 092 | 2.7 | 0.55 | 184 | 31 | 990.8 | 00 | 1008.6 | 974.2 |
| Aug | -29.1 | 00 | -10.2 | -46.0 | 4.7 | 00 | 100 | 2.5 | 0.54 | 181 | 28 | 989.4 | 00 | 1014.0 | 973.2 |
| Sep | -32.6 | 00 | -3.2 | -50.9 | 4.5 | 00 | 084 | 2.6 | 0.58 | 187 | 28 | 999.1 | 00 | 1021.6 | 975.5 |
| Oct | -18.9 | 00 | -3.9 | -30.8 | 5.5 | 03 | 133 | 2.5 | 0.45 | 180 | 29 | 990.2 | 00 | 1005.3 | 967.6 |
| Nov | -12.0 | 00 | -2.0 | -32.4 | 4.5 | 00 | 108 | 1.9 | 0.42 | 174 | 24 | 983.2 | 00 | 1002.0 | 965.9 |
| Dec | -4.7 | 00 | 3.9 | -14.4 | 3.9 | 00 | 078 | 2.0 | 0.51 | 182 | 21 | 989.5 | 00 | 1002.9 | 972.4 |
| MEAN | -20.2 |  |  |  |  |  |  |  |  |  |  | 990.3 |  |  |  |
| Pegasus South (8937) |  |  |  |  | 77.99S |  |  |  | 166.58E |  |  |  | 10 M |  |  |  |  |
| Jan | -5.9 | 01 | 2.1 | -21.0 | 2.2 | 01 | 080 | 1.4 | 0.63 | 153 | 8 | 992.0 | 01 | 1000.5 | 983.3 |
| Feb | -9.7 | 00 | -1.6 | -21.9 | 2.9 | 00 | 123 | 1.6 | 0.54 | 198 | 15 | 988.5 | 00 | 1001.0 | 955.1 |
| Mar | -21.0 | 00 | -9.5 | -40.0 | 2.3 | 00 | 139 | 0.8 | 0.35 | 201 | 15 | 990.8 | 00 | 1012.1 | 973.5 |
| Apr | -32.7 | 01 | -12.9 | -52.9 | 0.5 | 02 | 076 | 0.2 | 0.41 | 174 | 5 | 984.8 | 01 | 1006.3 | 955.8 |
| May | -27.6 | 00 | -5.4 | -42.5 | 0.5 | 00 | 143 | 0.2 | 0.35 | 202 | 6 | 991.8 | 00 | 1009.8 | 963.1 |
| Jun | -21.5 | 01 | -6.0 | -37.5 |  |  |  |  |  |  |  | 986.3 | 01 | 1001.3 | 967.2 |
| Jul | -30.0 | 02 | -11.0 | -46.9 |  |  |  |  |  |  |  | 989.7 | 02 | 1007.4 | 973.3 |
| Aug | -30.1 | 00 | -11.1 | -47.2 |  |  |  |  |  |  |  | 988.5 | 00 | 1013.0 | 973.3 |
| Sep | -33.6 | 01 | -3.0 | -51.4 |  |  |  |  |  |  |  | 998.2 | 01 | 1020.6 | 975.5 |
| Oct | -19.0 | 01 | -3.8 | -32.0 |  |  |  |  |  |  |  | 989.3 | 01 | 1004.4 | 967.0 |
| Nov | -12.2 | 00 | -3.0 | -33.6 |  |  |  |  |  |  |  | 982.4 | 00 | 1001.3 | 964.9 |
| Dec | -5.0 | 03 | 2.2 | -15.4 |  |  |  |  |  |  |  | 988.5 | 03 | 1001.9 | 970.7 |
| MEAN | -20.7 |  |  |  |  |  |  |  |  |  |  | 989.2 |  |  |  |
| Minna Bluff (21360) |  |  |  |  | 78.55S |  |  |  | 166.66E |  |  |  | 920 M |  |  |  |  |
| Feb | -15.5 | 00 | -6.8 | -25.5 | 7.3 | 00 | 221 | 3.8 | 0.52 | 022 | 30 | 877.6 | 00 | 889.0 | 845.6 |
| Mar | -24.6 | 00 | -15.9 | -36.9 | 7.3 | 00 | 209 | 2.2 | 0.30 | 123 | 25 | 875.8 | 00 | 894.7 | 859.0 |
| Apr | -30.4 | 00 | -20.0 | -41.0 | 7.3 | 00 | 330 | 5.5 | 0.75 | 351 | 39 | 867.2 | 00 | 890.0 | 841.8 |
| May | -28.5 | 00 | -13.8 | -44.6 | 9.1 | 00 | 348 | 6.6 | 0.73 | 338 | 38 | 874.2 | 00 | 889.8 | 845.9 |
| Jun | -23.6 | 00 | -15.2 | -33.1 | 3.0 | 00 | 200 | 2.5 | 0.83 | 236 | 28 | 871.9 | 00 | 887.7 | 856.9 |
| Jul | -29.5 | 00 | -18.6 | -43.6 |  |  |  |  |  |  |  | 871.8 | 00 | 887.1 | 849.3 |
| Aug | -28.6 | 00 | -16.2 | -38.6 |  |  |  |  |  |  |  | 871.2 | 00 | 891.0 | 853.2 |
| Sep | -30.5 | 00 | -15.0 | -39.2 |  |  |  |  |  |  |  | 878.5 | 00 | 899.9 | 858.4 |
| Oct | -23.9 | 00 | -10.6 | -35.4 |  |  |  |  |  |  |  | 873.8 | 00 | 888.5 | 848.9 |
| Nov | -17.3 | 00 | -6.6 | -31.8 |  |  |  |  |  |  |  | 870.5 | 00 | 887.3 | 849.2 |
| Dec | -10.6 | 00 | -1.6 | -20.6 |  |  |  |  |  |  |  | 879.8 | 00 | 893.1 | 864.6 |

|  | Mean | % of |  |  | Mean | % of |  |  |  |  |  | Mean | % of |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Air | Mon | Max Air | Min Air | Wind | Mon |  |  |  | Max |  | Air | Mon | Max Air | Min Air |
|  | Temp | Data | Temp | Temp | Speed | Data | Result |  |  | Wind |  | Press | Data | Press | Press |
| Mon | (C) | Abs | (C) | (C) | (m/s) | Abs | Wind (dir | vv) | Con | (dir | vv) | (mb) | Abs | (mb) | (mb) |
| Linda (8919) |  |  |  | 78.48S |  |  |  | 168.38E |  |  |  | 50 M |  |  |  |  |
| Jan | -9.5 | 46 | -2.0 | -20.4 | 3.6 | 46 | 198 | 0.8 | 0.21 | 172 | 14 | 965.6 | 46 | 973.2 | 959.3 |
| Feb | -12.7 | 00 | -1.8 | -24.4 | 6.7 | 00 | 192 | 5.5 | 0.82 | 213 | 25 | 962.5 | 00 | 974.5 | 929.3 |
| Mar | -23.8 | 00 | -14.4 | -41.9 | 7.8 | 00 | 201 | 6.7 | 0.86 | 202 | 26 | 963.4 | 00 | 985.5 | 943.3 |
| Apr | -33.9 | 00 | -15.4 | -53.8 | 5.9 | 00 | 200 | 4.4 | 0.76 | 191 | 27 | 957.0 | 00 | 979.9 | 927.2 |
| May | -30.0 | 00 | -8.5 | -47.6 | 9.3 | 20 | 197 | 8.5 | 0.91 | 198 | 32 | 964.0 | 00 | 981.6 | 933.3 |
| Jun | -23.1 | 00 | -11.6 | -42.8 | 11.0 | 26 | 198 | 10.8 | 0.98 | 199 | 28 | 958.6 | 00 | 973.5 | 940.0 |
| Jul | -32.1 | 00 | -16.2 | -52.4 | 11.5 | 43 | 197 | 11.3 | 0.98 | 201 | 27 | 961.7 | 00 | 979.4 | 944.4 |
| Aug | -32.3 | 00 | -15.8 | -50.4 | 13.1 | 40 | 197 | 12.9 | 0.98 | 199 | 32 | 960.3 | 00 | 984.9 | 942.3 |
| Sep | -35.0 | 00 | -10.4 | -51.1 | 9.7 | 32 | 194 | 9.4 | 0.96 | 198 | 28 | 969.9 | 00 | 992.2 | 945.6 |
| Oct | -22.2 | 00 | -9.0 | -34.4 | 8.6 | 00 | 199 | 7.8 | 0.91 | 210 | 27 | 961.7 | 00 | 977.1 | 938.3 |
| Nov | -14.4 | 00 | -5.5 | -35.6 | 6.3 | 00 | 201 | 4.8 | 0.76 | 201 | 23 | 955.8 | 00 | 974.8 | 935.2 |
| Dec | -6.9 | 00 | 2.2 | -17.6 | 4.6 | 00 | 200 | 2.2 | 0.48 | 212 | 19 | 963.7 | 00 | 977.7 | 947.2 |
| MEAN | -23.0 |  |  |  | 8.2 |  | 197 | 7.1 | 0.80 |  |  | 962.0 |  |  |  |
| Willie Field (8901) |  |  |  |  | 77.87S |  |  |  | 167.02E |  |  |  | 20 M |  |  |  |  |
| Jan | -6.5 | 00 | 2.9 | -20.9 | 3.1 | 00 | 068 | 1.9 | 0.60 | 180 | 14 | 991.2 | 00 | 997.9 | 981.3 |
| Feb | -10.6 | 00 | -1.2 | -24.8 | 3.6 | 00 | 079 | 2.1 | 0.57 | 189 | 17 | 986.9 | 00 | 1000.6 | 952.1 |
| Mar | -21.8 | 00 | -11.0 | -39.6 | 3.7 | 00 | 080 | 1.3 | 0.35 | 177 | 14 | 988.6 | 00 | 1011.2 | 968.3 |
| Apr | -33.2 | 47 | -19.2 | -52.4 | 1.7 | 47 | 072 | 1.1 | 0.68 | 168 | 11 | 987.7 | 47 | 1003.0 | 969.1 |
| May |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Jun |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Jul |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Aug |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sep |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Oct | -20.7 | 31 | -3.4 | -33.4 | 3.6 | 31 | 108 | 1.7 | 0.47 | 189 | 22 | 989.1 | 31 | 1003.0 | 963.8 |
| Nov | -12.8 | 00 | -1.4 | -35.9 | 3.2 | 00 | 098 | 1.0 | 0.33 | 185 | 18 | 980.5 | 00 | 997.9 | 963.0 |
| Dec |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Windless Bight (8901) |  |  |  |  | 77.72S |  |  |  | 167.70E |  |  |  | 50 M |  |  |  |  |
| Dec | -5.4 | 26 | 3.8 | -18.8 | 1.6 | 26 | 073 | 0.6 | 0.37 | 172 | 10 | 986.2 | 26 | 997.9 | 968.5 |
| White Out (8697) |  |  |  |  | 77.87S |  |  |  | 168.16E |  |  |  | 30 M |  |  |  |  |
| Dec | -6.6 | 00 | 1.9 | -17.1 | 2.9 | 00 | 187 | 0.7 | 0.23 | 192 | 16 | 984.0 | 00 | 997.9 | 966.9 |
| White Island (8722) |  |  |  |  | 78.09S |  |  |  | 168.91E |  |  |  | 30 M |  |  |  |  |
| Dec | -5.9 | 01 | 2.5 | -15.6 | 3.7 | 01 | 207 | 0.9 | 0.25 | 205 | 21 | 980.7 | 01 | 995.1 | 963.5 |

|  | Mean | % of |  |  | Mean | % of |  |  |  |  |  | Mean | % of |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Air | Mon | Max Air | Min Air | Wind | Mon |  |  |  | Max |  | Air | Mon | Max Air | Min Air |
|  | Temp | Data | Temp | Temp | Speed | Data | Result |  |  | Wind |  | Press | Data | Press | Press |
| Mon | (C) | Abs | (C) | (C) | (m/s) | Abs | Wind (dir | vv) | Con | (dir | vv) | (mb) | Abs | (mb) | (mb) |
| Whitlock (8921) |  |  |  | 76.14S |  |  |  | 168.39E |  |  |  | 274 M |  |  |  |  |
| Jan | -5.0 | 01 | 1.5 | -11.1 | 3.6 | 00 | 104 | 1.1 | 0.31 | 338 | 13 | 959.3 | 00 | 965.9 | 949.6 |
| Feb | -7.7 | 01 | -2.4 | -15.5 | 5.0 | 01 | 179 | 1.7 | 0.35 | 182 | 20 | 954.3 | 01 | 964.4 | 925.7 |
| Mar | -18.8 | 00 | -10.0 | -33.0 | 4.1 | 00 | 319 | 0.8 | 0.19 | 338 | 15 | 955.7 | 00 | 975.9 | 941.6 |
| Apr | -27.5 | 36 | -15.9 | -39.5 | 4.2 | 39 | 313 | 1.4 | 0.34 | 310 | 16 | 950.2 | 36 | 969.3 | 926.4 |
| Scott Island (8983) |  |  |  |  | 67.37S |  |  |  | 179.97W |  |  |  | 30 M |  |  |  |  |
| Jan | -0.1 | 04 | 5.1 | -3.2 |  |  |  |  |  |  |  | 981.9 | 04 | 997.3 | 958.8 |
| Feb | -0.2 | 06 | 2.1 | -2.9 |  |  |  |  |  |  |  | 977.1 | 06 | 998.3 | 947.1 |
| Mar | -2.4 | 08 | 1.9 | -7.6 |  |  |  |  |  |  |  | 972.6 | 08 | 987.7 | 950.6 |
| Apr | -5.4 | 08 | 0.4 | -18.4 |  |  |  |  |  |  |  | 970.1 | 08 | 999.0 | 946.9 |
| May | -11.9 | 09 | 0.4 | -21.9 |  |  |  |  |  |  |  | 979.8 | 09 | 1011.8 | 949.2 |
| Jun | -15.3 | 08 | -4.8 | -23.4 |  |  |  |  |  |  |  | 970.9 | 08 | 989.9 | 941.0 |
| Jul | -17.5 | 10 | 0.2 | -30.0 |  |  |  |  |  |  |  | 974.1 | 10 | 1001.3 | 940.1 |
| Aug | -18.9 | 08 | -0.5 | -32.6 |  |  |  |  |  |  |  | 968.8 | 08 | 997.5 | 946.7 |
| Sep | -14.6 | 10 | 0.2 | -25.0 |  |  |  |  |  |  |  | 975.7 | 10 | 1001.7 | 954.5 |
| Oct | -10.2 | 13 | 0.4 | -22.8 |  |  |  |  |  |  |  | 974.0 | 13 | 998.6 | 923.2 |
| Nov | -7.9 | 11 | 1.4 | -19.8 |  |  |  |  |  |  |  | 972.8 | 11 | 994.1 | 946.6 |
| Dec | -1.5 | 06 | 3.6 | -8.2 |  |  |  |  |  |  |  | 980.4 | 06 | 999.8 | 937.8 |
| MEAN | -8.8 |  |  |  |  |  |  |  |  |  |  | 974.9 |  |  |  |
| Possession Is (8984) |  |  |  |  | 71.89S |  |  |  | 171.21E |  |  |  | 30 M |  |  |  |  |
| Jan | 0.6 | 20 | 7.2 | -4.8 |  |  |  |  |  |  |  | 977.8 | 19 | 987.0 | 965.0 |
| Feb | -2.2 | 11 | 2.1 | -5.8 |  |  |  |  |  |  |  | 972.8 | 07 | 984.9 | 947.5 |
| Mar | -11.4 | 00 | -2.0 | -21.5 |  |  |  |  |  |  |  | 974.0 | 00 | 987.8 | 960.4 |
| Apr | -20.2 | 01 | -7.0 | -29.2 |  |  |  |  |  |  |  | 969.5 | 01 | 986.4 | 948.6 |
| May | -18.5 | 00 | -1.9 | -28.8 |  |  |  |  |  |  |  | 975.1 | 00 | 1000.3 | 951.6 |
| Jun | -16.4 | 01 | -7.8 | -26.2 |  |  |  |  |  |  |  | 971.3 | 01 | 984.5 | 955.1 |
| Jul | -20.7 | 02 | -8.0 | -29.8 |  |  |  |  |  |  |  | 974.2 | 02 | 991.2 | 954.5 |
| Aug | -19.3 | 01 | -10.6 | -30.0 |  |  |  |  |  |  |  | 970.3 | 01 | 994.8 | 947.6 |
| Sep | -20.5 | 01 | -5.2 | -29.5 |  |  |  |  |  |  |  | 979.0 | 01 | 1000.1 | 962.0 |
| Oct | -13.3 | 10 | 1.0 | -20.9 |  |  |  |  |  |  |  | 974.1 | 10 | 989.7 | 957.4 |
| Nov | -6.7 | 18 | 4.2 | -20.0 |  |  |  |  |  |  |  | 968.0 | 17 | 987.7 | 953.7 |
| Dec | 1.6 | 29 | 7.8 | -6.8 |  |  |  |  |  |  |  | 975.8 | 29 | 991.7 | 956.7 |
| MEAN | -12.3 |  |  |  |  |  |  |  |  |  |  | 973.5 |  |  |  |

|  | Mean | % of |  |  | Mean | % of |  |  |  |  |  | Mean | % of |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Air | Mon | Max Air | Min Air | Wind | Mon |  |  |  | Max |  | Air | Mon | Max Air | Min Air |
|  | Temp | Data | Temp | Temp | Speed | Data | Result |  |  | Wind |  | Press | Data | Press | Press |
| Mon | (C) | Abs | (C) | (C) | (m/s) | Abs | Wind (dir | vv) | Con | (dir | vv) | (mb) | Abs | (mb) | (mb) |
| Manuela (8905) |  |  |  | 74.95S |  |  |  | 163.69E |  |  |  | 80 M |  |  |  |  |
| Jan | -5.6 | 00 | 2.2 | -14.8 |  |  |  |  |  |  |  | 981.3 | 00 | 990.2 | 971.4 |
| Feb | -10.4 | 00 | -2.9 | -19.8 |  |  |  |  |  |  |  | 976.6 | 00 | 988.6 | 945.3 |
| Mar | -22.0 | 00 | -12.6 | -34.4 |  |  |  |  |  |  |  | 978.8 | 00 | 1000.0 | 962.2 |
| Apr | -27.4 | 00 | -7.2 | -35.6 |  |  |  |  |  |  |  | 971.6 | 00 | 993.4 | 945.1 |
| May | -25.7 | 00 | -12.4 | -38.2 |  |  |  |  |  |  |  | 978.9 | 00 | 997.1 | 950.3 |
| Jun | -23.5 | 00 | -10.5 | -33.8 |  |  |  |  |  |  |  | 974.5 | 00 | 989.9 | 952.1 |
| Jul | -27.6 | 00 | -13.8 | -37.1 |  |  |  |  |  |  |  | 976.9 | 00 | 996.5 | 957.7 |
| Aug | -25.6 | 00 | -5.6 | -34.1 |  |  |  |  |  |  |  | 975.3 | 00 | 997.8 | 951.9 |
| Sep | -28.2 | 00 | -9.6 | -39.1 |  |  |  |  |  |  |  | 984.4 | 00 | 1008.8 | 965.1 |
| Oct | -18.5 | 00 | -6.4 | -27.6 |  |  |  |  |  |  |  | 976.9 | 00 | 990.2 | 956.8 |
| Nov | -11.1 | 00 | -0.8 | -23.6 |  |  |  |  |  |  |  | 970.9 | 00 | 989.2 | 951.3 |
| Dec | -3.1 | 00 | 4.0 | -9.0 |  |  |  |  |  |  |  | 978.1 | 00 | 992.5 | 960.8 |
| MEAN | -19.1 |  |  |  |  |  |  |  |  |  |  | 977.0 |  |  |  |
| Marilyn (8931) |  |  |  | 79.95S |  |  |  | 165.13E |  |  |  | 75 M |  |  |  |  |
| Jan | -9.0 | 00 | 2.2 | -21.5 | 2.4 | 01 | 234 | 1.1 | 0.44 | 200 | 13 | 983.2 | 00 | 991.7 | 973.4 |
| Feb | -13.1 | 00 | -1.1 | -26.8 | 4.7 | 00 | 248 | 3.2 | 0.69 | 274 | 19 | 979.8 | 00 | 992.9 | 949.1 |
| Mar | -25.6 | 00 | -14.1 | -41.1 | 5.7 | 00 | 243 | 4.7 | 0.82 | 260 | 15 | 980.4 | 00 | 999.7 | 962.4 |
| Apr | -32.6 | 00 | -15.8 | -50.8 | 7.0 | 03 | 255 | 5.9 | 0.84 | 255 | 23 | 973.2 | 00 | 993.9 | 943.6 |
| May | -31.3 | 00 | -7.8 | -48.6 | 8.1 | 22 | 247 | 6.7 | 0.83 | 269 | 25 | 981.1 | 00 | 1001.6 | 951.7 |
| Jun | -23.5 | 00 | -9.6 | -36.8 | 8.1 | 05 | 243 | 6.8 | 0.84 | 262 | 20 | 975.7 | 00 | 990.9 | 955.2 |
| Jul | -32.6 | 00 | -17.5 | -52.9 | 8.2 | 15 | 251 | 7.5 | 0.92 | 243 | 28 | 978.5 | 00 | 997.1 | 961.5 |
| Aug | -32.4 | 00 | -15.8 | -52.8 | 8.1 | 14 | 254 | 7.3 | 0.90 | 262 | 24 | 977.5 | 00 | 1001.5 | 962.5 |
| Sep | -35.1 | 00 | -10.5 | -53.5 | 7.8 | 15 | 260 | 7.0 | 0.91 | 263 | 29 | 986.3 | 00 | 1009.7 | 967.0 |
| Oct | -24.2 | 00 | -10.6 | -38.1 | 8.8 | 27 | 237 | 7.8 | 0.89 | 264 | 27 | 978.9 | 00 | 992.7 | 959.8 |
| Nov | -15.8 | 00 | -5.2 | -36.8 | 4.0 | 09 | 237 | 2.7 | 0.69 | 228 | 13 | 972.5 | 00 | 990.5 | 954.7 |
| Dec | -7.4 | 01 | 1.6 | -16.8 | 2.4 | 02 | 241 | 0.9 | 0.37 | 179 | 10 | 979.7 | 00 | 993.5 | 963.8 |
| MEAN | -23.6 |  |  |  | 6.3 |  | 248 | 5.1 | 0.76 |  |  | 978.9 |  |  |  |
| Schwerdtfeger (8913) |  |  |  |  | 79.90S |  |  |  | 169.97E |  |  |  | 60 M |  |  |  |  |
| Jan | -9.9 | 00 | -0.5 | -20.6 | 2.9 | 16 | 243 | 1.0 | 0.33 | 177 | 10 | 981.8 | 00 | 990.9 | 973.0 |
| Feb | -14.7 | 00 | -2.2 | -26.8 | 4.3 | 00 | 224 | 2.8 | 0.65 | 208 | 16 | 977.2 | 00 | 990.1 | 945.6 |
| Mar | -28.5 | 00 | -15.2 | -44.8 | 4.8 | 00 | 220 | 3.7 | 0.76 | 188 | 15 | 977.9 | 00 | 998.5 | 957.9 |
| Apr | -38.9 | 00 | -18.5 | -55.1 | 4.9 | 00 | 244 | 3.5 | 0.72 | 175 | 17 | 971.2 | 00 | 992.7 | 939.2 |
| May | -34.9 | 00 | -11.4 | -54.5 | 5.5 | 00 | 232 | 3.8 | 0.70 | 161 | 20 | 978.8 | 00 | 997.3 | 948.5 |
| Jun | -27.7 | 00 | -12.5 | -45.8 | 5.7 | 00 | 230 | 4.4 | 0.76 | 258 | 19 | 972.8 | 00 | 988.2 | 954.2 |
| Jul | -38.6 | 00 | -17.5 | -58.8 | 5.2 | 00 | 233 | 4.4 | 0.83 | 262 | 17 | 976.1 | 00 | 994.7 | 956.8 |
| Aug | -37.4 | 00 | -19.8 | -55.4 | 5.6 | 00 | 236 | 4.5 | 0.81 | 185 | 18 | 975.1 | 00 | 998.9 | 958.2 |
| Sep | -40.7 | 00 | -12.0 | -57.1 | 5.4 | 00 | 243 | 4.3 | 0.78 | 258 | 26 | 984.3 | 00 | 1008.1 | 961.0 |
| Oct | -27.3 | 00 | -15.5 | -40.2 | 6.0 | 00 | 217 | 4.7 | 0.78 | 191 | 22 | 975.9 | 00 | 990.7 | 954.5 |
| Nov | -17.8 | 00 | -4.9 | -39.8 | 5.3 | 00 | 212 | 3.6 | 0.69 | 172 | 19 | 969.8 | 00 | 988.1 | 948.8 |
| Dec | -8.7 | 00 | -1.2 | -18.9 | 2.4 | 00 | 218 | 0.6 | 0.25 | 151 | 10 | 978.1 | 00 | 992.2 | 962.9 |
| MEAN | -27.1 |  |  |  | 4.8 |  | 230 | 3.4 | 0.67 |  |  | 976.6 |  |  |  |

|  | Mean | % of |  |  | Mean | % of |  |  |  |  |  | Mean | % of |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Air | Mon | Max Air | Min Air | Wind | Mon |  |  |  | Max |  | Air | Mon | Max Air | Min Air |
|  | Temp | Data | Temp | Temp | Speed | Data | Result |  |  | Wind |  | Press | Data | Press | Press |
| Mon | (C) | Abs | (C) | (C) | (m/s) | Abs | Wind (dir | vv) | Con | (dir | vv) | (mb) | Abs | (mb) | (mb) |
| Gill (8911) |  |  |  | 79.99S |  |  |  | 178.61W |  |  |  | 55 M |  |  |  |  |
| Jan | -9.1 | 00 | -1.1 | -18.0 | 0.8 | 00 | 325 | 0.3 | 0.43 | 118 | 8 | 985.2 | 00 | 995.0 | 975.8 |
| Feb | -14.4 | 00 | -4.2 | -28.2 | 4.3 | 00 | 211 | 1.9 | 0.44 | 214 | 12 | 979.2 | 00 | 994.0 | 950.5 |
| Mar | -29.0 | 00 | -9.9 | -42.0 | 4.4 | 00 | 213 | 2.9 | 0.65 | 217 | 12 | 979.1 | 00 | 1002.7 | 959.6 |
| Apr | -41.7 | 00 | -21.9 | -55.8 | 2.9 | 00 | 242 | 1.5 | 0.52 | 219 | 13 | 972.8 | 00 | 997.3 | 938.6 |
| May | -34.3 | 00 | -15.2 | -50.6 | 5.5 | 08 | 208 | 2.9 | 0.53 | 181 | 17 | 979.8 | 00 | 996.0 | 950.3 |
| Jun | -30.6 | 00 | -9.2 | -49.0 | 3.6 | 13 | 232 | 2.4 | 0.65 | 233 | 13 | 973.5 | 00 | 991.5 | 950.8 |
| Jul | -44.6 | 00 | -23.6 | -58.2 | 3.4 | 00 | 246 | 2.5 | 0.73 | 205 | 11 | 976.8 | 00 | 995.5 | 960.4 |
| Aug | -39.6 | 00 | -18.1 | -53.6 |  |  |  |  |  |  |  | 975.2 | 00 | 1002.8 | 958.9 |
| Sep | -42.8 | 00 | -18.0 | -54.8 |  |  |  |  |  |  |  | 985.5 | 00 | 1008.5 | 958.2 |
| Oct | -29.1 | 00 | -16.5 | -40.8 |  |  |  |  |  |  |  | 976.9 | 00 | 995.6 | 955.7 |
| Nov | -19.1 | 00 | -8.0 | -39.1 |  |  |  |  |  |  |  | 970.5 | 00 | 988.6 | 945.2 |
| Dec | -9.2 | 00 | -3.0 | -21.5 | 3.6 | 09 | 214 | 1.5 | 0.42 | 188 | 11 | 981.3 | 00 | 994.3 | 967.3 |
| MEAN | -28.6 |  |  |  |  |  |  |  |  |  |  | 978.0 |  |  |  |
| Elaine (8915) |  |  |  | 83.13S |  |  |  | 174.17E |  |  |  | 60 M |  |  |  |  |
| Jan | -8.9 | 00 | 0.9 | -19.6 | 3.1 | 17 | 107 | 1.1 | 0.36 | 119 | 14 | 988.0 | 00 | 995.6 | 977.8 |
| Feb | -11.9 | 00 | 0.4 | -29.5 | 3.9 | 00 | 131 | 2.9 | 0.73 | 116 | 24 | 984.2 | 00 | 998.3 | 958.0 |
| Mar | -24.3 | 00 | -8.6 | -43.0 | 4.1 | 00 | 150 | 2.7 | 0.65 | 125 | 16 | 986.4 | 00 | 1003.9 | 971.5 |
| Apr | -34.3 | 00 | -10.9 | -52.0 | 4.2 | 02 | 140 | 2.9 | 0.69 | 116 | 22 | 978.6 | 00 | 1000.3 | 948.3 |
| May | -31.1 | 00 | -2.8 | -50.2 | 5.2 | 00 | 142 | 2.8 | 0.54 | 116 | 25 | 986.6 | 00 | 1008.7 | 959.2 |
| Jun | -20.2 | 00 | -6.8 | -42.0 | 6.5 | 00 | 141 | 5.2 | 0.80 | 116 | 25 | 981.6 | 00 | 997.4 | 959.3 |
| Jul | -32.4 | 00 | -14.5 | -50.8 | 4.7 | 00 | 151 | 3.3 | 0.69 | 120 | 19 | 983.7 | 00 | 1002.1 | 964.9 |
| Aug | -35.6 | 00 | -14.2 | -58.6 | 3.4 | 00 | 166 | 2.4 | 0.71 | 119 | 21 | 982.5 | 00 | 1011.1 | 967.1 |
| Sep | -39.3 | 00 | -7.5 | -61.5 | 1.4 | 00 | 159 | 0.8 | 0.58 | 191 | 17 | 991.4 | 00 | 1014.7 | 972.1 |
| Oct | -25.6 | 00 | -6.8 | -42.8 | 4.6 | 00 | 138 | 3.6 | 0.78 | 119 | 19 | 984.9 | 00 | 999.0 | 967.7 |
| Nov | -14.3 | 00 | -2.9 | -37.1 | 4.6 | 00 | 137 | 2.9 | 0.63 | 116 | 23 | 977.4 | 00 | 994.4 | 958.8 |
| Dec | -6.7 | 00 | 1.0 | -13.5 | 2.0 | 00 | 142 | 0.2 | 0.13 | 184 | 10 | 984.0 | 00 | 997.6 | 969.2 |
| MEAN | -23.7 |  |  |  | 4.0 |  | 142 | 2.5 | 0.61 |  |  | 984.1 |  |  |  |
| Lettau (8908) |  |  |  | 82.52S |  |  |  | 174.45W |  |  |  | 55 M |  |  |  |  |
| Jan | -6.3 | 61 | 0.0 | -15.9 | 3.2 | 61 | 174 | 1.6 | 0.50 | 338 | 9 | 986.3 | 61 | 995.0 | 975.8 |
| Feb | -16.2 | 34 | -6.4 | -29.6 | 3.6 | 34 | 153 | 2.3 | 0.62 | 150 | 14 | 982.4 | 34 | 994.7 | 970.5 |
| Mar | -28.7 | 00 | -13.5 | -44.5 | 4.2 | 00 | 161 | 2.9 | 0.69 | 127 | 13 | 981.7 | 00 | 1001.5 | 967.5 |
| Apr | -40.6 | 00 | -13.0 | -57.0 | 3.1 | 00 | 158 | 1.8 | 0.56 | 132 | 17 | 974.3 | 00 | 997.2 | 938.8 |
| May | -34.3 | 00 | -8.0 | -51.1 | 4.9 | 00 | 159 | 2.2 | 0.44 | 136 | 18 | 982.1 | 00 | 1002.2 | 952.8 |
| Jun |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Jul |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Aug |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sep |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Oct | -31.6 | 52 | -11.9 | -40.5 | 2.8 | 61 | 173 | 1.6 | 0.56 | 132 | 17 | 985.2 | 51 | 996.4 | 973.1 |
| Nov | -18.2 | 53 | -6.5 | -38.9 | 6.5 | 55 | 147 | 5.4 | 0.83 | 132 | 19 | 970.5 | 53 | 990.8 | 947.1 |
| Dec | -7.1 | 62 | -1.0 | -12.5 | 2.7 | 62 | 172 | 1.6 | 0.58 | 167 | 8 | 982.2 | 62 | 995.6 | 969.6 |

|  | Mean | % of |  |  | Mean | % of |  |  |  |  |  | Mean | % of |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Air | Mon | Max Air | Min Air | Wind | Mon |  |  |  | Max |  | Air | Mon | Max Air | Min Air |
|  | Temp | Data | Temp | Temp | Speed | Data | Result |  |  | Wind |  | Press | Data | Press | Press |
| Mon | (C) | Abs | (C) | (C) | (m/s) | Abs | Wind (dir | vv) | Con | (dir | vv) | (mb) | Abs | (mb) | (mb) |
| Larsen Ice Shelf (8926) |  |  |  |  | 66.95S |  |  |  | 60.91W |  |  |  | 17 M |  |  |  |  |
| Jan | -1.0 | 03 | 5.5 | -6.4 | 2.6 | 01 | 146 | 0.8 | 0.31 | 182 | 12 | 981.0 | 01 | 996.2 | 960.5 |
| Feb | -2.1 | 00 | 5.2 | -12.9 | 3.2 | 00 | 179 | 0.3 | 0.09 | 209 | 12 | 986.3 | 00 | 1005.2 | 968.1 |
| Mar | -9.6 | 01 | 3.5 | -25.9 | 2.0 | 01 | 183 | 0.4 | 0.21 | 340 | 9 | 984.2 | 01 | 1001.3 | 970.3 |
| Apr | -12.7 | 00 | 4.9 | -27.5 | 2.7 | 00 | 250 | 0.9 | 0.32 | 258 | 15 | 981.8 | 00 | 1006.4 | 960.1 |
| May | -16.4 | 00 | 4.9 | -35.9 | 1.1 | 00 | 246 | 0.5 | 0.47 | 113 | 10 | 990.6 | 00 | 1013.0 | 962.6 |
| Jun | -12.6 | 35 | 3.8 | -36.2 | 1.5 | 36 | 238 | 0.6 | 0.43 | 209 | 12 | 991.1 | 35 | 1005.7 | 973.4 |
| Jul |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Aug |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sep | -18.3 | 06 | 2.5 | -44.9 | 2.7 | 47 | 189 | 2.1 | 0.76 | 223 | 10 | 983.8 | 06 | 1004.8 | 959.5 |
| Oct | -15.5 | 01 | 2.2 | -32.8 |  |  |  |  |  |  |  | 977.4 | 01 | 999.8 | 953.7 |
| Nov | -5.8 | 05 | 6.5 | -23.6 | 4.5 | 05 | 208 | 1.4 | 0.31 | 180 | 13 | 975.0 | 05 | 1007.0 | 945.9 |
| Dec | -2.3 | 04 | 5.5 | -17.5 | 3.7 | 04 | 154 | 1.2 | 0.33 | 147 | 12 | 977.3 | 04 | 1000.8 | 960.8 |
| Butler Island (8902) |  |  |  |  | 72.21S |  |  |  | 60.17W |  |  |  | 91 M |  |  |  |  |
| Jan | -2.3 | 06 | 3.9 | -10.0 | 5.1 | 06 | 195 | 1.7 | 0.34 | 187 | 23 | 986.2 | 06 | 1000.5 | 967.1 |
| Feb | -4.9 | 04 | 10.5 | -17.1 | 5.8 | 03 | 200 | 3.5 | 0.61 | 178 | 17 | 989.7 | 03 | 1007.1 | 973.7 |
| Mar | -13.6 | 00 | 3.0 | -23.5 | 5.5 | 00 | 199 | 4.0 | 0.73 | 195 | 16 | 988.9 | 01 | 1000.1 | 976.2 |
| Apr | -18.4 | 04 | 5.9 | -30.4 | 5.1 | 04 | 213 | 2.7 | 0.54 | 271 | 30 | 983.3 | 04 | 1003.1 | 964.4 |
| May | -14.0 | 03 | 6.6 | -33.9 | 4.3 | 03 | 216 | 2.3 | 0.54 | 172 | 17 | 991.6 | 03 | 1015.3 | 970.6 |
| Jun | -16.8 | 00 | 4.6 | -30.4 | 5.2 | 04 | 207 | 3.2 | 0.61 | 181 | 31 | 992.3 | 00 | 1008.7 | 966.7 |
| Jul | -25.0 | 00 | -11.4 | -32.8 | 3.4 | 00 | 192 | 2.8 | 0.83 | 184 | 14 | 984.3 | 00 | 1009.4 | 967.2 |
| Aug | -25.4 | 01 | 1.2 | -33.9 | 3.2 | 03 | 199 | 2.0 | 0.61 | 189 | 19 | 986.0 | 01 | 1004.0 | 969.2 |
| Sep | -22.8 | 00 | 1.6 | -36.8 | 6.5 | 72 | 185 | 6.3 | 0.98 | 098 | 28 | 987.6 | 00 | 1007.5 | 962.5 |
| Oct | -19.6 | 00 | 0.4 | -31.2 |  |  |  |  |  |  |  | 983.6 | 00 | 1008.3 | 959.1 |
| Nov | -6.4 | 01 | 6.6 | -21.2 | 5.8 | 01 | 197 | 2.6 | 0.45 | 195 | 25 | 980.2 | 01 | 1002.4 | 958.8 |
| Dec | -3.0 | 09 | 6.6 | -13.0 | 5.6 | 07 | 192 | 3.6 | 0.64 | 198 | 18 | 981.6 | 07 | 1001.8 | 960.1 |
| MEAN | -14.4 |  |  |  |  |  |  |  |  |  |  | 986.3 |  |  |  |
| Uranus Glacier (8920) |  |  |  |  | 71.43S |  |  |  | 68.93W |  |  |  | 780 M |  |  |  |  |
| Jan | -1.4 | 30 | 7.0 | -9.5 | 4.0 | 27 | 010 | 3.3 | 0.83 | 007 | 19 | 900.2 | 27 | 912.5 | 891.0 |
| Feb | -3.6 | 14 | 3.8 | -14.5 | 6.5 | 13 | 355 | 5.8 | 0.89 | 337 | 23 | 904.3 | 13 | 922.8 | 888.3 |
| Mar | -7.5 | 00 | 3.5 | -17.9 | 4.4 | 00 | 002 | 3.9 | 0.88 | 015 | 19 | 901.1 | 00 | 913.3 | 888.7 |
| Apr | -9.3 | 00 | 0.2 | -23.9 | 7.8 | 00 | 358 | 7.2 | 0.92 | 354 | 29 | 894.9 | 00 | 924.8 | 870.5 |
| May | -13.2 | 00 | -0.9 | -37.4 | 9.3 | 27 | 003 | 8.8 | 0.94 | 023 | 23 | 910.9 | 00 | 931.6 | 886.7 |
| Jun |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Jul |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Aug |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sep | -13.6 | 04 | -0.5 | -30.4 | 6.3 | 04 | 009 | 5.3 | 0.85 | 029 | 23 | 897.8 | 04 | 926.9 | 875.8 |
| Oct | -12.6 | 00 | -0.1 | -30.9 | 4.5 | 00 | 014 | 4.0 | 0.89 | 039 | 21 | 891.8 | 00 | 916.7 | 865.5 |
| Nov | -8.0 | 03 | 1.9 | -27.2 | 4.9 | 03 | 009 | 4.1 | 0.84 | 343 | 20 | 894.9 | 03 | 923.4 | 871.8 |
| Dec | -4.3 | 38 | 4.0 | -13.0 | 4.6 | 34 | 005 | 3.8 | 0.82 | 030 | 18 | 894.0 | 34 | 916.0 | 883.9 |

|  | Mean | % of |  |  | Mean | % of |  |  |  |  |  | Mean | % of |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Air | Mon | Max Air | Min Air | Wind | Mon |  |  |  | Max |  | Air | Mon | Max Air | Min Air |
|  | Temp | Data | Temp | Temp | Speed | Data | Result |  |  | Wind |  | Press | Data | Press | Press |
| Mon | (C) | Abs | (C) | (C) | (m/s) | Abs | Wind (dir | vv) | Con | (dir | vv) | (mb) | Abs | (mb) | (mb) |
| Limbert (8925) |  |  |  | 75.42S |  |  |  | 59.95W |  |  |  | 40 M |  |  |  |  |
| Jan | -4.4 | 01 | 3.1 | -17.9 | 4.2 | 00 | 150 | 1.6 | 0.38 | 236 | 14 | 983.7 | 00 | 1000.1 | 971.2 |
| Feb | -12.6 | 01 | 3.1 | -27.5 | 5.0 | 00 | 200 | 3.5 | 0.70 | 195 | 14 | 987.3 | 00 | 1004.2 | 973.4 |
| Mar | -22.0 | 01 | -7.9 | -35.8 | 5.2 | 04 | 196 | 2.9 | 0.56 | 067 | 15 | 987.4 | 01 | 998.2 | 975.0 |
| Apr | -29.0 | 00 | -0.8 | -42.9 | 5.6 | 34 | 203 | 4.7 | 0.85 | 209 | 26 | 980.6 | 00 | 1002.0 | 964.6 |
| May | -26.6 | 00 | -5.2 | -50.4 |  |  |  |  |  |  |  | 987.8 | 00 | 1012.9 | 969.4 |
| Jun | -26.9 | 01 | -7.2 | -43.5 |  |  |  |  |  |  |  | 989.6 | 01 | 1004.1 | 960.6 |
| Jul | -31.1 | 00 | -11.2 | -44.0 |  |  |  |  |  |  |  | 982.3 | 00 | 1007.7 | 963.4 |
| Aug | -33.5 | 01 | -3.5 | -46.0 |  |  |  |  |  |  |  | 984.1 | 01 | 1003.1 | 967.0 |
| Sep | -35.8 | 60 | -16.2 | -50.4 |  |  |  |  |  |  |  | 988.2 | 60 | 1000.9 | 980.9 |
| Oct | -24.7 | 00 | -12.5 | -35.5 |  |  |  |  |  |  |  | 981.4 | 00 | 1007.3 | 955.1 |
| Nov | -17.7 | 52 | -9.6 | -33.0 | 6.6 | 52 | 215 | 5.5 | 0.82 | 216 | 23 | 976.7 | 52 | 993.2 | 957.8 |
| Racer Rock (8947) |  |  |  |  | 64.07S |  |  |  | 61.61W |  |  |  | 17 M |  |  |  |  |
| Jan | 1.1 | 09 | 5.2 | -5.9 | 4.9 | 08 | 086 | 2.4 | 0.49 | 210 | 19 | 982.9 | 08 | 999.1 | 970.5 |
| Feb | 1.9 | 19 | 6.6 | -7.8 | 5.0 | 17 | 153 | 1.1 | 0.21 | 220 | 23 | 990.5 | 17 | 1009.2 | 971.0 |
| Mar | -0.7 | 15 | 3.6 | -7.0 | 4.5 | 15 | 064 | 1.5 | 0.33 | 073 | 23 | 987.2 | 15 | 1006.1 | 966.7 |
| Apr | -1.9 | 12 | 1.9 | -13.9 | 7.8 | 12 | 316 | 3.2 | 0.41 | 306 | 28 | 989.5 | 12 | 1015.9 | 962.2 |
| May | -3.0 | 18 | 2.5 | -15.2 | 4.9 | 18 | 211 | 1.8 | 0.37 | 212 | 27 | 997.2 | 18 | 1015.6 | 970.6 |
| Jun |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Jul |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Aug |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sep | -5.8 | 49 | 3.0 | -22.9 | 6.1 | 49 | 031 | 2.2 | 0.36 | 293 | 22 | 989.9 | 49 | 1010.9 | 961.4 |
| Oct | -3.4 | 11 | 3.2 | -22.2 | 6.6 | 11 | 059 | 4.0 | 0.61 | 295 | 24 | 977.8 | 11 | 1008.5 | 953.7 |
| Nov | -1.5 | 19 | 3.8 | -15.9 | 6.3 | 19 | 023 | 1.0 | 0.16 | 331 | 26 | 981.1 | 19 | 1016.1 | 952.0 |
| Dec | 0.0 | 17 | 4.6 | -15.1 | 5.7 | 17 | 109 | 1.4 | 0.25 | 208 | 21 | 980.3 | 17 | 1005.0 | 961.9 |
| Bonaparte Point (8923) |  |  |  |  | 64.78S |  |  |  | 64.07W |  |  |  | 8 M |  |  |  |  |
| Jan | 2.3 | 09 | 9.6 | -1.6 | 2.6 | 08 | 081 | 1.1 | 0.43 | 270 | 16 | 983.1 | 08 | 998.1 | 968.4 |
| Feb | 1.3 | 07 | 7.0 | -2.9 | 3.5 | 21 | 084 | 2.0 | 0.56 | 181 | 19 | 985.8 | 07 | 1002.8 | 963.6 |
| Mar | -0.1 | 16 | 7.0 | -3.6 | 3.9 | 18 | 129 | 0.8 | 0.22 | 236 | 22 | 980.5 | 16 | 997.1 | 958.3 |
| Apr | -0.9 | 13 | 3.1 | -4.8 | 6.1 | 14 | 078 | 2.7 | 0.44 | 073 | 26 | 981.4 | 13 | 1011.2 | 951.7 |
| May |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Jun | -2.5 | 42 | 3.9 | -8.2 | 3.6 | 42 | 075 | 1.3 | 0.37 | 284 | 17 | 985.3 | 42 | 1006.7 | 955.7 |
| Jul | -4.1 | 24 | 0.8 | -10.0 | 4.3 | 24 | 147 | 2.0 | 0.47 | 180 | 22 | 966.6 | 24 | 1001.2 | 940.4 |
| Aug |  |  |  |  | 4.2 | 34 | 096 | 1.3 | 0.31 | 257 | 20 | 972.7 | 34 | 1002.1 | 948.7 |
| Sep |  |  |  |  | 6.2 | 28 | 160 | 1.6 | 0.25 | 233 | 24 | 978.1 | 28 | 1009.5 | 950.4 |
| Oct | -3.3 | 58 | 1.8 | -10.9 | 4.3 | 29 | 120 | 1.5 | 0.34 | 197 | 18 | 966.6 | 29 | 996.0 | 939.8 |
| Nov | -1.4 | 14 | 3.8 | -7.2 | 4.0 | 14 | 098 | 2.3 | 0.58 | 164 | 20 | 969.3 | 14 | 1003.5 | 941.5 |
| Dec | -1.2 | 07 | 2.8 | -5.9 | 3.0 | 07 | 087 | 1.6 | 0.53 | 049 | 17 | 968.1 | 07 | 992.5 | 949.1 |

|  | Mean | % of |  |  | Mean | % of |  |  |  |  |  | Mean | % of |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Air | Mon | Max Air | Min Air | Wind | Mon |  |  |  | Max |  | Air | Mon | Max Air | Min Air |
|  | Temp | Data | Temp | Temp | Speed | Data | Result |  |  | Wind |  | Press | Data | Press | Press |
| Mon | (C) | Abs | (C) | (C) | (m/s) | Abs | Wind (dir | vv) | Con | (dir | vv) | (mb) | Abs | (mb) | (mb) |
| Ski-Hi (8917) |  |  |  | 74.98S |  |  |  | 70.77W |  |  |  | 1395 M |  |  |  |  |
| Jan | -4.0 | 65 | 5.5 | -11.4 | 4.1 | 65 | 043 | 2.3 | 0.55 | 360 | 19 | 828.7 | 65 | 842.1 | 821.8 |
| Feb |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Mar |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Apr |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| May |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Jun |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Jul | -32.2 | 65 | -21.9 | -40.9 | 5.2 | 68 | 009 | 4.7 | 0.90 | 002 | 16 | 814.6 | 65 | 830.2 | 799.3 |
| Aug | -35.9 | 56 | -10.8 | -50.8 | 3.6 | 56 | 009 | 3.3 | 0.91 | 012 | 37 | 820.4 | 56 | 834.7 | 804.8 |
| Sep | -35.9 | 56 | -23.5 | -47.5 | 3.4 | 64 | 037 | 1.8 | 0.54 | 360 | 11 | 820.6 | 56 | 845.8 | 810.3 |
| Santa Claus Is (21364) |  |  |  |  | 64.96S |  |  |  | 65.67W |  |  |  | 25 M |  |  |  |  |
| Feb | 1.5 | 05 | 3.9 | -1.2 | 5.5 | 05 | 158 | 1.9 | 0.35 | 132 | 17 | 991.0 | 05 | 1007.1 | 966.9 |
| Mar | 0.5 | 05 | 3.0 | -2.2 | 4.6 | 05 | 300 | 1.7 | 0.37 | 299 | 19 | 985.6 | 05 | 1003.5 | 963.1 |
| Apr | 0.0 | 05 | 3.0 | -5.4 | 6.9 | 05 | 198 | 3.6 | 0.53 | 198 | 24 | 988.0 | 05 | 1019.1 | 959.0 |
| May | -2.5 | 06 | 1.1 | -9.0 | 7.2 | 07 | 132 | 5.9 | 0.82 | 127 | 21 | 997.6 | 06 | 1016.0 | 968.3 |
| Jun | -2.0 | 07 | 0.0 | -7.4 | 4.0 | 41 | 299 | 3.0 | 0.75 | 303 | 22 | 991.0 | 07 | 1014.0 | 960.8 |
| Jul | -3.6 | 14 | 0.0 | -7.6 | 8.4 | 14 | 300 | 6.3 | 0.76 | 307 | 25 | 972.0 | 14 | 1005.5 | 947.7 |
| Aug | -5.0 | 09 | -0.8 | -15.0 | 6.6 | 09 | 299 | 3.8 | 0.58 | 309 | 20 | 978.8 | 09 | 1007.1 | 955.1 |
| Sep | -6.3 | 06 | 0.8 | -16.8 | 6.0 | 06 | 265 | 1.2 | 0.20 | 313 | 18 | 983.8 | 06 | 1016.7 | 953.9 |
| Oct | -3.0 | 06 | 0.9 | -12.0 | 6.1 | 06 | 305 | 3.3 | 0.54 | 309 | 17 | 974.8 | 06 | 1003.4 | 948.4 |
| Nov | -1.8 | 09 | 1.5 | -5.5 | 4.6 | 09 | 252 | 0.8 | 0.17 | 319 | 15 | 979.2 | 09 | 1012.3 | 949.5 |
| Dec | -0.4 | 05 | 2.2 | -3.1 | 3.9 | 05 | 277 | 0.6 | 0.16 | 347 | 12 | 978.5 | 05 | 1002.8 | 959.6 |
| Clean Air (8987) |  |  |  |  | 90.00S |  |  |  |  |  |  |  | 2835 M |  |  |  |  |
| Jan | -26.2 | 01 | -19.1 | -37.0 | 3.3 | 02 | 003 | 2.9 | 0.86 | 032 | 9 | 687.1 | 01 | 698.5 | 680.3 |
| Feb | -38.0 | 01 | -26.2 | -52.5 | 3.1 | 02 | 041 | 2.4 | 0.76 | 018 | 9 | 684.6 | 01 | 694.6 | 675.3 |
| Mar | -53.4 | 00 | -42.4 | -63.9 | 3.6 | 00 | 042 | 3.1 | 0.87 | 022 | 9 | 678.9 | 00 | 687.2 | 664.7 |
| Apr | -63.1 | 00 | -47.0 | -74.2 | 3.3 | 00 | 045 | 2.6 | 0.80 | 015 | 11 | 668.9 | 00 | 681.6 | 657.9 |
| May | -59.3 | 00 | -36.4 | -76.1 | 4.4 | 00 | 042 | 2.9 | 0.67 | 360 | 14 | 675.6 | 01 | 685.9 | 653.6 |
| Jun | -56.4 | 00 | -37.9 | -73.4 | 3.6 | 00 | 046 | 2.5 | 0.69 | 360 | 11 | 680.6 | 02 | 689.5 | 665.9 |
| Jul | -60.1 | 00 | -45.4 | -76.1 | 4.6 | 00 | 038 | 4.0 | 0.87 | 009 | 15 | 674.3 | 00 | 684.2 | 659.5 |
| Aug | -63.9 | 00 | -47.4 | -74.1 | 3.6 | 01 | 055 | 3.0 | 0.83 | 002 | 12 | 673.1 | 00 | 683.7 | 659.8 |
| Sep | -63.7 | 02 | -45.8 | -77.1 | 3.4 | 03 | 052 | 2.9 | 0.87 | 016 | 10 | 675.7 | 02 | 688.1 | 663.5 |
| Oct | -54.3 | 00 | -38.8 | -66.5 | 3.4 | 00 | 051 | 2.9 | 0.84 | 007 | 11 | 676.7 | 03 | 696.3 | 662.4 |
| Nov | -36.9 | 02 | -23.6 | -48.1 | 4.9 | 03 | 019 | 4.6 | 0.94 | 026 | 14 | 678.2 | 02 | 691.7 | 665.5 |
| Dec | -26.8 | 00 | -19.5 | -32.8 | 2.6 | 00 | 021 | 2.1 | 0.80 | 009 | 10 | 683.8 | 00 | 693.1 | 670.3 |
| MEAN | -50.2 |  |  |  | 3.7 |  | 037 | 2.9 | 0.81 |  |  | 677.5 |  |  |  |

|  | Mean | % of |  |  | Mean | % of |  |  |  |  |  | Mean | % of |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Air | Mon | Max Air | Min Air | Wind | Mon |  |  |  | Max |  | Air | Mon | Max Air | Min Air |
|  | Temp | Data | Temp | Temp | Speed | Data | Result |  |  | Wind |  | Press | Data | Press | Press |
| Mon | (C) | Abs | (C) | (C) | (m/s) | Abs | Wind (dir | vv) | Con | (dir | vv) | (mb) | Abs | (mb) | (mb) |
| Henry (8985) |  |  |  | 89.01S |  |  |  | 1.03W |  |  |  | 2755 M |  |  |  |  |
| Jan | -25.2 | 00 | -14.8 | -35.6 | 3.8 | 00 | 022 | 3.1 | 0.81 | 005 | 10 | 696.6 | 00 | 708.4 | 689.5 |
| Feb | -36.1 | 00 | -23.9 | -47.8 | 4.4 | 00 | 047 | 3.9 | 0.89 | 023 | 9 | 693.8 | 00 | 705.2 | 683.8 |
| Mar | -51.5 | 00 | -39.2 | -60.6 | 4.6 | 00 | 045 | 4.3 | 0.95 | 035 | 11 | 688.7 | 00 | 696.3 | 675.1 |
| Apr | -60.2 | 00 | -44.9 | -71.5 | 4.7 | 00 | 050 | 4.4 | 0.92 | 026 | 10 | 678.7 | 00 | 691.1 | 667.7 |
| May | -56.8 | 00 | -34.1 | -74.9 | 5.8 | 00 | 047 | 4.6 | 0.80 | 014 | 13 | 685.5 | 00 | 697.7 | 662.8 |
| Jun | -54.8 | 00 | -39.2 | -68.8 | 5.1 | 00 | 057 | 4.3 | 0.83 | 045 | 11 | 690.1 | 00 | 699.0 | 674.6 |
| Jul | -57.4 | 00 | -39.1 | -69.2 | 6.5 | 00 | 043 | 6.2 | 0.95 | 025 | 15 | 684.2 | 00 | 694.2 | 667.8 |
| Aug | -60.4 | 00 | -45.8 | -70.0 | 6.6 | 16 | 047 | 6.2 | 0.95 | 035 | 12 | 682.8 | 00 | 693.9 | 668.8 |
| Sep |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Oct | -46.8 | 50 | -34.5 | -53.6 | 6.2 | 50 | 047 | 5.9 | 0.95 | 040 | 12 | 683.6 | 50 | 700.1 | 671.5 |
| Nov | -35.0 | 00 | -22.9 | -45.9 | 5.1 | 00 | 032 | 4.9 | 0.97 | 016 | 14 | 688.0 | 00 | 702.4 | 674.6 |
| Dec | -25.0 | 00 | -19.6 | -29.2 | 3.3 | 00 | 036 | 2.8 | 0.84 | 033 | 9 | 693.0 | 00 | 703.4 | 679.7 |
| MEAN |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Nico (8924) |  |  |  | 89.00S |  |  |  | 89.67E |  |  |  | 2935 M |  |  |  |  |
| Jan | -26.5 | 00 | -18.5 | -37.5 | 3.8 | 00 | 260 | 3.0 | 0.79 | 220 | 9 | 676.9 | 00 | 688.1 | 670.7 |
| Feb | -37.3 | 00 | -27.2 | -49.8 | 3.6 | 00 | 302 | 2.9 | 0.80 | 237 | 8 | 674.1 | 00 | 684.6 | 665.0 |
| Mar | -52.2 | 00 | -41.6 | -64.1 | 3.2 | 00 | 302 | 2.8 | 0.88 | 285 | 9 | 668.4 | 00 | 677.1 | 654.2 |
| Apr | -61.2 | 00 | -47.0 | -73.5 | 2.9 | 00 | 306 | 2.3 | 0.81 | 348 | 11 | 658.0 | 00 | 671.7 | 647.3 |
| May | -58.3 | 00 | -38.5 | -76.2 | 4.5 | 02 | 303 | 3.1 | 0.70 | 234 | 14 | 665.0 | 00 | 674.6 | 642.1 |
| Jun | -56.2 | 00 | -36.9 | -72.4 | 3.2 | 00 | 314 | 2.4 | 0.74 | 355 | 11 | 669.4 | 00 | 677.9 | 654.3 |
| Jul | -58.2 | 00 | -42.8 | -75.1 | 4.0 | 00 | 292 | 3.7 | 0.90 | 274 | 15 | 663.9 | 00 | 673.4 | 650.0 |
| Aug | -61.7 | 00 | -46.2 | -75.4 | 4.1 | 04 | 309 | 3.7 | 0.89 | 260 | 10 | 662.4 | 00 | 673.0 | 650.2 |
| Sep | -61.8 | 00 | -46.9 | -75.1 | 3.9 | 08 | 304 | 3.6 | 0.93 | 291 | 10 | 664.8 | 00 | 677.2 | 652.6 |
| Oct | -52.4 | 00 | -38.5 | -65.9 | 3.7 | 12 | 310 | 3.2 | 0.86 | 261 | 9 | 666.0 | 00 | 685.5 | 651.8 |
| Nov | -36.4 | 00 | -24.6 | -48.2 | 4.5 | 00 | 285 | 4.1 | 0.92 | 270 | 12 | 668.4 | 00 | 682.2 | 655.7 |
| Dec | -26.3 | 00 | -19.6 | -30.9 | 2.0 | 00 | 282 | 1.5 | 0.76 | 253 | 7 | 673.5 | 00 | 682.9 | 660.2 |
| MEAN | -49.0 |  |  |  | 3.6 |  | 298 | 2.9 | 0.83 |  |  | 667.6 |  |  |  |
| Relay Station (8918) |  |  |  |  | 74.02S |  |  |  | 43.06E |  |  |  | 3353 M |  |  |  |  |
| Jan | -31.9 | 00 | -20.6 | -43.4 | 6.0 | 00 | 112 | 5.6 | 0.94 | 106 | 15 | 643.2 | 00 | 652.1 | 634.9 |
| Feb | -39.1 | 01 | -25.2 | -54.5 | 6.8 | 01 | 118 | 6.5 | 0.96 | 130 | 17 | 640.1 | 01 | 648.9 | 633.3 |
| Mar | -50.6 | 04 | -35.4 | -62.6 | 6.3 | 04 | 129 | 6.1 | 0.97 | 089 | 15 | 636.7 | 04 | 647.5 | 624.3 |
| Apr | -58.9 | 00 | -44.8 | -70.6 | 7.1 | 00 | 137 | 6.2 | 0.87 | 088 | 20 | 623.8 | 00 | 638.1 | 600.9 |
| May | -54.9 | 00 | -33.6 | -65.9 | 7.6 | 00 | 134 | 6.9 | 0.91 | 135 | 20 | 635.6 | 00 | 649.1 | 624.6 |
| Jun | -55.6 | 00 | -38.1 | -65.0 | 7.6 | 00 | 123 | 7.2 | 0.94 | 079 | 17 | 633.6 | 00 | 648.3 | 621.2 |
| Jul | -58.5 | 00 | -35.8 | -71.4 | 8.0 | 00 | 123 | 6.9 | 0.86 | 068 | 20 | 630.5 | 00 | 644.0 | 616.8 |
| Aug | -58.3 | 00 | -44.6 | -67.8 | 7.7 | 00 | 130 | 7.1 | 0.93 | 119 | 17 | 629.0 | 00 | 644.7 | 614.9 |
| Sep | -55.7 | 00 | -40.2 | -71.4 | 7.1 | 00 | 117 | 6.4 | 0.90 | 112 | 19 | 630.7 | 00 | 638.5 | 614.1 |
| Oct | -52.1 | 01 | -36.9 | -67.9 | 7.3 | 01 | 120 | 6.9 | 0.94 | 088 | 19 | 630.7 | 01 | 645.1 | 617.2 |
| Nov | -36.7 | 01 | -24.8 | -55.5 | 8.7 | 01 | 099 | 8.3 | 0.95 | 079 | 20 | 636.3 | 01 | 645.7 | 627.7 |
| Dec | -31.7 | 00 | -19.2 | -42.1 | 6.2 | 00 | 102 | 5.7 | 0.92 | 126 | 18 | 640.1 | 00 | 653.7 | 626.4 |
| MEAN | -48.7 |  |  |  | 7.2 |  | 120 | 6.5 | 0.92 |  |  | 634.2 |  |  |  |

|  | Mean | % of |  |  | Mean | % of |  |  |  |  |  | Mean | % of |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Air | Mon | Max Air | Min Air | Wind | Mon |  |  |  | Max |  | Air | Mon | Max Air | Min Air |
|  | Temp | Data | Temp | Temp | Speed | Data | Result |  |  | Wind |  | Press | Data | Press | Press |
| Mon | (C) | Abs | (C) | (C) | (m/s) | Abs | Wind (dir | vv) | Con | (dir | vv) | (mb) | Abs | (mb) | (mb) |
| Dome Fuji (8904) |  |  |  |  | 77.31S |  |  |  | 39.70E |  |  |  | 3810 M |  |  |  |  |
| Jan | -30.4 | 54 | -16.6 | -43.3 | 3.1 | 54 | 331 | 1.5 | 0.50 | 305 | 12 | 607.4 | 54 | 615.4 | 597.3 |
| Feb |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Mar | -58.0 | 34 | -43.2 | -70.7 | 3.6 | 34 | 293 | 1.5 | 0.41 | 286 | 25 | 596.2 | 34 | 603.1 | 589.0 |
| Apr | -65.0 | 11 | -45.2 | -75.1 | 10.7 | 28 | 035 | 4.3 | 0.40 | 094 | 29 | 586.2 | 08 | 599.1 | 570.4 |
| May | -59.9 | 26 | -38.6 | -71.7 | 5.6 | 26 | 309 | 1.1 | 0.19 | 282 | 45 | 597.1 | 26 | 607.6 | 585.9 |
| Jun | -60.8 | 14 | -44.5 | -72.7 | 3.9 | 14 | 280 | 1.0 | 0.26 | 323 | 15 | 596.1 | 14 | 609.7 | 586.1 |
| Jul | -61.4 | 46 | -41.8 | -74.2 | 13.8 | 46 | 018 | 2.4 | 0.17 | 296 | 36 | 594.4 | 46 | 606.3 | 581.5 |

4.2. Three Hourly Data Summaries

After the data are received from Service ARGOS, ten minute interval data are created for each AWS unit. The data are calibrated for the individual station instruments, but no other corrections are made. This data set is created for those users who need fairly current information. These data are available by anonymous FTP (see Section 8).

The 10 minute data set for each AWS unit for the month is scanned to pick out the nearest observation within forty minutes of the UTC hours 00, 03, 06, 09, 12, 15, 18, and 21 to produce the three hourly data set. If valid data are not available within forty minutes of the three hourly time interval, then the entry is left blank to indicate missing data. The means, standard deviations, resultant wind speed and direction, the distribution of temperature, and wind speed with wind direction are determined from the three hourly observations and are presented as a monthly summary at the bottom of each page. A wind direction value of zero indicates a wind speed less than 0.50 m s-1.  North is indicated by a value of 360 degrees. The maximum and minimum values are taken from the complete 10 minute data set, not the three hourly data set. The appropriate monthly data from the three hourly data set are used for the monthly summaries presented in Section 4.1. In the presence of sunlight the air temperatures are questionable if the wind speed is less than 1 m s-1. These summaries are available by anonymous FTP (see Section 8).  If you are unable to access the Internet, we will send the information either on diskettes or paper. Please contact us for further information (the address is at end of Section 8).

5.  AWS CALIBRATION

- 5.1. Temperature

The external and internal temperatures are calibrated using a 1000 ohm 0.05% resistor in place of the platinum resistance thermometers with 1000 ohms resistance at 0oC.  Because the other resistances in the temperature circuit are known only to 1%, the temperature calibration will vary from one electronic unit to another.  The correction factor determined from the calibration resistor is programmed into the read-only-memories for each unit.  After the correction factors have been programmed into the AWS, a calibration box with 0.1% resistors is used to check the temperature calibration.

- 5.2. Pressure

The atmospheric pressure transducer is a Parascientific model 215A Digiquartz pressure gauge.  The transducer frequency changes from a nominal 40 kHz at zero pressure to a nominal 36 kHz at 1000 hPa. The pressure resolution is about 0.05 hPa.

Paulin aneroid barometers calibrated against a mercury barometer of 10 mm bore are used to check the pressure gauge calibration.  Comparisons are made between AWS units, a Parascientific Model 760-16B accurate to +/- 0.1 hPa, and with the mercury barometers at Scott Base, Antarctica.  The calibrations should be within +/- 0.2 hPa.  Two mercury barometers have been purchased for use at McMurdo, Antarctica but are not yet available.

The reference vacuum on the older pressure transducers can degrade with time with a maximum observed 4 hPa shift to lower pressure after fifteen years. Recalibration of each pressure transducer would be desirable when each unit is serviced.

- 5.3. Wind direction and Speed

The Belfort model 123 aerovane measures wind direction and speed.  The aerovane rotates a potentiometer wiper, and the fraction of full scale of the potentiometer is measured.  The wind direction is checked by positioning the aerovane to the cardinal directions relative to the boom supporting the aerovane.  North or the potentiometer zero is towards the antenna on the boom and has a maximum dead zone of 3o.  During the field installation, the boom is usually aligned along the north-south line as determined from the sun's azimuth, longitude, and Greenwich Mean Time.  In some cases the 180o end of the boom may point in a direction other than south.  At Manuela site, the 180o end of the boom points up the glacier and a correction is added to the data during processing.  At Byrd site the wind is usually out of the north so the boom was rotated 120o and the correction added during the data processing.  The wind speed is determined from the aerovane tachometer voltage output as 0.0472 volt per meter per second.  The aerovane tachometers are spun at 1800 rpm with a load of 1071.5 ohms, and the output should be 9.20 +/-0.05 vdc.

Three additional wind sensors were used with AWS units for 1998.  These were the Vaisala anemometer model WAA-15, the R.M. Young wind monitor model 05103, and the Hydro-Tech WS-3 rotor anemometer.  The Vaisala WAA-15 and the Hydro-Tech WS-3 were used as backup sensors for measuring wind speed in the Adelie Coast area.  The WAA-15 is a 3-cup opto-electronic anemometer.  When rotating, the anemometer produces a pulsed output that is proportional to the wind speed.  Rated accuracy is +/- 2% up to 75 m/s.  The pulsed output was input into one of the digital counter channels for 5 seconds.  This resulted in a calibration value of .293 m/s/bit.  The Hydro-Tech WS-3 is a disk rotor, 3 in. high and 12 in. overall diameter, with radial cups, and the threshold sensitivity is 3 mph.  The anemometer utilizes a commercial dc tachometer generator.  Output is 0 to +5 vdc (and 0 to 1 ma) over the desired full scale wind speed of 85 m/s.  Accuracy is +/- 2%.

The R.M. Young monitor 05103 also used a 10,000 ohm potentiometer with a 3o dead zone so that the wind direction was recorded identically with the Belfort/Bendix aerovanes.  The wind speed was from the range of 0 to 1.0 volt full scale corresponding to 50 m/s.  Thus the calibration for wind speed was a nominal .195 m/s/bit for the R.M. Young with +/- 1% up to 50 m/s.

- 5.4. Relative Humidity

The Vaisala HMP-35A/45A humidity sensor output voltage varies linearly with relative humidity (U).  The sensor is calibrated by placing it over saturated salt solutions with known relative humidities at room temperature:  sodium chloride (U=75%), and lithium chloride (U=12%) are used.  In addition, a dry inert gas, forced past the sensor, gives a 0% U, and the sensor output can be zeroed.  Then, the gain setting can be set directly using a salt solution with a high relative humidity, such as sodium chloride.  The resolution of the humidity sensor is about 1% and the drift is 2 to 3% per year in the field.  The relative humidity data are not included on the summary pages but are included in the 3 hourly data sets.

- 5.5. Vertical Air Temperature Difference

Two junction thermocouples are used to measure the air temperature difference between 3 m and 0.5 m on the tower.  The output is about 78 microvolts for 1oC temperature difference between the junctions at 0.0oC, dropping to 60 microvolts at -80oC.  Zero output is adjusted to 0.4 volts, so that 0 to 1 volt corresponds to a -6oC to +9oC range of air temperature differences between 3 m and 0.5 m.  The resolution is 0.05oC.  Calibration of the individual systems is done by applying known voltages to the amplifier input. The vertical temperature difference data are not included on the summary pages but are included in the 3 hourly data sets.

- 6.  AWS OPERATIONS SUMMARY FOR 1998

- 6.1.  AWS Performance

Forty-two AWS units were installed at the start of 1998 and 45 were installed by the end of 1998.  Based on the installation months the AWS units delivered 76% of the temperature data, 77% of the pressure data and 62% of the wind data during 1998. Complete data sets were received from 9 AWS units and 26 AWS units operated for the installed period. Fifteen AWS units were not received for one month or more during the year or stopped during the year.  Many of the stations were not received during the winter months due to low battery voltage. Some exceptional periods of bad weather during the 1997-1998 austral summer prevented maintenance work including replacing batteries.

The wind system has the poorest performance. If the wind speed is zero or the wind direction is constant for extended periods (days to months) then the data is considered invalid. The reason for this behavior is not known but is believed to be due to the build up of frost on the wind system. This usually occurs in the winter season and at several AWS sites. The wind speed is most frequently zero when the wind direction is constant. Another problem with the wind system involves the tachometer for measuring wind speed. The brushes on the Belfort aerovane quickly wear down and fill the gaps between the contacts with brush material, shorting out the tachometer output. We have begun to install a new wind system manufactured by R.M. Young.

Site	Performance

D-10			New station electronics installed in January. The aerovane was "frozen" 					occasionally in April through June and August and September. The station 				transmitted erratically from the end of June through the middle of August. The 				station stopped transmitting from the middle of August to the middle of 					December. The station was rebooted and began to transmit again.

D-47			Station reinstalled 5 February. Temperature sensor not functioning. Station 				stopped transmitting 26 June due to low battery voltage. Station resumed 				transmitting 29 July and stopped 9 August. Station resumed transmitting 25 				August and stopped 15 September. Station resumed transmitting 27 September 				and stopped 30 December.

Dome C II		OK.

Cape Denison		Sporadic transmission April, May, August, and September. Pressure corrected for high wind speed conditions. Hydro-Tech anemometer continued to function after aerovane failed.

Penguin Point		Repaired and back on the air 21 December. Pressure corrected for high wind 				speed conditions.

Sutton			Wind speed and direction not functioning properly because the tower may be leaning or have fallen. A Hydro-Tech anemometer is installed instead of a delta-T sensor. Station stopped transmitting 30 April due to low battery voltage. Station resumed transmitting 5 September. Station stopped transmitting again 5 October and resumed 26 November. Station stopped again 19 December.

Byrd			Aerovane not functioning 12 July to 2 August, parts of August and September, the last half of October, most of November and early December.

Elizabeth		Aerovane "frozen" mid May through mid June. Station stopped transmitting 3 				August due to low battery voltage and started transmitting again 1 October.

Erin			Aerovane not functioning. Station stopped transmitting 19 July due to low battery 				voltage and resumed intermittent transmissions in late September as battery 				recharged in the austral spring.

Theresa		Delta-T sensor buried after 22 February until late November.

Doug			Station started transmitting 11 October. Relative humidity sensor not functioning.

Mount Siple		Site has a "dog house" AWS without wind speed and direction. Pressure sensor 				not functioning correctly January, February, November, and December.

Siple Dome		Aerovane not functioning or only partially functioning during the colder months. 				Delta-T sensor not installed.

Swithinbank		Delta-T sensor not functioning.

Marble Point		Station stopped transmitting 31 May due to low battery voltage. Sporadic 					transmissions August and September. Transmitting with gaps in November.

Ferrell			Aerovane "frozen" occasionally May through August. Station stopped transmitting 25 August due to low battery voltage.

Pegasus North		Relative humidity sensor not functioning. Aerovane not functioning until 30 				January.

Pegasus South		Relative humidity sensor not functioning. Wind speed not functioning June 				through December.

Minna Bluff		New AWS installed with high wind unit 26 January. Relative humidity sensor not 				functioning. Aerovane not functioning from July through December.

Linda			New station electronics installed 15 January. Aerovane occasionally "frozen" May through September.

Willie Field		Station stopped transmitting 16 April due to low battery voltage. Resumed 				transmitting sporadically during September and the first half of October. Station 				removed 5 December.

Windless Bight		Station installed 9 December. Relative humidity not functioning.

White Out		Station installed 1 December.

White Island		Station installed 1 December.

Whitlock		Station began transmitting erratically in April and stopped transmitting 10 May due to low battery voltage. Delta-T sensor not functioning.

Scott Island		Site has a "dog house" AWS without wind speed and direction.

Possession Island	Site has a "dog house" AWS without wind speed and direction.

Marilyn			Aerovane "frozen" occasionally April through November.

Schwerdtfeger		Relative humidity sensor stopped functioning in March. It began working again for the first half of June, the first half of October, and most of November and December.

Gill			Relative humidity sensor not functioning. Aerovane not functioning 7 August to 				beginning of December.

Elaine			Relative humidity sensor worked sporadically throughout the year.

Lettau			Transmitting with gaps in January and February. Transmissions continuous from 				18 February to 2 June when the station stopped due to low battery voltage. 				Station began transmitting again in the second half of October. November and 				December had many gaps in the transmission.

Manuela		Aerovane broken.

Lynn			Station removed 4 January.

Larsen Ice Shelf	Station stopped transmitting 20 June due to low battery voltage. Station started 				transmitting again 8 August, although the aerovane was not functioning for most 				of August through October.

Butler Island		Aerovane not functioning from 10 September to 27 October. Pressure continues 				to need correction due to the failure of the precision time-based correction to the 				system clock.

Uranus Glacier		Gaps in transmission in January and February. Station stopped on 4 June due to low battery voltage. Station resumed transmitting 24 August.

Limbert			Aerovane occasionally "frozen" from mid April through mid May and then 					continuously through most of October. Station stopped transmitting 2 December.

Racer Rock		Station transmission sporadic June through 23 September due to low battery 				voltage.

Bonaparte Point		Occasional problems with wind direction. Relative humidity sensor not functioning. Station stopped transmitting 6 May due to low battery voltage. Station resumed transmitting 4 June with periodic gaps in the data through October. Temperature not functioning from beginning of August to middle of October.

Ski-Hi			Intermittent data transmissions. No transmissions received for December.

Santa Claus Island	Station electronics repaired in January. Relative humidity sensor 	not functioning very well. Aerovane "frozen" first week of June.

Clean Air		Occasional problems with relative humidity sensor.

Henry			Aerovane occasionally "frozen" in August. Station stopped transmitting 11 				September due to low battery voltage. Station resumed transmitting 16 October 				as the battery recharged in the austral spring.

Nico			Aerovane occasionally "frozen" August through October.

Relay Station		OK.

Dome Fuji		Transmitted erratically the last half of January and all of February. Began 					transmitting continuously 11 March with occasional gaps in May, June, and July. 				Station stopped transmitting 30 July. Station transmitted sporadically in October 				and December.

- 6.2.  AWS Antarctic Field Activities

Field work in the McMurdo and Ross Ice Shelf areas was performed by Robert Holmes of the University of Wisconsin and John Cassano of the University of Wyoming. Mr. Cassano had made two trips to Antarctica while a graduate student at the University of Wisconsin.

On  5 January 1998 Lynn site was visited by Twin Otter and AWS 8935 was removed. The site has now been abandoned. On 6 January Elaine site was visited by Twin Otter. The aerovane was replaced and the delta-T raised. After a week of bad weather and the absence of the Twin Otter, a Twin Otter flight was made to Schwerdtfeger site on 13 January. The aerovane was replaced and the delta-T raised.

With constant bad weather settling in, operations shifted to the area closer to McMurdo. On 14 January, Willie Field site was visited by Twin Otter van. The CR-10 for the acoustic depth gauge was removed for upgrading. A test was made of the PICO hand drill. Linda site was visited on 15 January using a USCG helicopter. The electronics and aerovane were replaced with AWS 8919. A return flight on 16 January via a PHI helicopter was made to replace the batteries, solar panel and junction box and to retrieve the old batteries.

After more than a week of bad weather, a Twin Otter flight was made to Gill site on 27 January. The aerovane was changed and the delta-T raised. Minna Bluff site was also visited on 27 January using a USCG helicopter. A new station was installed (AWS 21360) with a high wind speed unit. Pegasus North site was visited using the Twin Otter van on 30 January. The sensor boom, R.M. Young, antenna and batteries were replaced.

Field work in Adelie Land is performed by the Institut Francais Pour la Recherche et la Technologie Polaires (IFRTP).  The M/S Astrolabe visited Cape Denison site but was unable to land due to bad weather. The ship also visited Port Martin site and found that the tower was broken and a complete new station was needed. On 28 January a new electronics box was installed at D-10 site, and the tower was raised. On 5 February D-47 site was reinstalled, and the tower raised.

Field work on the Antarctic Peninsula is performed by the British Antarctic Survey. On 18 January, Uranus Glacier site was visited, and the tower raised one section. Ski-Hi site was visited on 28 January, and the batteries were replaced. On 31 January, Limbert site was visited. The tower was raised two sections, and the antenna was replaced. Larsen Ice Shelf site was visited on 1 February. The tower was raised, and it was noted that the site is now 17 km from the ice edge. Finally, Butler Island site was visited on 5 February, and the tower raised one section.

AWS sites near Palmer Station are serviced by the Long Term Ecological Research group and Palmer Station personnel. Bonaparte Point site was visited, and the electronics were repaired.

Austral spring field work in 1998 was performed by C.R. Stearns on board the ice breaker Polar Sea. The first stop was at D-10 site on the Adelie Coast near Dumont D’Urville on 20 December. Visibility was near zero due to fog, and it was not possible to fly to Dumont D’Urville, D-10, Sutton, Port Martin or Cape Denison. The poor visibility was forecast to continue so the decision was made to transit to Cape Webb, arriving there on 21 December. A helicopter flight was made to Penguin point site where AWS 8929 as well as the boom and aerovane were removed. AWS 8910 was installed with a high wind speed and direction system. Two helicopters were loaded for the trip to Cape Webb site. G. Wendler and C. Stearns looked for a new location to place the AWS. The choice was made to locate the unit on the rim of the bowl where the old site was located. The old site was dismantled, but the equipment had sustained heavy damage. The tower had collapsed and was partially buried in the snow covered ice. The antenna end of the boom was buried in the ice along with the electronics box on the underside of the tower and the aerovane tail. Due to the damage to the equipment and the inability to remove any of the chain, it was not possible to install the unit at the new site without going to the icebreaker for the necessary equipment. Contact with the icebreaker revealed that the icebreaker was in fog, and no flights would be able to take off. Battery boxes, tower sections, and the solar panel were left at the new site. Visibility continued to be limited so the cruise continued around the Balleny Islands to head for McMurdo.

In the McMurdo area, SPAWAR personnel removed Willie Field site (AWS 8901) on 4 December. Two new stations were installed by SPAWAR personnel at White Out (AWS 8697) and White Island (AWS 8722) on 1 December.

The British Antarctic Survey also performed some field work during the austral spring. Limbert site was visited on 25 October, and the aerovane was replaced. Uranus Glacier site was raised 2 tower sections on 3 November, a damaged solar panel was replaced, and new connections were made to the cable to the regulator box which is buried in the snow. The aerovane was replaced at Butler Island site on 25 October because the wind direction had jammed due to ice.

Finally, two boxes of three batteries each and battery cables were added to Racer Rock unit on 23 December by Kevin Bliss.

7.  GLOBAL TELECOMMUNICATIONS SYSTEM

The data from 35 Antarctic AWS units were entered into the Global Telecommunications System (GTS) during 1998. The data are collected by Service ARGOS. As soon as the data are received, Service ARGOS processes them and sends them on to the National Weather Service which distributes the data to the GTS.The data headers are:

- SMAA14 KARS YYGGgg

SIAA14 KARS YYGGgg

SNAA14 KARS YYGGgg

where S indicates surface, M is main observations (at 00, 06, 12, and 18 UT), I is intermediate observations (at 03, 09, 15, and 21 UT), and N is any other time. AA14 is for Antarctica, and KARS stands for the Largo receiving center (backup is LFPW for the center in Toulouse, France). YY indicates the day in the month, GG is the hour, and gg is the minutes. Table 3.1 contains the WMO identification number used by the GTS grouped according to their purpose and proximity where possible.

The University of Wisconsin-Madison is responsible for obtaining WMO numbers for AWS sites and for providing Service ARGOS with calibration information for processing the data. The main reason for getting the AWS data into the GTS is to make sure that the data are available in near real time for all organizations operating in Antarctica.

- 8.  DATA AVAILABILITY

The data from our Automatic Weather Stations are available by anonymous FTP.  The IP address and domain name are 128.104.109.33 (ice.ssec.wisc.edu).  The login is "anonymous" (do not use the quotation marks), and the password is your email address.  Once you have logged in, change to the pub subdirectory.  A listing of our station locations, names, and ARGOS ID numbers is located in the file "biglist" in this subdirectory.  It is meant to serve as a guide to our stations as their ID numbers sometimes change. A complete guide for navigating the site may be found in the file “readme.faq”.

Our three-hourly interval data for Antarctica in ASCII text format are contained in the year subdirectories of pub/antrdr.  The data have been corrected, i.e. an effort has been made to remove the bad data points.  These data take longer to process, so the data for recent months are not available in this format.  Within each of the year subdirectories of pub/antrdr, there are text files named "3hrlist??" (where ?? indicates the last two digits of the year). These files list what station's data are contained in which files. The files “readme.updates?” in pub/antrdr contain information on updates and/or corrections to the data, and the file “readme.3format” contains file name construction information and format of the three-hourly data. The file “readme.mailinglist” contains information on joining a mailing list which distributes information on data updates and changes. To subscribe, send email to aws@ssec.wisc.edu with the subject line left blank. In the message body, type “subscribe three yourname@email.address” (do not use quotation marks) and substitute your own email address for ”yourname@email.address”.

The directory pub/summary/monthly contains ASCII printable text files of the paper data summary sheets.  The format of the files can be found in the file "readme.sum" while updates and corrections to the data are located in "readme.sumupdates".  The data are located in year subdirectories of pub/summary/monthly.

For those users who need more current information, we have created 10 minute interval data in ASCII text format for each station.  These data are located in year subdirectories of pub/10min/rdr.  The data have been calibrated for the individual station instruments, but no other corrections have been made.  The data are generally available up to and including the last full month of this year.  The year subdirectories also contain a text file named "namelist??" (where ?? indicates the last two digits of the year in question).  These files list  what station's data are contained in which files.

Several important readme files are located in pub/10min/rdr. The file "readme.5digit" contains information on the Siple Coast stations which have a different station identification. The file "readme.format" contains information on filename construction of the data, as well as well as file content, and is a must for those unfamiliar with the data. The files "readme.updates?" contain important information on changes/additions to the data, and the file “readme.mailinglist” contains information on joining a mailing list to receive notification by email of data changes and updates. To subscribe, send email to aws@ssec.wisc.edu with the subject line left blank. In the message body, type “subscribe ant yourname@email.address” (do not use quotation marks) and substitute your own email address for “yourname@email.address”.

Our site is available 24 hours a day, 7 days a week.  If you have questions or problems, send email to aws@ssec.wisc.edu.  We can also be reached by phone at (608) 265-4816 or fax at (608) 262-5947.

By mail, please contact:

Linda M. Keller

University of Wisconsin

Department of Atmospheric and Oceanic Sciences

1225 W. Dayton St.

Madison, WI  53706

USA

- 9.  ACKNOWLEDGMENTS

This work is supported by the National Science Foundation, Office of Polar Programs, Grant 9726040 under the management of Dr. Bernhard Lettau.  Expeditions Polaires Francaises installs and maintains the AWS units from the Adelie Coast to Dome C II.  The British Antarctic Survey maintains the AWS units on the east side of the Antarctic Peninsula and south of Adelaide Island. The Japanese Antarctic Research Expedition (JARE) installs and maintains the AWS units at Relay Station and Dome Fuji.